3GPP TSG-SA5 Meeting #163	S5-254537

Wuhan, China, 13. - 17. October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0726 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Correct clause title |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause title is too unspecific. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause title corrected to be more restrictive in scope. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause title would be confusing for the reader. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-253673 |  |  |  |  |  |  |  |  |

First change

##### 5.1.1.6.11	LTM  cell switch layer 1 measurements

5.1.1.6.11.1	Total number of TA acquisition requests for the candidate target LTM cell

a)	This measurement provides the total number of TA acquisition requests as part of the Early TA acquisition procedure for LTM (L1/L2 Triggered Mobility).

b)	CC

c)	This measurement is pegged by successful transmission of RA Preamble by the source gNB-DU to UE for Early TA acquisition procedure as defined in the chapter 9.2.6 of TS 38.300 [49]. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Note 1: In case of intra gNB-DU LTM the source gNB-DU is also candidate target gNB-DU.

Note 2: The source gNB-DU identifies the RA preamble sent to UE is related to Early TA acquisition procedure based on the procedures related to LTM decision in gNB-CU which are followed with UE context modification for the impacted UE as defined in the clauses 8.2.1.4 and 8.2.1.5 of TS 38.401 [66].

d)	Each measurement is an integer value.

e)	The measurement name has the form MM.TAAckReqLTM.

f)	NRCellDU;
NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS

5.1.1.6.11.2	Number of successful TA acquisitions for the candidate target LTM cell

a)	This measurement provides the number of successful TA acquisitions as part of the Early TA acquisition procedure for LTM (L1/L2 Triggered Mobility) for the scenario when possible Cell Switch Command has not been yet sent to UE for the Candidate Cell ID.

b)	CC

c)	This measurement is pegged

-	in case of inter gNB-DU LTM by reception of the CU-DU TA INFORMATION TRANSFER message at source gNB-DU from gNB-CU CP with included TA Value within the TA Information List IE (clause 9.2.1.24, 3GPP TS 38.473 [6]) and possible Cell Switch Command has not been yet sent to UE for the Candidate Cell ID.  In case the TA Information List IE contains the TA value for more than one Candidate Cell ID (candidate target cell) the pegging is done for each of the cells. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

-	in case of intra gNB-DU LTM internally at source gNB-DU after successful TA acquisition per each Candidate Cell ID (candidate target cell) and possible Cell Switch Command has not been yet sent to UE for the Candidate Cell. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Note: In case of intra gNB-DU LTM the source gNB-DU is also candidate target gNB-DU.

d)	Each measurement is an integer value.

e)	The measurement name has the form MM.TAAckSuccLTM.

f)	NRCellDU;
NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS

5.1.1.6.11.3	Number of successful but late TA acquisitions for the candidate target LTM cell

a)	This measurement provides the number of successful but late TA acquisitions as part of the Early TA acquisition procedure for LTM (L1/L2 Triggered Mobility).

b)	CC

c)	This measurement is pegged

-	in case of inter gNB-DU LTM by reception of the CU-DU TA INFORMATION TRANSFER message at source gNB-DU from gNB-CU CP with included TA Value within the TA Information List IE (clause 9.2.1.24, TS 38.473 [6]) but Cell Switch Command has been already sent to UE for the Candidate Cell ID. In case the TA Information List IE contains the TA value for more than one Candidate Cell ID (candidate target cell) the pegging is done for each of the cells. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

-	in case of intra gNB-DU LTM internally at source gNB-DU after successful TA acquisition per each Candidate Cell ID (candidate target cell) but Cell Switch Command has been already sent to UE for the Candidate Cell. The measurement is pegged in source gNB-DU per source and target candidate cell pair.

Note: In case of intra gNB-DU LTM the source gNB-DU is also candidate target gNB-DU.

d)	Each measurement is an integer value.

e)	The measurement name has the form MM.TAAckSuccLateLTM.

f)	NRCellDU;
NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS

End of changes