3GPP TSG-SA5 Meeting #163	S5-254558

# Wuhan, China , 13 – 17 October 2025

# Source:	Ericsson

# Title:	Rel-19 DP on Initial ML model training

# Document for:	Discussion and Endorsement

Agenda Item:	6.19.1

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management”.

# 3	Rationale

## 3.1	Observations

Clause 3 of TS 28.105 [1] provides the following definition for the concept of ML model initial training: “a process of training an initial version of an ML model”.

Observation 1: The input of the initial training is the initial version of an ML model. The output of the initial training is a new version of the same ML model.

Clause 7.3a.1.2.2.1 of TS 28.105 [1] provides a description of MLTrainingRequest IOC, which represents a consumer-initiated ML model training request. This IOC includes information that allows MnS consumer to request one of the following training types: initial training, pre-specialized training, re-training and fine-tuning. For initial training, the clause quotes that it represents a case  “where the MnS consumer requests to train a ML model of which the instance does not exist yet”.

Observation 2: There does not exist an MLModel MOI representing an initial version of the ML model.

Clause 7.3a.1.2.2.2 and 7.3a.1.2.2.3 of TS 28.105 [1] lists the IOC attributes and associated constraints, respectively. There are two noteworthy attributes: “aimlInferenceName” (CM)  and “mlModelRef" (M). The first attribute needs to be implemented (on the producer side) when the following condition is met: “Any of the following training types are supported: Initial training, pre-specialized training, fine-tuning”. The second attribute needs to be implemented (on the producer side) always, regardless of the ML training type.

Table 7.3a.1.2.2.3-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLInferenceName | CM | T | T | T | T |
| candidateTrainingDataSource | O | T | T | F | T |
| trainingDataQualityScore | O | T | T | F | T |
| trainingRequestSource | M | T | T | F | T |
| requestStatus | M | T | F | F | T |
| expectedRuntimeContext | M | T | T | F | T |
| performanceRequirements | M | T | T | F | T |
| rLRequirement | CM | T | T | F | T |
| fLRequirement | CM | T | T | F | T |
| cancelRequest | O | T | T | F | T |
| suspendRequest | O | T | T | F | T |
| trainingDataStatisticalProperties | O | T | T | F | T |
| distributedTrainingExpectation | O | T | T | F | T |
| mLKnowledgeName | CM | T | T | F | T |
| mLTrainingType | M | T | T | F | T |
| expectedInferenceScope | CM | T | T | F | T |
| clusteringInfo | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| mLModelRef | M | T | T | T | T |
| mLModelCoordinationGroupRef | CM | T | T | T | T |

Table 7.3a.1.2.2.3-1

| Name | Definition |
| --- | --- |
| aIMLInferenceName | Condition: Any of the following training types are supported: Initial training, pre-specialized training, fine-tuning. |
| mLModelCoordinationGroupRef | Condition: ML model joint training is supported. |
| mLKnowledgeName | Condition: ML-knowledge-based transfer learning is supported. Knowledge is indicated only if candidateTrainingDataSource is not indicated |
| rLRequirement | Condition: Reinforcement learning is supported. |
| fLRequirement | Condition: FL is supported |
| expectedInferenceScope | Condition: The MLTrainingRequest is for an ML model pre-specialised training. |

Observation 3: If the MnS producer supports initial training, the MnS producer shall implement the attribute “aimlInferenceName” attribute.

Observation 4: To request for initial training of an ML model, the MnS consumer specifies a value for “aimlInference” attribute, and leaves “mlModelRef” attribute empty/unspecified.

## 3.2	Limitations

Current description of initial training notes that the initial version of an ML model cannot be represented with an MLModel MOI. This poses several limitations.

Limitation #1: It prevents from having a scenario where the MLModel MOI exists before the initial training request is issued, which makes sense in operational networks. For example, let’s assume that the operator buys an AI/ML enabled feature associated to a license. Following this, the software supporting such a feature gets updated, and the ML model powering this feature is ready for use in 3GPP management system. This is the initial version of the ML model.  Once this is done, the AI/ML enabled feature is ready for use, and the management of AI/ML model can get started. However, without an MLModel MOI representing that ML model, the operator cannot have visibility on that ML model.

Limitation #2: It makes initial training a mandatory feature to support/implement. The reason is that inference requires having an MLModel MOI. And current specifications note that MLModel MOI is only created after initial training.

Limitation #3: It prevents operators from using an initial version of an ML model (which is already trained in vendor domain) for inference, forcing them to always request for initial training before any other action. Some operators may be interested to conduct inference using the ML model provided as such, without any further training. However, the current specifications prevents this.

## 3.3	Solution

It is proposed to remove the restriction that prevents the 3GPP management system from having a MLModel MOI representing an initial version of the ML model.  This removal is proposed in S5-254559. With this removal, for initial training the following scenarios can be supported.

- Current scenario (i.e. the MLModel MOI does not exist). The operator (MnS consumer) requests for initial training with the createMOI operation for MLTrainingRequest IOC. This request includes the “aimlInferenceName” attribute and NULL value for “mlModelRef”.

- New scenario (i.e. the MLModel MOI does exist). The request includes “mlModelRef” = DN of the MOI. This approach also seeks for alignment on how re-training is requested, facilitating a common approach from operator (MnS consumer) side.

# 4	Detailed proposal

This DP proposes to endorse the following: agree on S5-254559.