3GPP TSG-SA5 Meeting #163	S5-254907

Wuhan, China, 13. - 17. October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR | 0004 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.561 Make NDTDataValue and NDTScalingFactor as conditional mandatory attributes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | L.M. Ericsson Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the PerformanceData datatype, the performanceDataName is mandatory but the NDTDataValue and NDTScalingFactor are optional. This means a user could provide a performanceDataName without specifying what data/scale to be applied. Tables in 6.2.1.3.12.2 does not follow the rules in 32.156 regarding layout and fonts. and their attributes Added tables 6.2.1.3.12.X, 6.2.1.3.12.Y |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed NDTDataValue and NDTScalingFactor from “O” to “CM”. Correct the tables layout |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. User could provide performanceDataName with no indication of the use of said data as the inputs were optional. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3.12.2, 6.2.1.3.12.X (new), 6.2.1.3.12.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.2.1.3.12	PerformanceData <<dataType>>

###### 6.2.1.3.12.1	Definition

This dataType describes the performance data injected in NDT to represent network events.

The attribute performanceDataName identifies a performance measurement or KPI.

The attribute performanceDataValue is used to overwrite values which are synchronized from the real network. Alternatively, performanceDataScalingFactor is used to apply a multiplication factor to values which are synchronized from the real network. For example, in the scenario of using NDT to verify the network response to a signalling storm event, the consumer can request to inject the number of UE requests with multiplication factor of 300% in NDT, which represents three times of the number of UE requests in the real network.

###### 6.2.1.3.12.2	Attributes

The PerformanceData <<dataType>> includes the following attributes.

Table 6.2.1.3.12.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| performanceDataName | M | T | T | F | F |
| CHOICE_1.1 performanceDataValue | CM | T | T | F | F |
| CHOICE_2.1 performanceDataScalingFactor | CM | T | T | F | F |

###### 6.2.1.3.12.X	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1.1 performanceDataValue | This attribute shall be supported when the MnS producer supports performance data inputs which shall be configured with a data value. |
| CHOICE_2.1 performanceDataScalingFactor | This attribute shall be supported when the MnS producer supports performance data inputs which shall be configured with a scaling factor. |

###### 6.2.1.3.12.Y	Notifications

The notifications specified by the notification subclause of the <<IOC>> using this <<dataType>> are applicable.

End of  changes