3GPP TSG-SA5 Meeting #164	S5-255177

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Introduce the solution#2 on LBO Inter CHFs scenario Key issue #3

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce a solution on Local Breakout Inter CHFs scenario key issue3.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

#### 5.1.4.e	Solution #3.y: One charging session is terminated, and the other is also terminated

A possible solution for key issue #3 covering requirements REQ-3GPPCH-LBIC-3, handling the association of two charging sessions.

When the H-CHF terminates the charging session with V-CHF, e.g., the H-CHF triggers an abort notification to terminate the charging session with V-CHF, the V-CHF triggers an abort notification to terminate the charging session with V-SMF(CTF).

When the V-CHF terminates the charging session with H-CHF, e.g., the V-CHF terminates the charging session due to the expiry of the Unit Count Inactivity Timer, the V-CHF triggers an abort notification to terminate the charging session with V-SMF(CTF).

When the V-CHF terminates the charging session with V-SMF(CTF), e.g., the V-CHF triggers an abort notification to terminate the charging session with V-SMF(CTF), the V-CHF terminates the charging session with H-CHF.

When the V-SMF(CTF) terminates the charging session with V-CHF, e.g., the V-SMF(CTF) terminates the charging session due to the expiry of the Unit Count Inactivity Timer, the V-CHF terminates the charging session with H-CHF.

End of change