3GPP TSG-SA5 Meeting #164	S5-255205

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0055 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.540 add missing concepts and backgrouds of management of NG-RAN and 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some features whose concepts are missing, such as SA2 feature ATSSS (Acronym: MASSS) and 5G Prose (Acronym: 5G_ProSe_Ph3), Network sharing (Acronym: TEI19_NetShare) and RAN2 feautre NR mobility (Acronym: NR_Mob_Ph4-Core), SON function (Acronym: NR_ENDC_SON_MDT_Ph4-Core). However, the Stage 2 and Stage 3 solutions of these feature already been supported in TS 28.541. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing concept and background in clause 4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concept and the requirement do not align. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x1(new), 4.X2(new), 4.X3(new), 4.X4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[11]	3GPP TS 23.369: " Ambient power-enabled Internet of Things; Stage 2".

[12]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[13]	3GPP TS 28.314: "Management and orchestration; Plug and Connect; Concepts and requirements".

[x1]	3GPP TS 28.313: "Management and orchestration; Self-Organizing Networks (SON) for 5G networks".

[x2]	3GPP TS 28.310: " Management and orchestration; Energy efficiency of 5G".

[x3]	3GPP TS 32.130: " Telecommunication management; Network sharing;".

1st Change

4.X1	Management of ProSe

The AMF stores the ProSe Capability for 5G ProSe operation. The ProSe Capability indicates whether the UE supports one or more of the following ProSe capabilities defined in TS 23.304 [12]. The Operator can indicate the supported ProSe Capability by the PCF. So, the 3GPP management system should have the capability allowing an authorized consumer to configure ProSe capabilities.

4.X2	Management of ATSSS

The functionality in an ATSSS-capable UE that can steer, switch and split the MA PDU Session traffic across 3GPP access and non-3GPP access, is called a "steering functionality" as defined in TS 23.501[2]. The ATSSS Capability including ATSSS-LL functionality, MPTCP functionality. To enable operators to to understand ATSSS capabilities, 3GPP management system should have capability to obtain the ATSSS capability.

4.X3	SON management

In RAN, Self-optimisation including Mobility Load Balancing, Mobility Robustness Optimization, RACH Optimization, Coverage and Capacity Optimisation and Energy saving is the process of optimizing network performance and requires OAM management as defined in TS 38.300 [3]. The 3GPP management system should have the capability allowing an authorized consumer to switching on/off the SON function, making policies for the SON function, and evaluating the performance of the SON function.

The detailed information and requirements can be refered to TS 28.313 [x1] and TS 28.310 [x2].

4.X4	Network sharing management

Various network sharing scenarios including Passive RAN sharing and Active RAN sharing, and for Active RAN sharing, where active network elements of the RAN are shared, including RAN-only sharing, Gateway Core Network and Indirect Network Sharing. The 3GPP management system should have capability allowing its authorized consumer to manage S-RAN according to these scenarios.

The detailed information and requirements can be refered to TS 32.130 [x3].

End of changes