3GPP TSG-SA5 Meeting #164	S5-255219

Dallas, USA, 17 - 21 November 2025

Source:	CATT

Title:	Pseudo-CR on Add new use case and requirements on distributed inference

Document for:	Approval

Agenda item:	 6.20.2

Spec:	3GPP TR 28.882

Version:	0.1.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This pCR proposes to add use case and requirements on distributed inference to TR 28.882.

Proposed Changes

* * * First Change * * * *

### 5.5.1	Use cases

#### 5.5.1.x	Management capabilities enhancements for AI/ML inference

#### 5.5.1.x.1	Management support for distributed inference

#### 5.5.1.x.1.1	Description

When AI/ML inference is performed in a centralized way, a substantial volume of real-time operational data would need to be collected and transported to that central inference point. Such centralization may lead to data transport bottlenecks and increased end-to-end latency, which can adversely affect the timeliness and efficiency of analytics and closed-loop automation.

Distributed inference refers to the deployment and execution of a trained ML model or set of ML models at multiple network or management functions (e.g., gNBs, NWDAFs, MDAFs, Edge nodes). The AI/ML inference function with a specific ML model or models may run locally, in cooperation with other inference nodes, or in a hierarchical/disaggregated way. For instance, while NWDAF acts as a FL server may run in a central CN function, distributed inference may be used at edge gateways or regional NWDAFs acts as FL clients to process local traffic data and then feed summaries to central NWDAF.

The 3GPP mobile network architecture, encompassing RAN, CN and Management System, is inherently distributed. When AI/ML capabilities are integrated for automation, optimisation and analytics, distributed inference is employed to enhance throughput and reduce latency through workload parallelisation. As for this, the management system are required to support capabilities of the distributed inference.

NOTE 1: How ML model(s) and data exchange between different inference functions depends on implementation which is not in scope for standardization.

#### 5.5.1.x.1.2	Potential Requirements

REQ-DIST_MGMT-01: The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to quiry the capability pertaining to distributed inference.

REQ-DIST_MGMT-02: The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to request an ML distributed inference for a specific ML model or models.

REQ-DIST_MGMT-03: The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to specify requirements on distributed inference.

#### 5.5.1.x.1.3	Possible solutions

TBD

#### 5.5.1.x.1.4	Possible solutions evaluation

TBD

* * * End of Change * * * *