3GPP TSG-SA5 Meeting #164	S5-255237

Dallas, USA, 17 – 21 November 2025

Source:	NTT DOCOMO

Title:	Pseudo-CR on Rel-20 TR 28.881 Enhance solution for Use-Case#11 and add evaluation, recommendations

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	0.2.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This pCR provides enhancements to the potential solution of Use-Case#11 for removing editor’s note, and adds evaluation and recommendations for the use-case.

Proposed Changes

* * * First Change * * * *

## 4.11	Use case #11: Enhancing intent feasibility check capability

### 4.11.1	Description

This use case describes proposed enhancements for feasibility check, enabling intent-driven MnS consumer to receive information regarding how to make an infeasible intent feasible together with feasibility check in case the intent is deemed as infeasible. This will support MnS consumers to understand the changes needed for a feasible intent.

In 3GPP TS 28.312 [1], clause 5.3.3.1 states the following:

“In case the result of intent fulfillment feasibility check is infeasible, MnS producer notifies the MnS consumer the reason of infeasibility and corresponding recommendations, then the MnS consumer decides how to handle the issue that intent is infeasible, e.g. update the intent, suspend the intent, delete the intent, etc.”

However, the issue is the IntentFeasibilityCheckReport specified in 3GPP TS 28.312 [1], clause 6.2.1.3.10 does not include corresponding recommendations. The MnS consumer may request exploration as specified in 3GPP TS 28.312 [1] Clause 5.3.5, based on inFeasibleExpectationInfos which is an optional attribute indicating the infeasible expectations including infeasible targets.  However, if the inFeasibleExpectationInfos attribute is not supported, then it is very difficult for the intent-driven MnS consumer to update an infeasible intent as they do not have the recommendation on how to make it feasible.

### 4.11.2	Potential requirements

REQ-Intent_FEAS1: The intent driven MnS producer should have the capability to enable the authorized MnS consumer to receive the intent exploration report together with intent feasibility check report, if the feasibility check result is infeasible.

### 4.11.3	Potential solutions

#### 4.11.3.1	Potential solution #1 for a new procedure of feasibility check with exploration

This solution proposes to reuse the existing generic intent exploration report and intent feasibility check report defined in 3GPP TS 28.312 [1] and to enhance the Intent <<IOC>>.

In order to enable an MnS consumer to request a feasibility check with corresponding recommendations, a procedure is proposed with ensuring feasibility check with exploration is supported and enabled by a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute of Intent <<IOC>> which is specified in 3GPP TS 28.312, clause 6.2.1.4. When feasibility check with exploration is requested by the intent-driven MnS consumer, the intent driven MnS producer can report corresponding recommendations for infeasible expectations with existing IntentExplorationReport <<dataType>>, together with existing IntentFeasibilityCheckReport <<dataType>>.

When the MnS producer creates the intent instances for intent feasibility check with exploration, based on the received intent creation request with the new intentMgmtPurpose “FEASIBILITYCHECK_WITH_EXPLORATION”, the intent lifecycle management state is to be FEASIBILITYCHECK. If the MnS producer decides the intent is infeasible, the MnS producer triggers the exploration process for possible values of the infeasible targets and sends an exploration report. In this case, the intent lifecycle management state is to be EXPLORATION.

### 4.11.4 Evaluation of potential solutions

Only one potential solution provided in clause 4.11.3.1 is identified. This potential solution proposes to enhance Intent <<IOC>>, with a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute in order to satisfy the use-case requirement.

The implementation of this potential solution is not complex as it only requires an additional value to be allowed for existing intentMgmtPurpose attribute. Therefore, the potential solution described in clause 4.11.3.1 is a feasible solution for the MnS consumer to receive the intent exploration report together with intent feasibility check report, if the feasibility check result is infeasible.

* * * Second Change * * * *

# 5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

## 5.X	Use case #11: Enhancing intent feasibility check capability

The use case description, requirements and a potential solution for enhancing intent feasibility check capability are described in clause 4.11. This use case enables an MnS consumer to receive the intent exploration report together with intent feasibility check report, if the feasibility check result is infeasible. So that the MnS consumer may receive recommendations with intent exploration report regarding how to make an infeasible intent feasible.

The potential solution described in clause 4.11.3.1, which proposes a new allowed value “FEASIBILITYCHECK_WITH_EXPLORATION” that is to be included for intentMgmtPurpose attribute, can be used as baseline for normative work.

* * * End of Changes * * * *