3GPP TSG-SA5 Meeting #164	S5-255255

Dallas, USA, 17 - 21 November 2025

Source:	ZTE Corporation

Title:	Pseudo-CR TR 28.881 Add Evaluation and Conclusion for UC #8 and UC #9

Document for:	Approval

Agenda item:	6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_IDMS_MN_Ph4

Comments

Use case #8: Support to express guarantee requirements in an intent and Use case #9 Intent handling capability configuration, registration and discovery, and corresponding solutions are discussed and approved. This contribution proposes to add evaluation and conclusion.

Proposed Changes

* * * First Change * * * *

4.8 Use case #8: Support to express guarantee requirements in an intent

4.8.1 Description

In today’s highly dynamic and heterogeneous network environment, Intent-Driven Management Service (IDMS) is emerging as the foundational paradigm for advanced network operations and automation.  Nevertheless, practical deployments reveal a critical capability gap: the absence of robust support for dynamic resource reservation and release.

Intent expression, translation, negotiation, and fulfilment are intrinsically coupled to resource allocation—e.g., bandwidth, capacity and spectrum.  When the IDMS is used for communication service assurance in a specific time window, the MnS consumer should be allowed to express the guarantee requirements in the intent. Then, the MnS Producer can perform some actions (e.g., resource reservation) to guarantee the intent fulfilment in the future. Because these resources are highly time-variant, the current IDMS lacks the ability for the MnS Consumer to proactively reserve or release them on demand.  This shortcoming results in inefficient resource utilisation and reduced flexibility in intent fulfilment. Additionally, after an MnS Producer has classified an intent as “FEASIBLE” following the initial feasibility check, subsequent changes in network resource can render the intent “INFEASIBLE” before intent fulfilment is requested without actions taken for guarantee. This temporal inconsistency, caused by the absence of continuous feasibility validation, will lead to fulfilment failures, undermining the reliability of intent-based operations and degrading overall network performance.

An intent defines one or more requirements, goals and constraints that should be ensured for the intent to be considered fulfilled. The requirements are evaluated against the state of the network during the intent fulfilment feasibility check. Even if the result is feasible, no guarantees are explicitly provided by the MnS producer to ensure that the requirements will be continuously fulfilled over time. While the MnS producer may find a solution that meets the requirements when the intent is accepted, it may later degrade due to over-allocation, faults, or other issues.

To tackle this issue, the concept of guaranteed requirements is introduced. Such requirements should be considered for forward-looking sustained compliance. The MnS Producer should be able to utilize different methods to guarantee the intent fulfilment in the future.

NOTE: The guaranteed requirements may apply to all or part of an intent - that is, to the entire intent or to specific expectations. This study will determine which aspects of an intent should be covered by guaranteed requirements.

Since in reality there is no way to guarantee something all the time (i.e., there is always a probability that a requirements will be breached), Guaranteed requirements are associated with a confidence level that expresses the probability of successfully in fulfilling the requirements. The confidence level is specified as a probability threshold and is provided by the MnS producer. How to express confidence may depend on the context of the intent, e.g. for a connectivity service-related intent, the confidence may be expressed as service availability and service reliability. How the MnS producer will provide the confidence level to the requirements is implementation specific.

Since guarantees may not extend indefinitely, the MnS consumer may specify a guarantee period over which the MnS producer provides the confidence level. The guarantee period begins when an intent is created or updated.

4.8.2 Potential requirements

REQ-IDMS_Guarantee-1: The intent driven MnS producer should have the capability to allow MnS consumer to express an intent to be guaranteed and which expectations requirement in an intent should be guaranteed.

REQ-IDMS_Guarantee-2: The intent driven MnS producer should have the capability to allow MnS consumer to specify the guarantee period.

REQ-IDMS_Guarantee-3: The intent driven MnS producer should have the capability to report to the MnS consumer about the confidence level of fulfilling the guaranteed expectations at time of reporting.

4.8.3 Potential solutions

To support to express guarantee requirements in an intent, the following enhancements are proposed:

Add a new attribute guaranteeIndicator in the Intent IOC to allow the MnS Consumer to specify Guarantee Requirements in the intent. guaranteeIndicator attribute can be on intent expectation level.

4.8.4  Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

4.9 Use case #9: Intent handling capability configuration, registration and discovery

4.9.1 Description

4.9.1.1 Intent handling capability description

In TS 28.312 [1], the existing use case, requirements and solution (including IntentHandlingCapability <<dataType>>) for intent handling capability obtaining are defined, which allows the MnS consumer to query the intent handling capabilities for a specific intent handling function. The use case also describes that different intent handling functions are deployed to support different areas of the same intent expectation object domain. However, the IntentHandlingCapability <<dataType>> (including supportedExpectationObjectType and supportedExpectationTargetInfoList) is not allowed to describe the supported area information. In additional, for radio network intent, multiple intent handling functions can be deployed to support different radio access technologies (e.g. EUTRAN, NR) and/or different frequencies. It is important to describe the supported radio access technologies (e.g. and frequencies for the radio network intent handling functions.

Several optional intent negotiation functionalities (including Intent Feasibility check, Intent Exploration and Intent Fulfilment Negotiation) are introduced in the TS 28.312 [1]. One intent handling function may support all the negotiation functionalities or part of the negotiation functionalities. For example, intent handling function A support both Intent Feasibility check and Intent Exploration functionalities, while intent handling function B only support Intent Feasibility check functionality. So, it is important to allow MnS consumer to know which negotiation capabilities can be provided by a specific intent handling function.

4.9.1.2 Intent handling capability registration and discovery

In 3GPP TS 28.537 [6], the existing use case, requirements and solution (including MnSInfo IOC) for MnS discovery are defined, which allows MnS consumer to retrieve the list of MnS instances which can provide the intent driven management capability from MnS Registry. In this case, MnS consumer needs to send the query request to each intent handling function (represented by mnsAddress of the retrieved MnS instances from MnS Registry) to obtain the detailed intent handling capabilities.

However, in some scenarios, MnS consumer may need to request to retrieve a MnS instance with a specific intent handling capability from MnS Registry.

4.9.1.3 Intent handling capability configuration

As TS 28.312 [1] described, multiple intent handling functions maybe deployed to support different expectation objects or to support different areas of the same intent expectation object domain. Operator may need to configuration the intent handling capability for each intent handling function with different responsibilities. For example, intent handling function A is configured to support handling radio service intent in the Venue A, while intent handling function B is configured to support handling radio service intent in the Venue B.

4.9.2 Potential requirements

REQ-IDMS_IHCO-CON-1: The intent driven MnS producer should have capabilities enabling an MnS consumer to obtain intent handling capabilities of each intent handling function, including supported contexts.

NOTE: Examples of contexts to be included as part of the intent handling capabilities are the ObjectContexts and the ExpectationContexts as defined in clause 6.2.2.1 of 3GPP TS 28.312 [2].

REQ-IDMS_IHCO-CON-2: The intent driven MnS producer should have capabilities enabling an MnS consumer to obtain intent negotiation capability of a specific intent handling function.

REQ-IDMS_IHCO-CON-3: The 3GPP management system should have capabilities enabling the configuration of the intent handling capability for a specific intent handling function.

Editor’s Note: which content of the intent handling capability can be configured needs further investigation.

4.9.3 Potential solution

This solution proposes to reuse and enhance the existing IntentHandlingFunction defined in 3GPP TS 28.312 [2].

Enhancement Aspect1: Add following attributes for IntentHandlingCapability <<dataType>>:

- supportedObjectContextInfoList, it represents the list of specific ObjectContexts supported by the intent handling function. The new SupportedObjectContextInfo datatype includes the supportedObjectContextAttribute, supportedObjectContextCondition and supportedObjectContextValueRange attributes.

- supportedExpectationContextInfoList – it represents the list of specific ExpectationContexts supported by the intent handling function. The new SupportedExpectationContextInfo datatype includes the supportedExpectationContextAttribute, supportedExpectationContextCondition and supportedExpectationContextValueRange attributes.

Enhancement Aspect2: Add following attributes for IntentHandlingFunction <<IOC>>:

- supportedNegotiationFunctionalities, it represents the list of intent negotiation functionalities supported by the intent handling function. The type is ENUM with allowed values: FEASIBILITYCHECK, EXPLORATION and FULFILMENT_WITH_NEGOTIATION. If the value is absent, none of the intent negotiation capabilities are not supported by the intent handling function.

- intentHandlingScope, representing the scope of received intent that the intent handling function can support to address, the allowed values can be RAN, or CN.

4.9.4 Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

* * * End of Changes * * * *

5	Conclusions and Recommendations

Editor's note: this clause will contain conclusions and recommendations for corresponding key issues identified in clause 4.

5.X	Use case #8: Support to express guarantee requirements in an intent

The proposed solution satisfies the potential requirements in the clause 4.8.3 with the minimal impact on the Intent Model. It’s recommended for normative work with the proposed solution as baseline.

5.Y	Use case #9: Intent handling capability configuration, registration and discovery

The proposed solution satisfies the potential requirements in the clause 4.9.3 with the minimal impact on the Intent Model. It’s recommended for normative work with the proposed solution as baseline.