3GPP TSG-SA5 Meeting #164	S5-255366

# Dallas, USA, 17 - 21 November 2025

# Source:	Nokia

# Title:	Discussion paper on ES scenarios and actors

# Document for:	Endorsement

Agenda Item:	6.20.5

# 1	Decision/action requested

The group is requested to discuss and endorse the proposals below.

# 2	References

[1]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[2]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Rationale

## 3.1	Introduction

TS 28.310 [1] defines solutions for ES. It introduces a number of actors (see clause 6.2) including:

- Domain-Centralized ES management

- MnS Producer of Domain-Centralized ES

- MnS Producer of Distributed ES management

- Distributed ES function

- Domain-SON ES functionality

- Domain-centralized SON ES

- Distributed SON ES functionality

- Domain Centralized SON ES management

- CESManagementFunction (IOC defined in TS 28.541 [2])

- DESManagementFunction (IOC defined in TS 28.541 [2])

The relations among these are unclear. In some cases the functionalities are over lapping.

## 3.2	Analysis of terminology use

### 3.2.1	Three scenarios are addressed for ES but not fully listed out as such.

Observations:

Scenario1 (Clause 6.2.2 of TS 28.310 [1])	: A domain centralized entity configures the NR cells directly: Clause 6.2.2.1.1	Energy saving activation sates the following

"Figure 6.2.2.1.1-1 depicts a procedure that describes how MnS producer of Domain Centralized SON ES management makes the NR capacity booster cell enter the energySaving state".

This seems to imply that Domain Centralized SON ES management makes NR cell to enter energySaving State

Scenario 2 (Clause 6.2.3 of TS 28.310 [1]): A domain distributed entity configures the NR cells: The introduction states the following

" Figure 6.2.3.2.1-1 depicts a procedure that describes how MnS producer of Distributed ES management makes the NR capacity booster cell enter the energySaving state".

This seems to imply that the DESM makes the NR cells to enter energySaving state.

Scenario 3 ( Clause 6.2.3, figure 6.2.3.2.1-1 of TS 28.310 [1]): A distributed entity local to the gNBs that is configured on how to control ES in NR cells : Figure 6.2.3.2.1-1 shows the producer of Distributed ES is configuring cell overload relations, Es policy, etc on the Distributed Energy saving Function It is expected then that the Distributed Energy saving Function decides how to switch the energySaving states of the NR cells

Proposals:

Proposal 1: These variations of scenarios for how ES states may be configured should be explicitly clarified as illustrated by Figure 3.2.2-1.

Figure 3.2.2-1: Scenarios for ES that can be concluded form the descriptions and illustrations in TS28.310.

### 3.2.2	Functional entities and their naming:

Observations:

Based on the three scenarios in 3.2.1 above, there are four (4) functional entities involved in ES

- The NR cells that are configured to enter or exit energySaving state. There are no concerns with the  naming of the NR cells

- A domain centralized entity that configures the NR cells directly. It is understood that this entity is referred to as CES Management Function or “Domain centralized SON ES management", since clause 6.2.2.2.2.2 states : "parameters defined in CESManagementFunction are used to control the Domain centralized SON ES functionality ". So, the CESManagementFunction and the Domain centralized SON ES functionality refer to the same entity.

- A domain distributed entity that configures the NR cells. It is understood that this entity is referred to as DES Management Function or “Distributed SON ES management", since  clause 6.2.3.1.2.2 states:	parameters defined in DESManagementFunction IOC, which is defined in TS 28.541 [11], are used to control the Distributed SON ES functionality. So, DESManagementFunction and the Distributed SON ES functionality refer to the same entity.

- A distributed entity in the RAN that internally configures the NR cells states. It is understood that this entity is not explicitly referred to yet some descriptions refer to it. figure 6.2.3.2.1-1 shows the producer of Distributed ES is configuring cell overload relations, Es policy, etc on the Distributed Energy saving Function. This implies that the Distributed ES Management function (exposing the producer of Distributed ES MnS) configures the policies of the Distributed ES function with which the DES function decides how switch energySaving states.

Figure 6.2.3.2.1-1: Distributed energy saving activation

Proposals:

Proposal 2: These actors involved in ES and their roles should be differentiated, preferably with naming that distinguishes their roles.

Proposal 3: The several different names for the actors should be reduced to a number that matches the number of involved actors.

### 3.2.2		Centralized ES activation and Deactivation

Observations:

- Figures 6.2.2.1.1-1 and 6.2.2.1.2-1 seem to indicate that Prov MnS producer should have the intelligence to decide how to configure the other cells when only given a configuration for the NR capacity booster cell. Step 3 in both Figure 6.2.2.1.1-1 6.2.2.1.2-1 shows that Prov MnS producer is only configure with the energySaving state and that the Prov MnS producer derives the rest. This is not always true. It should be clarified that the producer of Centralized ES takes responsibility of deriving all the require attributes and configures them to the respective cells.

Figure 6.2.2.1.1-1: Centralized energy saving activation

Figure 6.2.2.1.2-1: Centralized energy saving deactivation

Proposals:

- Proposal 3: Figure can be clarified to show that the NR cells expose the prov. MnS.

- Proposal 4: the producer of Centralized ES takes should derive all the require attributes for all NR capacity and for coverage cells and configures them to the respective cells.

### 3.2.2		Distributed ES activation and Deactivation

Observations:

The distributed ES activation and Deactivation mixes the scenarios that are related to distributed ES (scenarios 2 and 3 in clause 3.2.1 above). It mixes the functionalities of the domain distributed function and the gNB local distributed ES function i.e. "A domain distributed entity that configures the NR cells" and " A distributed entity in the RAN that internally configures the NR cells states ". Even when they are considered the same, it would appear (according to figure 6.2.3.2.1-1) that a function is configuring itself.. if the producer of Distributed ES is configuring a Distributed ES Function, then the two entities are different. It should be clear that the Distributed ES function is responsible for controlling ES states.

Figure 6.2.3.2.1-1: Distributed energy saving activation

Proposals:

- Proposal 5: Introduce scenario 3 as specifically different from scenario 2

- Proposal 6: Clarify in scenario 3 that the Distributed ES function is the one that controls the energy saving states (energSaving state and notEnergySaving state) of the NR cells.

- Proposal 7: Remove provisioning MnS from the figure as it is inherently expected to be exposed by anything that is configured. Otherwise, as is it seems to imply that provisioning MnS producer is different from the entity that is being configured.

- Proposal 8: The entity that configures the Distributed ES function should be referred to as "MnS producer of Distributed ES Management", otherwise the current names i.e. "MnS producer of Distributed ES" is to close and is easily confused with the configured entity. i.e., the "Distributed ES Function"

# 4	Detailed proposal

The group is asked to endorse to fix the issues with enhancements in Rel-20 as proposed in the following:

Proposal 1: These variations of scenarios for how ES states may be configured should be explicitly clarified.

Proposal 2: These actors involved in ES and their roles should be differentiated, preferably with naming that distinguishes their roles.

Proposal 3: The several different names for the actors should be reduced to a number that matches the number of involved actors.

Proposal 4: Figure can be clarified to show that the NR cells expose the prov. MnS.

Proposal 5: the producer of Centralized ES takes should derive all the require attributes for all NR capacity and for coverage cells and configures them to the respective cells.

Proposal 6: Introduce scenario 3 as specifically different from scenario 2

Proposal 7: Clarify in scenario 3 that the Distributed ES function is the one that controls the energSaving states of the NR cells-

Proposal 8: Remove provisioning MnS from the figure as it is inherently expected to ne exposed by anything that is configured. Otherwise, as is it seems to imply that provisioning MnS producer is different from the entity that is being configured.

Proposal 9: The entity that configures the Distributed ES function should be referred to as "MnS producer of Distributed ES Management", otherwise the current names i.e. "MnS producer of Distributed ES" is to close and is easily confused with the configured entity. i.e., the "Distributed ES Function"