3GPP TSG-SA5 Meeting #164	S5-255375

Dallas, USA, 17 - 21 November 2025

Source:	Ericsson

Title:	Rel-20 pCR TR 28.881 Add use case for Enhancement of radio service for protection

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	V0.1.0

Work Item:	FS_IDMS_MN_Ph4

Comments

This contribution proposes enhancing the radio service scenario to include requirements for service protection.

This pCR is related to WT-1.2.

Proposed Changes

* * * First Change * * * *

## 4.X Use case #X: Enhancement of radio service scenarios for service protection

### 4..1 Description

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation for delivering a radio service is described in clause 5.1.2. The RadioServiceExpectation is defined to represent MnS consumer's expectations for radio service delivering and assurance in the specified area. However, current definitions do not provide means for expressing protection expectations, i.e., requirements related to the level of protection or security controls to be applied to the managed service or resources.

Service protection requirements are relevant to prevent malicious access to the different attack surfaces (i.e., entry points) exposed by managed services or resources.

The MnS consumer may want to express expectations related to the level of protection coverage, ranging from basic perimeter protection to advanced, fine-grained protection covering multiple attack surfaces.

Such qualitative protection levels could be mapped to quantitative observability indicators (e.g., number of protected entry points, compliance ratios, or threat-detection coverage).

This use case studies the capability for the MnS consumer to include service protection requirements as part of the intent and for the MnS producer to evaluate and report the fulfilment of these requirements.

* * * Next Change * * * *

### 4.1.2 Potential requirements

REQ-RadioServiceIntent-PROT-1: The intent-driven MnS producer should have the capability to enable the MnS consumer to express protection requirements.

REQ-RadioServiceIntent-PROT-2: The intent-driven MnS producer should have the capability to enable the MnS consumer to specify different qualitative protection coverage levels.

NOTE: Possible values for the qualitative level can be Basic, Enhanced, Advanced, or vendor-specific. This should be part of the present study.

REQ-RadioServiceIntent-PROT-3: The intent-driven MnS producer should have the capability to evaluate the requested protection requirements against the current protection configuration and report compliance or deviation within the intent-fulfilment reports.

* * * Next Change * * * *

### 4.1.3 Potential solutions

This solution proposes to reuse and enhance the existing RadioServiceExpectation defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Add a new attribute ProtectionLevelTarget as an ExpectationTarget that indicates the qualitative level of protection expected by the MnS consumer.

Possible enumeration values:

- Basic – Simple protection controls applied everywhere and enabled by default.

- Enhanced – More complex protection mechanisms requiring configuration or design but without hardware dependency.

- Advanced – Comprehensive and expensive protection that may require specialized hardware or high-complexity controls.

Enhancement Aspect2 – Add a new attribute ProtectionScopeContext as an ExpectationContext that indicates the set of network function of interest to apply protection level on. If it is not stated, the protection level is applied on the whole Radio Service equally.

NOTE: ProtectionScopeContext may be some of the existing Radio Service ObjectContexts as listed in clause 6.2.2.1.5.2 of TS 28.312.

### * * * End of Changes * * * *