3GPP TSG-SA5 Meeting #164	S5-255562

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add use case solution on access control for data

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y1]	3GPP TS 28.319: "Access control for management services".

[Y2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Y3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[Y4]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[Y5]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration   Reference Point (IRP); Information Service (IS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 4.3 	Enhancement of Management Services Access Control (MSAC)

#### 4.1.A.4      Potential Solution

In this proposed solution, it is proposed to add a new attribute called “allowedPerformanceMetrics” to the AccessRule class (defined in clause 7.3.3 of TS 28.319[Y1]) to replace the existing componentCData attribute:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| allowedPerformanceMetrics | List of the performance metrics that the MnS consumer is allowed to collect on a set of managed object(s) (as identified by the dataNoteSelector attribute). allowedValues: Includes performance measurements defined in TS 28.552[Y2] and key performance indicators (KPIs) defined in TS 28.554[Y3]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

With this enhancement, the performance metrics that an MnS consumer can collect on a given managed object (represented by the dataNodeSelector attribute) is defined.

Accordingly, when an MnS producer receives a request for performance metrics collection from the MnS consumer (i.e., When the MnS consumer creates the PerfMetricJob MOI(see clause 4.3.31 of TS 28.622[Y5]) on the MnS producer), it checks if the requested “performanceMetrics” on a managed object is part of the configured “allowedPerformanceMetrics” for the MnS consumer for that managed object (represented by the dataNodeSelector attribute) in the AccessRule class. If this is the case, the MnS producer will authorize the performance metrics data collection request from the MnS consumer and begin the performance metrics collection and reporting as per the configured intervals.

* * * End of Changes * * * *