3GPP TSG-SA5 Meeting #165	S5-260282

# India, India, 9th Feb 2026 - 13th Feb 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0098 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.550 Corrections on GPB schema and descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute India |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The sequence and order of the values of “'performanceMetrics' of the establishStreamingConnection operation”, “performanceMetrics in Info Content” and the “measResults in Data Content” should be aligned in order to correctly parse the streaming data. This also, imply that if the particular metric is not collected then the vaklue shall have NULL to preserve the sequence and order. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow measResults to have NULL value when the data is not actually collected. Clarify that the requence and order shall be maintained between “'performanceMetrics' of the establishStreamingConnection operation”, “performanceMetrics in Info Content” and the “measResults in Data Content” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk for non multi-vendor implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C, G2, Annex J |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF FIRST CHANGE ***

Annex C (normative): 
Performance Data Stream Unit content description

Table C-1 lists all the Performance Data Stream Unit content items. It also provides an explanation of the individual items.

Table C-1: Performance Data Stream Unit content description

| Performance Data Stream Unit Content |  | Description |
| --- | --- | --- |
| Data Content | streamId | The streamId of the performance data stream. This shall be the same as the Info Content frame associated with this Data Content frame. |
|  | granularityPeriodEndTime | Time stamp referring to the end of the granularity period. |
|  | measResults | This parameter contains the sequence of result values for the observed metrics (measurement types or KPIs). The "measResults" sequence shall have the same number of elements, which follow the same order as the measurement types or KPIs presented in "performanceMetrics" for the subject stream in the input parameter streamInfoList of the establishStreamingConnection operation (see clause 6.2). When a particular performanceMetrics is not actually collected, the measResults shall contain the NULL value. |
| Info Content | streamType | This carries the information of the kind of data a stream is carrying and this would have the value “PERFORMANCE”. |
|  | SerializationFormat | This indicates the format of the serioalization, it can contain “ASN.1” or “GPB”. |
|  | streamId | This contains the id of the stream.This shall be the same streamId as used in the Info Content frame associated with this Data Content frame |
|  | measObjDn | This contains the DN (FDN) of the measured object that is reported as part of this stream. |
|  | performanceMetrics | This contains the performance measurements by name only that are reported by this stream. If more than one performanceMetric are included in the Info Content, a comma separation/delimiter shall be used, i.e. "metric1, metric2, metric3, …". All standardized performanceMetrics for this measObjDn should be listed first, followed by all vendor specific performanceMetrics. “The 'performanceMetrics' sequence shall have the same number of elements, which follow the same order as the measurement types or KPIs presented in 'performanceMetrics' for the subject stream in the input parameter streamInfoList of the establishStreamingConnection operation (see clause 6.2).” |

*** START OF NEXT CHANGE ***

## G.2	ASN.1 definition

-- ASN1START

PerformanceDataStreamUnits-Schema DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- PDSUs-START

PDSUs ::=                      SEQUENCE OF PDSU

PDSU ::=  SEQUENCE {

infoFrameIndicator         BOOLEAN,

....frameContent                FrameContent }

FrameContent ::= CHOICE {

infoContent    InfoContent,

dataContent    DataContent

}

DataContent ::= SEQUENCE {

streamId                   INTEGER,

granularityPeriodEndTime   DATE-TIME,

standardizedMeasResults    SEQUENCE OF MeasValue,

vendorSpecificMeasResults  SEQUENCE OF MeasValue OPTIONAL -- may be omitted

}

InfoContent ::= SEQUENCE {

streamType             VisibleString,

serializationFormat    VisibleString,

streamId               INTEGER,

measObjDn              VisibleString,

performanceMetrics     VisibleString

}

MeasValue ::= CHOICE {

integerValue   INTEGER,

realValue      REAL,

stringValue    VisibleString,

subCounters    SubCounterListType,

noValue		   NULL

...                                             -- allow extension in futher version

}

-- uses recursion for the value to support multi-dimensional measurements

SubCounterListType ::= SEQUENCE {

subCounterIndex   SubCounterIndexType,

subCounterValue   MeasValue OPTIONAL            -- "empty" bins are allowed

}

SubCounterIndexType ::= CHOICE {

sum       VisibleString ("SUM"),

binIndex  INTEGER,

qOS-5QI   INTEGER,

qOS-QCI   INTEGER,

cause     INTEGER,

stringIndex   VisibleString,

plMN      OCTET STRING (SIZE(3)),              -- definition from TS 38.413

sNSSAI    SEQUENCE {                           -- definition from TS 38.413

sst   OCTET STRING (SIZE(1)),

sd    OCTET STRING (SIZE(3))

},

...                                            -- allow extension in futher version

}

-- PDSUs-STOP

END

-- ASN1STOP

*** START OF NEXT CHANGE ***

Annex J (Informative):
Example of ASN.1 Streaming of PMs

This annex provides an example for ASN.1 schema for Performance Data Stream Units.

PDSUs ::= {

PDSU {

{

streamId ::= 123123,

granularityPeriodEndTime ::= 20230301141430,

standardizedMeasResults {

{

integerValue ::= 2322

},

{

realValue ::= 3.1416

},

{

stringValue ::= "This is example String"

},

{

subCounters {

{

subCounterIndex {

{

binIndex ::= 1

}

},

subCounterValue {

{

integerValue ::= 5441

}

}

}

}

}

}

},

{

streamId ::= 122323,

granularityPeriodEndTime ::= 20230301181130,

standardizedMeasResults {

{

integerValue ::= 1122

},

{

realValue ::= 55.336

},

{

stringValue ::= "This is example String"

},

{

subCounters {

{

subCounterIndex {

{

qOS-5QI ::= 3

}

}

}

}

}

}

}

{

streamId ::= 122324,

granularityPeriodEndTime ::= 20230301131150,

standardizedMeasResults {

{

},

}

}

}

}

*** END OF CHANGE ***