3GPP TSG-SA5 Meeting #165	S5-260375

Goa, India, 09 - 13 February 2026

Source:	L.M. Ericsson Limited

Title:	Rel-20 pCR TR 28.883 Solution for Clarification of NDTJob Modification Behaviour

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.2.0

Work Item:	FS_NDT_Ph2

Comments

Solution for the Clarification of the NDTJob Modification

Proposed Changes

* * * First Change * * * *

# 4	Concepts and background

Editor’s note: This clause provides a description of concepts and background.

# 5	Use cases

## 5.10	Use Case #10: Clarification of NDTJob Modification Behaviour

### 5.10.1	Description

The MnS Consumer requires clarity on the conditions and procedures for modifying an existing NDTJob instance. At the moment, the specification leaves this open to interpretation, even though there are clear limitations associated with different implementation techniques for NDTJobs. For instance, if an NDTJob is implemented as a script that performs calculations based on input from the MnS Consumer and the replicated network, modifying such a script during execution would effectively mean rerunning the script with new parameters as resumption of certain scripts would be impossible. On the other hand, if the implementation is utilising a technology which can enable modification during runtime, then modification during the initial stages—or at specific points within the runtime—may still be possible.

Currently, the specification does not allow for clear modelling of the different potential stages executed during an NDTJob. As a result, modification of an NDTJob will be inconsistent across implementations, making MnS Consumer integration bespoke to specific NDTs.

The current specification does not define a procedure or consequences of the modification of an NDTJob. TS 28.561 Clause 4.4 (“NDT life-cycle management”) states that “the NDT job instance can be configured by the MnS Consumer at any time”, yet no guidance is provided on permissible modification timing, allowable attribute changes, or the expected system behaviour when an NDTJob is already under execution.

This ambiguity raises questions such as:

- When, during the NDTJob lifecycle, modification requests are acceptable.

- Whether modification during execution affects job consistency or results.

- How the MnS Producer should report modification status or failures.

A clearer model describing NDTJob state transitions and management conditions would help ensure consistent interpretation across implementations. Such a model could make it explicit when an NDTJob is in a state that permits modification and when it is not.

### 5.10.2	Potential requirements

REQ-DTLCM-XX: The MnS Producer should expose the execution status of each NDTJob instance.

REQ-DTLCM-XX: The MnS Producer should allow for modification of the NDTJob based on certain documented conditions.

### 5.10.3	Potential solutions

To ensure predictable behaviour across different NDTJobs, the NDTJob IOC should be extended with an attribute to expose its current lifecycle status. The proposed statuses of an NDTJob are as follows:

“NOT_STARTED”, “RUNNING”, “SUSPENDED”, “CANCELLED” and “COMPLETED”.

An NDTJob should only be modifiable during certain states of an NDTJob’s lifecycle. For example, while the state is ‘NOT_STARTED”, any request to modify attributes of the NDTJob IOC should be accepted by the MnS Producer.

To support consistent decision-making regarding when an NDTJob can be locked for modification, the MnS Producer should expose the execution status of the NDTJob via a status monitoring attribute (e.g. jobStatus). The MnS Producer should update this attribute throughout the NDTJob lifecycle. The status monitoring attribute should be optional for the MnS Producer. The conditions under which an NDTJob instance may accept or reject modification are implementation-specific and should be documented by the MnS Producer to support system integration and operational alignment.

### 5.10.4	Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

# 6	Conclusions and Recommendations

## 6.X	Use case #10: Clarification of NDTJob Modification Behaviour

There is only a single solution proposed that solves the use-case, the recommendation is to take the solution further into REL20 normative work.

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

* * * End of Changes * * * *