3GPP TSG-SA5 (Telecom Management)	S5-070140

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0060 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduce Charging Support for IMS Early Media |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Telefónica (blanca.segurovelasco@telefonica.es) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7 |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Current charging specifications do not provide for scenarios where SDP negotiation is completed before a final SIP message is received. In such scenarios, as soon as media resources are available, media could be exchanged before a successful or unsuccessful SIP final answer is received. Moreover CT1 (S5-061365) sees no possibility within the specifications produced by IETF, 3GPP, 3GPP2, TISPAN, OMA and other standardization bodies, to use the 200 (OK) response to the INVITE request as an appropriate indication for the network to start charging for early media usage. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | A new parameter is defined in IMS CDRs to describe changes in the state (active/inactive) of the media components negotiated according to the SDP signalling exchanged during the SIP session establishment, previously to the reception of a SIP final response to the initial SIP Invite. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inability to report and charge usage of negotiated media resources before the final SIP answer is received and thus is a risk of fraud. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.3.1, New subclauses in section 5.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.260, 32.298 |  |  |  |
| Other comments:  |  | Related R7 CR 32.260 in S5-070138 Related R7 CR 32.299 in S5-070139 |  |  |  |  |  |  |  |  |

New Clause

##### 5.1.3.1.17A	List of Early SDP Media Components

This is a grouped field which may occur several times in one CDR.  This field describes session, media parameters and timestamps related to media components set to active according to SDP signalling exchanged during a SIP session establishment and before the final successful or unsuccessful SIP answer to the initial SIP INVITE message is received. Once a media component has been set to active, subsequent status changes shall also be registered.

This field applies only to SIP session related cases, but it may be present both in event CDRs (unsuccessful session establishment) and session CDRs (successful session establishment).

The List of Early SDP Media Components contains the following elements:

- SDP Offer Timestamp;

- SDP Answer Timestamp;

- SDP Media Components;

- Media Initiator flag;

- SDP Session Description.

These fields are described in the appropiate subclause.

New Clause

##### 5.1.3.1.31A	SDP Answer Timestamp

This parameter contains the time of the response to the SDP Offer.

New Clause

##### 5.1.3.1.35A	SDP Offer Timestamp

This parameter contains the time of the SDP Offer.

1st Modified Section

#### 5.2.3.1	IMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.260 [20].

IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

RecordType, IPAddress, ManagementExtensions, NodeAddress, LocalSequenceNumber, SubscriptionID, TimeStamp, ServiceContextID

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  IMS RECORDS

--

------------------------------------------------------------------------------

IMSRecord ::= CHOICE

--

-- Record values 63..69 are IMS specific

--

{

sCSCFRecord			[63] SCSCFRecord,

pCSCFRecord			[64] PCSCFRecord,

iCSCFRecord			[65] ICSCFRecord,

mRFCRecord			[66] MRFCRecord,

mGCFRecord			[67] MGCFRecord,

bGCFRecord			[68] BGCFRecord,

aSRecord			[69] ASRecord

}

SCSCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

privateUserID					[8] GraphicString OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[22] NodeAddress OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

list-Of-Message-Bodies			[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

list-of-subscription-ID			[31] SEQUENCE OF SubscriptionID OPTIONAL,

applicationServersInformation	[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

requested-Party-Address			[41] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity [42] ListOfInvolvedParties OPTIONAL,

list-Of-Early-SDP-Media-Components [43] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

PCSCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[22] NodeAddress OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

list-Of-Message-Bodies			[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

list-of-subscription-ID			[31] SEQUENCE OF SubscriptionID OPTIONAL,

servedPartyIPAddress			[50] ServedPartyIPAddress OPTIONAL,

list-Of-Early-SDP-Media-Components [51] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

ICSCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

list-Of-Associated-URI			[27] ListOfInvolvedParties OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

s-CSCF-Information				[61] S-CSCF-Information OPTIONAL

}

MRFCRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[22] NodeAddress OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

list-of-subscription-ID			[31] SEQUENCE OF SubscriptionID OPTIONAL,

applicationServersInformation	[40] SEQUENCE OF ApplicationServersInformation OPTIONAL,

service-Id						[70] Service-Id OPTIONAL,

requested-Party-Address			[71] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity [72] ListOfInvolvedParties OPTIONAL,

list-Of-Early-SDP-Media-Components [73] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

MGCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

trunkGroupID					[80] TrunkGroupID OPTIONAL,

bearerService					[81] TransmissionMedium OPTIONAL,

list-Of-Early-SDP-Media-Components [82] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

BGCFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

list-Of-Early-SDP-Media-Components [31] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

ASRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method OPTIONAL,

role-of-Node					[3] Role-of-Node OPTIONAL,

nodeAddress						[4] NodeAddress OPTIONAL,

session-Id						[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address	[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address			[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp			[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp	[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp		[11] TimeStamp OPTIONAL,

recordOpeningTime				[12] TimeStamp OPTIONAL,

recordClosureTime				[13] TimeStamp OPTIONAL,

interOperatorIdentifiers		[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber		[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[16] INTEGER OPTIONAL,

causeForRecordClosing			[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication		[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier			[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components	[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress						[22] NodeAddress OPTIONAL,

serviceReasonReturnCode			[23] UTF8String OPTIONAL,

list-Of-Message-Bodies			[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions				[25] ManagementExtensions OPTIONAL,

expiresInformation				[26] INTEGER OPTIONAL,

event							[28] UTF8String OPTIONAL,

accessNetworkInformation		[29] OCTET STRING OPTIONAL,

serviceContextID				[30] ServiceContextID OPTIONAL,

list-of-subscription-ID			[31] SEQUENCE OF SubscriptionID OPTIONAL,

serviceSpecificInfo				[100] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

requested-Party-Address			[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity [102] ListOfInvolvedParties OPTIONAL,

list-Of-Early-SDP-Media-Components [103] SEQUENCE OF Early-Media-Components-List OPTIONAL

}

------------------------------------------------------------------------------

--

--  IMS DATA TYPES

--

------------------------------------------------------------------------------

AccessCorrelationID ::= CHOICE

{

gPRS-Charging-Id				[2] INTEGER OPTIONAL,

accessNetworkChargingIdentifier	[4] GraphicString OPTIONAL

}

ACRInterimLost ::= ENUMERATED

{

no 		(0),

yes 	(1),

unknown (2)

}

ApplicationServersInformation ::= SEQUENCE 
{
	applicationServersInvolved 			[0] NodeAddress OPTIONAL,

applicationProvidedCalledParties	[1] SEQUENCE OF InvolvedParty OPTIONAL

}

CauseForRecordClosing ::= ENUMERATED

{

serviceDeliveryEndSuccessfully	(0),

unSuccessfulServiceDelivery		(1),

timeLimit						(3),

serviceChange					(4), -- e.g. change in media due to Re-Invite

managementIntervention			(5)  -- partial record generation reasons to be added

--  Additional codes are for further study

}

- Early-Media-Components-List ::= SEQUENCE

{

sDP-Offer-Timestamp 	[0] TimeStamp OPTIONAL,

sDP-Answer-Timestamp	[1] TimeStamp OPTIONAL,

sDP-Media-Components	[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag		[3] NULL OPTIONAL,

sDP-Session-Description	[4] SEQUENCE OF GraphicString OPTIONAL

}

IMS-Charging-Identifier ::= OCTET STRING

Incomplete-CDR-Indication ::= SET

{

aCRStartLost	[0] BOOLEAN, –- TRUE if ACR[Start] was lost, FALSE otherwise

aCRInterimLost	[1] ACRInterimLost,

aCRStopLost		[2] BOOLEAN –- TRUE if ACR[Stop] was lost, FALSE otherwise

}

InterOperatorIdentifierlist ::= SEQUENCE OF InterOperatorIdentifiers

InterOperatorIdentifiers ::= SEQUENCE

{

originatingIOI[0] GraphicString OPTIONAL,

terminatingIOI[1] GraphicString OPTIONAL

}

InvolvedParty ::= CHOICE

{

sIP-URI[0] GraphicString, -- refer to rfc3261

tEL-URI[1] GraphicString  -- refer to rfc3966

}

Editor’s note:  the constructs below are imported from the generic module

ListOfInvolvedParties ::= SEQUENCE OF InvolvedParty

Media-Components-List ::= SEQUENCE

{

sIP-Request-Timestamp 	[0] TimeStamp OPTIONAL,

sIP-Response-Timestamp	[1] TimeStamp OPTIONAL,

sDP-Media-Components	[2] SEQUENCE OF SDP-Media-Component OPTIONAL,

mediaInitiatorFlag		[3] NULL OPTIONAL,

sDP-Session-Description	[4] SEQUENCE OF GraphicString OPTIONAL

}

MessageBody ::= SEQUENCE

{

content-Type			[0] GraphicString,

content-Disposition		[1] GraphicString OPTIONAL,

content-Length			[2] INTEGER,

originator				[3] InvolvedParty OPTIONAL

}

Role-of-Node ::= ENUMERATED

{

originating (0),

terminating (1),

proxy       (2),

b2bua       (3)

}

S-CSCF-Information ::=  SEQUENCE

{

mandatoryCapabilities [0] SEQUENCE OF GraphicString OPTIONAL,

optionalCapabilities	 [1] SEQUENCE OF GraphicString OPTIONAL,

serverName 				 [2] GraphicString OPTIONAL

}

SDP-Media-Component ::=  SEQUENCE

{

sDP-Media-Name        	[0] GraphicString OPTIONAL,

sDP-Media-Descriptions	[1] SDP-Media-Description OPTIONAL,

accessCorrelationID			AccessCorrelationID OPTIONAL,

-- [2] is used by gPRS-Charging-Id

-- [4] is used by accessNetworkChargingIdentifier

authorized-QoS        	[3] GraphicString OPTIONAL

}

SDP-Media-Description ::= SEQUENCE OF GraphicString

ServedPartyIPAddress  ::=  IPAddress

Service-Id ::= GraphicString

ServiceSpecificData  ::=  SEQUENCE

ServiceSpecificInfo  ::=  SEQUENCE

{

serviceSpecificData	[0] GraphicString OPTIONAL, 
serviceSpecificType		[1] INTEGER OPTIONAL

}

Session-Id ::= GraphicString

--

-- rfc3261 [110]: example for SIP Call-ID: f81d4fae-7dec-11d0-a765-00a0c91e6bf6@foo.bar.com

--

SIP-Method ::= GraphicString

TransmissionMedium ::= SEQUENCE

{

tMR	[0] OCTET STRING (SIZE (1)) OPTIONAL, -- required TM, refer to ITU-T Q.763

tMU	[1] OCTET STRING (SIZE (1)) OPTIONAL  -- used TM, refer to ITU-T Q.763

}

TrunkGroupID ::= CHOICE

{

incoming	[0] GraphicString,

outgoing	[1] GraphicString

}

END

End of modifications