3GPP TSG-SA5 (Telecom Management)	S5-070192

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0046 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Corrections to BGCF state during PSTN and IMS intiated session establishment and releases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent (njmazzarella@alcatel-lucent.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7 |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | IMS and PSTN Session release flows incorrectly show the BGCF as always in a stateful condition, when it should be stateless . TS 24.229 clause 5.6 states that the BGCF may not remain in the signalling path after the initial INVITE. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add corrections in both PSTN and IMS Session Initiations and Releases stating that the BGCF will not remain in the signalling path nor store interim and release information in its CDR. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | The current flow and CDR/ACR information is incorrect, and can cause incorrect implementation for BGCF status during session releases. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.1.1, 5.2.2.1.7, 5.2.2.1.8, 5.2.2.1.9, 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | This case was verified through incoming CT1 LS to SA5 SWGB in S5-070065 (C1-062275) |  |  |  |  |  |  |  |  |

Triggering SIP Method /ISUP message1st Modified Section

Table 5.2.1.1: Accounting Request messages triggered by SIP Methods or ISUP Messages
for all IMS nodes except for MRFC and AS

| ACR [Start] | SIP 200 OK acknowledging an initial SIP INVITE (see note 2) (not applicable for BGCF) |
| --- | --- |
|  | ISUP:ANM (applicable for the MGCF) |
| ACR [Interim] | SIP 200 OK acknowledging a SIP RE-INVITE or SIP UPDATE [e.g. change in media components] (see note 2) |
|  | Expiration of AVP [Acct-Interim-Interval] (see note 2) |
| ACR [Stop] | SIP BYE message (both normal and abnormal session termination cases) (see note 2) (not applicable for BGCF) |
|  | ISUP:REL (applicable for the MGCF) |
| ACR [Event] | SIP 200 OK acknowledging non-session related SIP messages, which are: |
|  | SIP NOTIFY (see note 1 and note 2) |
|  | SIP MESSAGE |
|  | SIP REGISTER (see note 1) |
|  | SIP SUBSCRIBE (see note 3) |
|  | SIP PUBLISH |
|  | SIP 200 OK acknowledging an initial SIP INVITE (only for BGCF) |
|  | SIP 202 Accepted acknowledging a SIP REFER or any other method |
|  | SIP Final Response 2xx (except SIP 200 OK) |
|  | SIP Final/Redirection Response 3xx |
|  | SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful SIP session set-up |
|  | SIP Final Response (4xx, 5xx or 6xx), indicating an unsuccessful session-unrelated procedure |
|  | SIP CANCEL, indicating abortion of a SIP session set-up (see note 2) |
|  | I-CSCF completing a Cx Query that was issued in response to a SIP INVITE |
| NOTE 1: SIP REGISTER with its "Expires" header field or "Expires" parameter equal to 0 or local deregistration due to expiry means Deregistration (see 3GPP TS 24.229 [204]). NOTE 2: (only for I-CSCF): This trigger may only occur if I-CSCF is acting in THIG mode. NOTE 3: SIP SUBSCRIBE with the field "Expires" set to 0 means unsubscribe. |  |

Next Modified Section

##### 5.2.2.1.7	Session Establishment - IMS Initiated

The following figure shows the Diameter transactions that are required between BGCF, MGCF and CDF during session establishment initiated from the IMS side.

- Figure : Message Sequence Chart for Session Establishment (IMS Initiated)

1.	The session is originated from the IMS.

2.	A session towards PSTN is established.

3.	The destination party answers and an answer message are received.

4.	A final response message is sent to the session originator.

5.	Upon reception of the answer message, the MGCF sends an Accounting-Request with
Accounting-Record-Type indicating START_RECORD to record start of a user session and start of a media component in the MGCF CDR.

6.	The CDF acknowledges the reception of the data and opens a MGCF CDR.

7.	Upon reception of the 200 OK message, the BGCF sends an Accounting-Request with
Accounting-Record-Type indicating EVENT_RECORD to record start of a user session and start of a media component in the BGCF CDR.

8.	The CDF acknowledges the reception of the data and creates a BGCF CDR.

Next Modified Section

##### 5.2.2.1.8	Session Release - PSTN initiated

The following figure shows the Diameter transactions that are required between MGCF and CDF during a PSTN initiated session release.

- Figure : Message Sequence Chart for Session Release (PSTN initiated)

1.	The session release is initiated from PSTN.

2.	Session release continues within IMS.

3.	The reception of the release message is acknowledged.

4.	Upon reception of the release message, the MGCF sends an Accounting-Request with
Accounting-Record-Type indicating STOP_RECORD to record stop of a session in the MGCF CDR.

5.	The CDF acknowledges the reception of the data and closes the MGCF CDR.

Next Modified Section

##### 5.2.2.1.9	Session Release - IMS Initiated

The following figure shows the Diameter transactions that are required between MGCF and CDF during a IMS initiated session release.

- Figure : Message Sequence Chart for Session Release (IMS initiated)

1.	The session release is initiated from the IMS side.

2.	A release message is sent towards PSTN.

3.	The acknowledgement of the release message is received from PSTN.

4.	Upon reception of the BYE message the MGCF sends an Accounting Request with Accounting Record Type indicating STOP_RECORD to record stop of a session in the MGCF CDR.

5.	The CDF acknowledges the reception of the data and closes the MGCF CDR.

Next Modified Section

### 6.3.2	Detailed Message Format for offline charging

The following chapter specifies per Operation Type the charging data that are sent by each of the IMS network elements:

- S-CSCF

- P-CSCF

- CSCF

- MRFC

- MGCF

- BGCF

- AS

The Operation Types are listed in the following order: S (start)/I (interim)/S (stop)/E (event). Therefore, when all Operation Types are possible it is marked as SISE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. SIS or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. SI-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Charging Data Request message for IMS offline charging.

Table 6.3.2.1 : Supported fields in Charging Data Request Message

| Field | Node Type | S-CSCF | P-CSCF | I-CSCF | MRFC | MGCF | BGCF | AS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | E | S/I/S | S/I/S/E | E | S/I/S/E |
| Session Identifier |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Originator Node |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Originator Domain |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Destination Domain |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Operation Type |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Operation Number |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Operation Identifier |  | SISE | SISE | E | SIS | SISE | E | SISE |
| User Name |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Operation Interval |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Origination State |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Origination Timestamp |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Proxy Information |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Route Information |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Operation Token |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Subscriber Identifier |  | SISE | SISE | - | SIS | - | - | SISE |
| Service Information with PS and IMS Information |  |  |  |  |  |  |  |  |
| Event Type |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Role of Node |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Node Functionality |  | SISE | SISE | E | SIS | SISE | E | SISE |
| User Session Id |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Calling Party Address |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Called Party Address |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Requested Party Address |  | S--E | - | - | S-- | - | - | S--E |
| Called Asserted Identity |  | S--E | - | - | S-- | - | - | S--E |
| Associated URI |  | ---E | ---E | E | - | - | - | - |
| Time stamps (see note 3) |  | SISE | SISE | E | SIS | SISE | E | SISE |
| Application server Information (see note 1) |  | SISE | - | - | SIS | - | - | - |
| Inter Operator Identifiers (see note 1) |  | SISE | SISE | E | SIS | SISE | E | SISE |
| IMS Charging Identifier |  | SISE | SISE | E | SIS | SISE | E | SISE |
| SDP Session Description |  | SI-- | SI-- | - | SI-- | SI-- | - | SI-- |
| SDP Media Component |  | SI-- | SI-- |  | SI-- | SI-- | - | SI-- |
| GGSN Address |  | SI-- | SI-- |  | SI-- | SI-- | - | SI-- |
| Served Party (see note 1) |  | - | SISE | - | - | - | - | - |
| Authorized QoS (see note 1) |  | - | SI-- | - | - | - | - | - |
| Server Capabilities (see note 1) |  | - | - | E | - | - | - | - |
| Trunk Group ID (see note 1) |  | - | - | - | - | SISE | - | - |
| Bearer Service (see note 1) |  | - | - | - | - | SISE | - | - |
| Service Id (see note 1) |  | - | - | - | SIS | - | - | - |
| Service Specific Info (see note 1) |  | - | - | - | - | - | - | SISE |
| Message Bodies (see note 2) |  | SISE | SISE |  |  | SISE |  | SISE |
| Cause Code |  | --SE | --SE | E | --S | --SE | E | --SE |
| Access Network Information |  | SISE | SISE | E | SIS | SISE | E | SISE |
| NOTE 1: Only present if available in the CTF of the IMS node. NOTE 2: Present only if Messages Bodies is included in the SIP message that triggered the Charging Data Request. NOTE 3: Only present if ACR is triggered on a SIP message (e.g. SIP INVITE, SIP UPDATE). |  |  |  |  |  |  |  |  |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Charging Data Response message for IMS offline charging.

Table 6.3.2.2 : Supported fields in Charging Data Response Message

| Field | Node Type | S-CSCF | P-CSCF | I-CSCF | MRFC | MGCF | BGCF | AS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Supported Operation Types | S/I/S/E | S/I/S/E | E | S/I/S | S/I/S/E | S/I/S/E | S/I/S/E |
| Session Identifier |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Originator Node |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Originator Domain |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Destination Domain |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Operation Type |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Operation Number |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Operation Identifier |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| User Name |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Operation Interval |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Origination State |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Origination Timestamp |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Proxy Information |  | SISE | SISE | E | SIS | SISE | SISE | SISE |
| Route Information |  | SISE | SISE | E | SIS | SISE | SISE | SISE |

End of modifications

Diameter message