3GPP TSG-SA5 (Telecom Management)	S5-070200

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.272 | CR | 0022 |  rev | - |  Current version: | 7.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add PoC session initiation type into PoC-information - Alignment with OMA PoC Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei (Cloud.zhao@huawei.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7-PoC-CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | C |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | As described in 32.272 Sec.5.1.1 (“PoC session related charging”), two session establishment types are supported, by which PoC Server can provide for users different call latency, QoE, etc. This should be considered in charging. See also details in OMA-TS-PoC-ControlPlane-V1_0. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add PoC Session Initiation Type to identify the initiation type each participant chooses when initiating session. The value types can be: Pre-established or On-demand |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inability to identify which type of session a PoC subscriber uses to initiate a PoC session. Misalignment with OMA PoC Requirements |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  | X |  | O&M Specifications |  |  | 32.298, 32.299 |  |  |  |
| Other comments:  |  | Related R7 CR 32.298 in S5-070201 Related R7 CR 32.299 in S5-070203 |  |  |  |  |  |  |  |  |

1st Modified Section

#### 6.3.1.2	Definition of the PoC Information

PoC specific charging information is provided within the PoC Information. The detailed structure of the PoC-Information can be found in table 6.3.1.2.

Table 6.3.1.2: Structure of the PoC_Information

| Field | Category | Description |
| --- | --- | --- |
| PoC Server Role | OM | Identifies the PoC server as participating or controlling PoC server. |
| PoC User Role | OC | Identifies the PoC user’s role detailed information which should be a list of roles information group. See further details in OMA-AD-POC document [203]. |
| PoC Session Type | OM | Type of the PoC session as defined in appendix C.5.1 in OMA-CP-POC [204]. |
| Number Of Participants | OM | Indicates the number of invited parties of the PoC session when included in the initial charging request message. When included in interim / update charging messages, it indicates the number of parties currently who are attached to the session at the time the interim / update charging messages are sent. |
| List Of Participants | OC | Holds the information for participants, e.g., the addresses, the access priority of the invited parties of the PoC session when included in the initial charging request message. When included in the interim /update charging messages, it holds the addresses and access priority of the parties currently who are attached to the session at the time the interim / update charging messages are sent. |
| Called Party Address | OC | The address (Public User ID, SIP URL, E.164, etc.) of the participants. |
| Participant Access Priority | OC | Indicates the user priority level when participating in the PoC session. |
| PoC Session initiation type | OC | Indicates PoC session initiation type.It can be only used for the served parties. |
| List Of Talk Burst-Exchange | OC | Applicable to offline charging only - a list of changes in charging conditions for the PoC session, each change is time stamped. Charging conditions are used to categorize charging, such as per tariff period or based on the number of participants. A set of charging data (number of talk bursts, talk burst bearer volume, sum of talk bursts time) for sent and received talk burst. |
| PoC Controlling Address | OC | Identifies the PoC server performing the controlling function. This is only included when PoC Server Role indicates "participating". This information may be obtained from the "Contact" header of SIP message received from the controlling PoC function. |
| PoC Group Name | OC | Identifies a pre-arranged group. Included if the session is a pre-arranged group session. This information may be obtained from the "P-Asserted-Identity" header of the SIP message received from the controlling PoC function, or from the "Request-URI" header from the PoC user. |
| PoC Session Id | OC | Uniquely identifies an end-to-end PoC session. Note that the PoC Session-Id may not be available in the initial charging interactions for the PoC session. |
| Served Party | OM | Applicable to offline charging only – holds the identity of the party that the charging information relates to. |

End of modifications