3GPP TSG-SA5 (Telecom Management)	S5-070204

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0064 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add PoC User Access Priority into PoC-information - Alignment with TS 32.272 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei (Cloud.zhao@huawei.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7-PoC-CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | PoC user access priority should be taken into account in charging according to requirements in TS 32.272. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Modify the List Of Participants as a field of type Grouped which holds the sub-fields: Called Party Address and Participant Access Priority to indicate the user access level when participating in a PoC session. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inability to differentiate service level for each PoC user in charging when they are participating in a PoC session with different access priorities. Misalignment with TS 32.272 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.1.4.3.8 5.1.4.3.8A 5.1.4.3.8B 5.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | Related R7 CR 32.272 in S5-061705 and R7 CR 32.299 in S5-061706 were approved at SA#34 |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.1.4.3.8	Participants involved

This field indicates the participants involved in the PoC session.

The field is of type grouped. It contains the participant address (Called party address) and the participant access prioritiy.

Next Modified Section

##### 5.1.4.3.8A	Called Party Address

Called Party Address is of type UTF8String. It indicates address (Public User ID, SIP URL, E.164, etc.) of the participants involved in the PoC session.

Next Modified Section

##### 5.1.4.3.8B	Participant Access Priority

This field indicates the access prioritiy for each participant involved in the PoC session.

Next Modified Section

#### 5.2.4.3	PoC CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.272 [32].

POCChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) pocChargingDataTypes (7) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration, RecordType, Diagnostics, IMSI, IMEI, IPAddress, ISDN-AddressString, LocalSequenceNumber, ManagementExtensions, MSISDN, NodeAddress, RecordingEntity, ServiceKey, TimeStamp, ServiceContextID

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

GSNAddress, NodeID, SGSNPLMNIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (2) asn1Module (0) version1 (0)}

IMS-Charging-Identifier, Incomplete-CDR-Indication, InterOperatorIdentifiers, InvolvedParty, MessageBody, Media-Components-List, Session-Id, SIP-Method

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  POC RECORDS

--

------------------------------------------------------------------------------

POCRecord	::= CHOICE

{

--

-- Record values 80..81 are PoC specific

--

pPFRecord			[80] PPFRecord,

cPFRecord			[81] CPFRecord

}

PPFRecord 	::= SET

{

recordType						[0] RecordType,

retransmission					[1] NULL OPTIONAL,

sIP-Method						[2] SIP-Method	OPTIONAL,

nodeAddress						[3] NodeAddress	OPTIONAL,

session-Id						[4] Session-Id	OPTIONAL,

calling-Party-Address			[5] InvolvedParty	OPTIONAL,

called-Party-Address			[6] InvolvedParty	OPTIONAL,

servedParty					[7] GraphicString	OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp	OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp	OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp	OPTIONAL,

recordOpeningTime				[11] TimeStamp	OPTIONAL,

recordClosureTime				[12] TimeStamp OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber OPTIONAL,

recordSequenceNumber			[15] INTEGER	OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing	OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication	OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier	OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE	OF	Media-Components-List	OPTIONAL,

gGSNaddress						[20] NodeAddress	OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE	OF	MessageBody	OPTIONAL,

userLocationInfo				[23] OctetString	OPTIONAl,

poCInformation					[24] POCInformation	OPTIONAL,

recordExtensions				[25] RecordExtensions	OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

CPFRecord		::=	SET

{

recordType						[0] RecordType,

retransmission					[1] NULL	OPTIONAL,

sIP-Method						[2] SIP-Method	OPTIONAL,

nodeAddress						[3] NodeAddress	OPTIONAL,

session-Id						[4] Session-Id	OPTIONAL,

calling-Party-Address			[5] InvolvedParty	OPTIONAL,

called-Party-Address			[6] InvolvedParty	OPTIONAL,

servedParty					[7] GraphicString	OPTIONAL,

serviceRequestTimeStamp			[8] TimeStamp	OPTIONAL,

serviceDeliveryStartTimeStamp	[9] TimeStamp	OPTIONAL,

serviceDeliveryEndTimeStamp		[10] TimeStamp	OPTIONAL,

recordOpeningTime				[11] TimeStamp	OPTIONAL,

recordClosureTime				[12] TimeStamp	OPTIONAL,

interOperatorIdentifiers		[13] InterOperatorIdentifiers	OPTIONAL,

localRecordSequenceNumber		[14] LocalSequenceNumber	OPTIONAL,

recordSequenceNumber			[15] INTEGER	OPTIONAL,

causeForRecordClosing			[16] CauseForRecordClosing	OPTIONAL,

incomplete-CDR-Indication		[17] Incomplete-CDR-Indication	OPTIONAL,

iMS-Charging-Identifier			[18] IMS-Charging-Identifier	OPTIONAL,

list-Of-SDP-Media-Components	[19] SEQUENCE	OF	Media-Components-List	OPTIONAL,

gGSNaddress						[20] NodeAddress	OPTIONAL,

serviceReasonReturnCode			[21] UTF8String	OPTIONAL,

list-Of-Message-Bodies			[22] SEQUENCE	OF	MessageBody	OPTIONAL,

userLocationInfo				[23] OctetString	OPTIONAl,

poCInformation					[24] POCInformation	OPTIONAL,

recordExtensions				[25] RecordExtensions	OPTIONAL,

serviceContextID				[26] ServiceContextID OPTIONAL

}

------------------------------------------------------------------------------

--

--  PoC DATA TYPES

--

------------------------------------------------------------------------------

AccessPriority			::= ENUMERATED

{

pre-emptive 		   (0),

high			       (1),

normal 			 	   (2),

low				       (3)

}

CauseForRecordClosing	::= ENUMERATED

{

normalRelease					(0),

abnormalRelease					(1),

serviceChange					(2), -- e.g. change in media due to Re-Invite

volumeLimit						(3),

timeLimit						(4),

numberofTalkBurstLimit			(5),

maxChangeCond					(6),

sessionTypeChange				(7),

managementIntervention			(8)

}

ChangeCondition			::= ENUMERATED

{

serviceChange					(0), -- e.g. change in media due to Re-Invite

volumeLimit						(1),

timeLimit						(2),

numberofTalkBurstLimit			(3),

numberofActiveParticipants		(4),

tariffTime						(5)

}

ListofTalkBurstExchange			::= SET

{

number-Of-Talk-Bursts				 [1] INTEGER OPTIONAL,

talk-Burst-Volume					 [2] INTEGER OPTIONAL,		-- measured in octets

talk-Bursts-Time					 [3] CallDuration OPTIONAL,

number-Of-Received-Talk-Bursts		 [4] INTEGER OPTIONAL,

received-Talk-Burst-Volume			 [5] INTEGER OPTIONAL,		-- measured in octets

received-Talk-Burst-Time			 [6] CallDuration OPTIONAL,

changeCondition						 [7] ChangeCondition OPTIONAL,

changeTime							 [8] TimeStamp,

numberofParticipants				 [9] INTEGER OPTIONAL

}

POCInformation			::= SET

{

pOCSessionType					 [1] POCSessionType OPTIONAL,

numberofParticipants			 [2] INTEGER OPTIONAL,

listofParticipants				 [3] SEQUENCE OF POCParticipant OPTIONAL,

listofTalkBurstExchange			 [4] SEQUENCE OF ListofTalkBurstExchange OPTIONAL,

pOCControllingAddress			 [5] UTF8String OPTIONAL,

pOCGroupName					 [6] UTF8String OPTIONAL,

pOCSessionId					 [7] UTF8String OPTIONAL

}

POCParticipant			::= SET

{

called-party-address			[1] InvolvedParty,

participant-access-priority		[2]	AccessPriority	OPTIONAL

}

POCSessionType			::= ENUMERATED

{

1-to-1session              (0),

chat-group-session         (1),

pre-arranged-group-session (2),

ad-hoc-group-session       (3)

}

End of modifications