3GPP TSG-SA5 (Telecom Management) 	Tdoc S5-070228

Meeting #51, Sevilla, Spain, 22 - 26 Jan 2007

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0172 |  rev | - |  Current version: | 7.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Corrections to description of usage of Trigger-Type AVP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Vodafone, Nokia (gavin.wong (at) vodafone.com, benni.alexander@nokia.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | There are contraditions and inaccuracy in Trigger-Type AVP descriptions. The functionality of the OCS controlling the reauthorisations triggered by the CTF is critical to the operation of subscriber specific charging models. The Trigger AVP and Trigger-Type AVP is used to dynamically control the reauthorisations. The Trigger AVP in the CCA is used to control the active set of dynamic triggers (defined in the Trigger-Type AVPs) that the CTF uses to monitor for events that require reauthorisation. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Notations are aligned and clarified the usage (and functionality) of Trigger-Type AVP. Trigger AVP changed as Grouped AVP which contains 0-n Trigger-Type AVPs. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Missing description / specification of the Trigger AVP. Contradictory information would lead to implementation errors. Inability to control the reauthorisations leading to excessive signalling. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.4.2, 6.4.3, 6.5.1.2, 7.2, 7.2.119A and 7.2.120 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | Rel-7 Mirror CR of S5-070319 |  |  |  |  |  |  |  |  |

1st Modified Section

### 6.4.2	Credit-Control-Request Message

The CCR messages, indicated by the Command-Code field set to 272 is sent by the CTF to the OCF in order to request credits for the request bearer / subsystem /service.

The CCR message format is defined according to IETF RFC 4006 [402] as follows:

<CCR> ::= < Diameter Header: 272, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Auth-Application-Id }

{ Service-Context-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ Destination-Host ]

[ User-Name ]

[ CC-Sub-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Origin-State-Id ]

[ Event-Timestamp ]

*[ Subscription-Id ]

[ Service-Identifier ]

[ Termination-Cause ]

[ Requested-Service-Unit ]

[ Requested-Action ]

*[ Used-Service-Unit ]

[ Multiple-Services-Indicator ]

*[ Multiple-Services-Credit-Control ]

*[ Service-Parameter-Info ]

[ CC-Correlation-Id ]

[ User-Equipment-Info ]

*[ Proxy-Info ]

*[ Route-Record ]

[ Service-Information ]

*[ AVP ]

Table 6.4.2 illustrates the basic structure of a 3GPP Diameter Credit Control Credit-Control-Request message as used for Online Charging.

Table 6.4.2: 3GPP Credit-Control-Request Message Content

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Destination-Realm | M | This field contains the realm of the operator domain. The realm will be addressed with the domain address of the corresponding public URI. |
| Auth-Application-Id | M | The field corresponds to the application ID of the Diameter Credit Control Application and is defined with the value 4. |
| Service-Context-Id | M | This field indicates the supported protocol version. |
| CC-Request-Type | M | This field defines the transfer type: event for event based charging and initial, update, terminate for session based charging. |
| CC-Request-Number | M | This field contains the sequence number of the transferred messages. |
| Destination-Host | OC | This field contains the destination peer address of the OCS identity. |
| User-Name | OC | Contains the user name determined by the domain: bearer, sub-system or service as described in middle tier TS. |
| CC-Sub-Session-Id | - | Not used in 3GPP. |
| Acct-Multi-Session-Id | - | Not used in 3GPP. |
| Origin-State-Id | OC | This field contains the state associated to the CTF. |
| Event-Timestamp | OC | This field corresponds to the exact time the quota is requested. |
| Subscription-Id | OM | This field contains the identification of the user that is going to access the service in order to be identified by the OCS. |
| Subscription-Id-Type | OM | This field determines the type of the identifier, e.g. t value 0 is used for the international E.164 format according to ITU-T E.164 numbering plan. |
| Subscription-Id-Data | OM | This field contains the user data content e.g. the MSISDN. |
| Service-Identifier | - | Not used in 3GPP. |
| Termination-Cause | OC | This field contains the reason the credit control session was terminated. |
| Requested-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Requested-Action | OC | The field defines the type of action if the CC-Request-Type indicates EVENT. |
| Used-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Tariff-Change-Usage | - | Not used in 3GPP. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Multiple-Services-Indicator | OM | This field indicates whether the CTF is capable of handling multiple services independently. |
| Multiple-Services-Credit Control | OM | This field contains all parameters for the CTF quota management and defines the quotas to allow traffic to flow. |
| Granted-Service-Unit | - | Not used in CCR. |
| Tariff-Change-Usage | - | Not used in CCR. |
| CC-Time | - | Not used in CCR. |
| CC-Money | - | Not used in CCR. |
| Unit-Value | - | Not used in CCR. |
| Value-Digits | - | Not used in CCR. |
| Exponent | - | Not used in CCR. |
| Currency-Code | - | Not used in CCR. |
| CC-Total-Octets | - | Not used in CCR. |
| CC-Input-Octets | - | Not used in CCR. |
| CC-Output-Octets | - | Not used in CCR. |
| CC-Service-Specific-Units | - | Not used in CCR. |
| AVP | - | Not used in 3GPP. |
| Requested-Service-Unit | OC | This field contains the amount of requested service units for a particular category. |
| CC-Time | OC | This field contains the amount of requested time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the requested amount of octets to be sent and received. |
| CC-Input-Octets | OC | This field contains the requested amount of octets to be received. |
| CC-Output-Octets | OC | This field contains the requested amount of octets to be sent. |
| CC-Service-Specific-Units | OC | This field contains the requested amount of service specific units, e.g. number of events. |
| AVP | - | Not used in 3GPP. |
| Used-Service-Unit | OC | This field contains the amount of used non-monetary service units measured for a particular category to a particular quota type. |
| Reporting-Reason | OC | Used as defined in clause 7.2. |
| Tariff-Change-Usage | OC | This field identifies the reporting period for the used service unit, i.e. before, after or during tariff change. |
| CC-Time | OC | This field contains the amount of used time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the amount of sent and received octets. |
| CC-Input-Octets | OC | This field contains the amount of received octets. |
| CC-Output-Octets | OC | This field contains the amount of sent octets. |
| CC-Service-Specific-Units | OC | This field contains the amount of service specific units, e.g. number of events. |
| Event-Charging-TimeStamp | Oc | Used as defined in clause 7.2. |
| AVP | - | Not used in 3GPP. |
| Tariff-Change-Usage | - | Not used in 3GPP. |
| Service-Identifier | OC | This field contains identity of the used service. This ID with the Service-Context-ID together forms an unique identification of the service. |
| Rating-Group | OC | This field contains the identifier of a rating group. |
| G-S-U-Pool-Reference | - | Not used in CCR. |
| G-S-U-Pool-Identifier | - | Not used in CCR. |
| CC-Unit-Type | - | Not used in CCR. |
| Unit-Value | - | Not used in CCR. |
| Value-Digits | - | Not used in CCR. |
| Exponent | - | Not used in CCR. |
| Validity-Time | - | Not used in CCR. |
| Result-Code | - | Not used in CCR. |
| Final-Unit-Indication | - | Not used in CCR. |
| Final-Unit-Action | - | Not used in CCR. |
| Restriction-Filter-Rule | - | Not used in CCR. |
| Filter-Id | - | Not used in CCR. |
| Redirect-Server | - | Not used in CCR. |
| Redirect-Address-Type | - | Not used in CCR. |
| Redirect-Server-Address | - | Not used in CCR. |
| Time-Quota-Threshold | - | Not used in CCR. |
| Volume-Quota-Threshold | - | Not used in CCR. |
| Quota-Holding-Time | - | Not used in CCR. |
| Quota-Consumption-Time | - | Not used in CCR. |
| Reporting-Reason | OC | Used as defined in clause 7.2. |
| Trigger | OC | Used as defined in clause 7.2. |
| Trigger-Type | OC | Used as defined in clause 7.2. |
| AVP | - | Not used in 3GPP. |
| Service-Parameter-Info | - | Not used in 3GPP. |
| Service-Parameter-Type | - | Not used in 3GPP. |
| Service-Parameter-Value | - | Not used in 3GPP. |
| CC-Correlation-Id | - | Not used in 3GPP. |
| User-Equipment-Info | OC | This field contains the identification of the identity and terminal capability the subscriber is using for the connection to mobile network if available. |
| User-Equipment-Info-Type | M | This field determines the type of the identifier. The used value is 0 for the international mobile equipment identifier and software version according to 3GPP TS 23.003. |
| User-Equipment-Info-Value | M | This field contains the user IMEISV. |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | OC | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| Service-Information | OM | This parameter holds the individual service specific parameters as defined in the corresponding ‘middle tier’ TS. |
| AVP | OC |  |

Next Modified Section

### 6.4.3	Credit-Control-Answer Message

The Credit-Control-Answer (CCA) messages, indicated by the Command-Code field set to 272 is sent by the OCF to the CTF in order to reply to the CCR.

The CCA message format is defined according to IETF RFC 4006 [402] as follows:

<CCA> ::=  < Diameter Header: 272, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Auth-Application-Id }

{ CC-Request-Type }

{ CC-Request-Number }

[ User-Name ]

[ CC-Session-Failover ]

[ CC-Sub-Session-Id ]

[ Acct-Multi-Session-Id ]

[ Origin-State-Id ]

[ Event-Timestamp ]

[ Granted-Service-Unit ]

*[ Multiple-Services-Credit-Control ]

[ Cost-Information]

[ Final-Unit-Indication ]

[ Check-Balance-Result ]

[ Credit-Control-Failure-Handling ]

[ Direct-Debiting-Failure-Handling ]

[ Validity-Time]

*[ Redirect-Host]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ Failed-AVP ]

[ Service-Information ]

*[ AVP ]

Table 6.4.3 illustrates the basic structure of a 3GPP Diameter Credit-Control Credit-Control-Answer message as used for online charging. This message is always used by the OCF as specified below, independent of the receiving CTF and the CCR record type that is being replied to.

Table 6.4.3: 3GPP Credit-Control-Answer Message Content

| AVP | Category | Description |
| --- | --- | --- |
| Session-Id | M | This field identifies the operation session. |
| Result-Code | M | This field contains the result of the specific query. |
| Origin-Host | M | This field contains the identification of the source point of the operation and the realm of the operation originator. |
| Origin-Realm | M | This field contains the realm of the operation originator. |
| Auth-Application-Id | M | The field corresponds to the application ID of the Diameter Credit Control Application and is defined with the value 4. |
| CC-Request-Type | M | This field defines the transfer type: initial, update, terminate for session based charging and event for event based charging. |
| CC-Request-Number | M | This field contains the sequence number of the transferred messages. |
| User-Name | - | Not used in 3GPP. |
| CC-Session Failover | OC | This field contains an indication to the CTF whether or not a failover handling is to be used when necessary. |
| CC-Sub-session-Id | - | Not used in 3GPP. |
| Acct-Multi-Session-Id | - | Not used in 3GPP. |
| Origin-State-Id | - | Not used in 3GPP. |
| Event-Timestamp | - | Not used in 3GPP. |
| Granted-Service-Unit | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Tariff-Time-Change | - | Not used in 3GPP. |
| CC-Time | - | Not used in 3GPP. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | - | Not used in 3GPP. |
| CC-Input-Octets | - | Not used in 3GPP. |
| CC-Output-Octets | - | Not used in 3GPP. |
| CC-Service-Specific-Units | - | Not used in 3GPP. |
| AVP | - | Not used in 3GPP. |
| Multiple-Services-Credit-Control | OM | This field contains all parameters for the CTF quota management and defines the quotas to allow traffic to flow. |
| Granted-Service-Unit | OC | This field contains the amount of granted service units for a particular category. |
| Tariff-Time-Change | OC | This field identifies the reporting period for the granted service units, i.e. before, after or during tariff change. |
| CC-Time | OC | This field contains the amount of granted time. |
| CC-Money | - | Not used in 3GPP. |
| Unit-Value | - | Not used in 3GPP. |
| Value-Digits | - | Not used in 3GPP. |
| Exponent | - | Not used in 3GPP. |
| Currency-Code | - | Not used in 3GPP. |
| CC-Total-Octets | OC | This field contains the amount for sent and received octets. |
| CC-Input-Octets | OC | This field contains the amount for received octets. |
| CC-Output-Octets | OC | This field contains the amount for sent octets. |
| CC-Service-Specific-Units | OC | This field contains the amount for service specific units, e.g. number of events. |
| AVP | - | Not used in CCA. |
| Requested-Service-Unit | - | Not used in CCA. |
| Tariff-Time-Change | - | Not used in CCA. |
| CC-Time | - | Not used in CCA. |
| CC-Money | - | Not used in CCA. |
| Unit-Value | - | Not used in CCA. |
| Value-Digits | - | Not used in CCA. |
| Exponent | - | Not used in CCA. |
| Currency-Code | - | Not used in CCA. |
| CC-Total-Octets | - | Not used in CCA. |
| CC-Input-Octets | - | Not used in CCA. |
| CC-Output-Octets | - | Not used in CCA. |
| CC-Service-Specific-Units | - | Not used in CCA. |
| Used-Service-Unit | - | Not used in CCA. |
| Tariff-Time-Change | - | Not used in CCA. |
| CC-Time | - | Not used in CCA. |
| CC-Money | - | Not used in CCA. |
| Unit-Value | - | Not used in CCA. |
| Value-Digits | - | Not used in CCA. |
| Exponent | - | Not used in CCA. |
| Currency-Code | - | Not used in CCA. |
| CC-Total-Octets | - | Not used in CCA. |
| CC-Input-Octets | - | Not used in CCA. |
| CC-Output-Octets | - | Not used in CCA. |
| CC-Service-Specific-Units | - | Not used in CCA. |
| Tariff-Change-Usage | - | Not used in 3GPP. |
| Service-Identifier | OC | This field contains identity of the used service. This ID with the Service-Context-ID together forms an unique identification of the service. |
| Rating-Group | OC | This field contains the identifier of a rating group. |
| G-S-U-Pool-Reference | OC | Only used in ECUR and SCUR. |
| G-S-U-Pool-Identifier | M |  |
| CC-Unit-Type | M |  |
| Unit-Value | M |  |
| Value-Digits | M |  |
| Exponent | OC |  |
| Validity-Time | OC | This field defines the time in order to limit the validity of the granted quota for a given category instance. |
| Result-Code | OC | This field contains the result of the query. |
| Final-Unit-Indication | OC | This field indicates that the Granted-Service-Unit containing the final units for the service. |
| Final-Unit-Action | OC |  |
| Restriction-Filter-Rule | OC |  |
| Filter-Id | OC |  |
| Redirect-Server | OC |  |
| Redirect-Address-Type | M |  |
| Redirect-Server-Address | M |  |
| Time-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Volume-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Unit-Quota-Threshold | OC | Used as defined in clause 7.2. |
| Quota-Holding-Time | OC | Used as defined in clause 7.2. |
| Quota-Consumption-Time | OC | Used as defined in clause 7.2. |
| Reporting-Reason | - | Not used in CCA. |
| Trigger | OC | Used as defiined in clause 7.2. |
| Trigger-Type | OC | Used as defined in clause 7.2. |
| PS-Furnish-Charging-Information | OC | Used as defined in clause 7.2. |
| AVP | - | Not used in 3GPP. |
| Cost-Information | OC | Used as defined in DCCA [402]. |
| Unit-Value | M | Used as defined in DCCA [402]. |
| Value-Digits | M | Used as defined in DCCA [402]. |
| Exponent | OC | Used as defined in DCCA [402]. |
| Currency-Code | M | Used as defined in DCCA [402]. |
| Cost-Unit | OC | Used as defined in DCCA [402]. |
| Final-Unit-Indication | - | Not used in 3GPP, see Multiple-Services-Credit-Control. |
| Final-Unit-Action | - | Not used in 3GPP. |
| Restriction-Filter-Rule | - | Not used in 3GPP. |
| Filter-Id | - | Not used in 3GPP. |
| Redirect-Server | - | Not used in 3GPP. |
| Redirect-Address-Type | - | Not used in 3GPP. |
| Redirect-Server-Address | - | Not used in 3GPP. |
| Check-Balance-Result | - | Not used in 3GPP. |
| Credit-Control-Failure-Handling | OC | Used as defined in DCCA [402]. |
| Direct-Debiting-Failure-Handling | OC | Used as defined in DCCA [402]. |
| Validity-Time | - | Not used in 3GPP. |
| Redirect-Host | OC |  |
| Redirect-Host-Usage | OC |  |
| Redirect-Max-Cache-Time | OC |  |
| Proxy-Info | OC | This field contains information of the host. |
| Proxy-Host | M | This field contains the identity of the host that added the Proxy-Info field. |
| Proxy-State | M | This field contains state local information. |
| Route-Record | OC | This field contains an identifier inserted by a relaying or proxying node to identify the node it received the message from. |
| Failed-AVP | OC |  |
| Service-Information | OC | This parameter holds the individual service specific parameters as defined in the corresponding ‘middle tier’ TS. |
| AVP | OC |  |

Next Modified Section

#### 6.5.1.2	Change of charging conditions

There are a number of mid-session service events (re-authorisation triggers), which could affect the rating of the current service usage, e.g. end user QoS changes or location updates. When allocating resources, the server may instruct the credit control client to re-authorize the quota upon a number of different session related triggers that can affect the rating conditions. The server instruct the Network Element to monitor for such events by using the Trigger AVP containing one or more Trigger-Type AVPs in the CCA command. These events are in addition to the static triggers defined in the service specific document (middle tier TS).

Once the OCS has armed one or more triggers using the Trigger AVP at the Network Element, these triggers shall remain in effect until another Trigger AVP is received for the same Rating Group, where the Network Element shall arm all triggers present in the Trigger AVP and reset all other triggers. The presence of the Trigger AVP without any Trigger-Type AVPs in a CCA allows OCS to disable all the triggers that were armed in a previous Trigger AVP.

NOTE:	This removes the need for the OCS to send trigger information in every CCA message when they have not changed.

When one of the armed triggers happen, a credit re-authorization shall be sent to the server including information related to the service event even if all the granted service units have not been used. The quota is also being reported.

For example, if the Trigger AVP is used, then the client shall only re-authorise the quota for the service usage associated with  events which were included in the last received Trigger AVP.

If the server does not control the events for re-authorisation using the Trigger AVP, the Network Element shall only monitor for default events defined in the relevant service specific document (middle tier TS).

Multiple triggers monitoring may be associated to a single quota allocation by including multiple Trigger-Type AVPs.

Next Modified Section

## 7.2	3GPP specific AVPs

For the purpose of offline charging additional AVPs are used in ACR / ACA and for online charging additional AVPs are used in CCR / CCA. All 3GPP specific AVPs mentioned are relevant for both offline and online charging unless specifically excluded. The information is summarized in the following table along with the AVP flag rules.

The 3GPP Charging Application uses the value 10415 (3GPP) as Vendor-Id.

Detailed descriptions of AVPs that are used specifically for 3GPP charging are provided in the subclauses below the table. However, for AVPs that are just borrowed from other applications only the reference (e.g. TS 29.229 [204]), is provided in the following table and the detailed description is not repeated.

Where 3GPP RADIUS VSAs are re-used, they shall be translated to Diameter AVPs as described in IETF RFC 4005 [407] with the exception that the 'M' flag shall be set and the ''P' flag may be set.

Table 7.2: 3GPP specific AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not | May Encr. |
| 3GPP-CAMEL-Charging-Info | 24 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Charging-Characteristics | 13 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Charging-Id | 2 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-GGSN- MCC-MNC | 9 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-GPRS-Negotiated-QoS-Profile | 5 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-IMSI-MCC-MNC | 8 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-MS-TimeZone | 23 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-NSAPI | 10 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-PDP-Type | 3 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-RAT-Type | 21 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Selection-Mode | 12 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-Session-Stop-Indicator | 11 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-SGSN-MCC-MNC | 18 | - | - | X | - | refer [207] |  |  |  |  |  |
| 3GPP-User-Location-Info | 22 | - | - | X | - | refer [207] |  |  |  |  |  |
| Access-Network-Charging-Identifier-Value | 503 | X | - | X | - | refer [214] |  |  |  |  |  |
| Access-Network-Information | 1263 | X | - | X | - | OctetString | V,M | P |  |  | N |
| Adaptations | 1217 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Additional-Content-Information | 1207 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Additional-Type-Information | 1205 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Data | 897 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Address-Domain | 898 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Address-Type | 899 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Addressee-Type | 1208 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Applic-ID | 1218 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-provided-called-party-address | 837 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server | 836 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Application-Server-Information | 850 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Associated-URI | 856 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Authorized-QoS | 849 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Aux-Applic-Info | 1219 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Bearer-Service | 854 | X | - | - | - | OctetString | V,M | P |  |  | N |
| Called-Party-Address | 832 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Calling-Party-Address | 831 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Called-Asserted-Identity | 1250 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Cause-Code | 861 | X | - | X | - | Integer32 | V,M | P |  |  | N |
| CG-Address | 846 | X | - | X | - | Address | V,M | P |  |  | Y |
| Charged-Party | 857 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Charging-Rule-Base-Name | 1004 | - | - | X | - | refer [205] |  |  |  |  |  |
| Class-Identifier | 1214 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Class | 1220 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Content-Disposition | 828 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Content-Length | 827 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Size | 1206 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Content-Type | 826 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Deferred-Location-Event-Type | 1230 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Delivery-Report-Requested | 1216 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Domain-Name | 1200 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| DRM-Content | 1221 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Event | 825 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Event-Type | 823 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Event-Charging-TimeStamp | 1258 | - | - | X | - | Time | V,M | P |  |  | N |
| Expires | 888 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| File-Repair-Supported | 1224 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| GGSN-Address | 847 | X | - | X | - | Address | V,M | P |  |  | N |
| IMS-Charging-Identifier | 841 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| IMS-Information | 876 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Incoming-Trunk-Group-Id | 852 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Inter-Operator-Identifier | 838 | X | - | - | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Dialed-By-MS | 1233 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-External-ID | 1234 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Id | 1232 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Name | 1231 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Client-Name | 1235 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Client-Type | 1241 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Data-Coding-Scheme | 1236 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Format-Indicator | 1237 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| LCS-Information | 878 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Name-String | 1238 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| LCS-Requestor-Id | 1239 | - | - | X | - | Grouped | V,M | P |  |  | N |
| LCS-Requestor-Id-String | 1240 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Location-Estimate | 1242 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Location-Estimate-Type | 1243 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Location-Type | 1244 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Mandatory-Capability | 604 | X | - | - | - | refer [204] |  |  |  |  |  |
| Media-Initiator-Flag | 882 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Message-Body | 889 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MBMS-Information | 880 | X | - | X | - | Grouped | V,M | P |  |  | N |
| MBMS-Service-Area | 903 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Session-Identity | 908 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-Service-Type | 906 | X | - | X | - | refer [207] |  |  |  |  |  |
| MBMS-User-Service-Type | 1225 | X | - | X | - | Enumerated | V,M | P |  |  | Y |
| MBMS-2G-3G-Indicator | 907 | X | - | X | - | refer [207] |  |  |  |  |  |
| Message-Class | 1213 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Message-ID | 1210 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Message-Type | 1211 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Message-Size | 1212 | - | - | X | - | Unsigned32 | V,M | P |  |  | N |
| MMBox-Storage-Requested | 1248 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| MM-Content-Type | 1203 | - | - | X | - | Grouped | V,M | P |  |  | N |
| MMS-Information | 877 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Node-Functionality | 862 | X | -. | X | - | Enumerated | V,M | P |  |  | N |
| Number-Of-Participants | 885 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| Optional-Capability | 605 | X | - | - | - | refer [204] |  |  |  |  |  |
| Originating-IOI | 839 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Originator | 864 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Originator-Address | 886 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Outgoing-Trunk-Group-Id | 853 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Participants-Involved | 887 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Participant-Access-Priority | 1259 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Participant-Group | 1260 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PDG-Address | 895 | X | - | X | - | Address | V,M | P |  |  | N |
| PDG-Charging-Id | 896 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| PDP-Address | 1227 | - | - | X | - | Address | V,M | P |  |  | Y |
| PDP-Context-Type | 1247 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Condition | 1261 | X | - | - | - | Enumerated | V,M | P |  |  | N |
| PoC-Change-Time | 1262 | X | - | - | - | Time | V,M | P |  |  | N |
| PoC-Controlling-Address | 858 | X |  | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Group-Name | 859 | X |  | X |  | UTF8String | V,M | P |  |  | N |
| PoC-Information | 879 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-Server-Role | 883 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-Session-Id | 1229 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-Session-Type | 884 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| PoC-User-Role | 1252 | X | - | X | - | Grouped | V,M | P |  |  | N |
| PoC-User-Role-IDs | 1253 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| PoC-User-Role-info-Units | 1254 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| Positioning-Data | 1245 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Priority | 1209 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| PS-Append-Free-Format-Data | 867 | X | - | - | X | Enumerated | V,M | P |  |  | N |
| PS-Free-Format-Data | 866 | X | - | - | X | OctetString | V,M | P |  |  | N |
| PS-Furnish-Charging-Information | 865 | X | - | - | X | Grouped | V,M | P |  |  | N |
| PS-Information | 874 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Quota-Consumption-Time | 881 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Quota-Holding-Time | 871 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| RAI | 909 | X | - | X | - | refer [207] |  |  |  |  |  |
| Read-Reply-Report-Requested | 1222 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Recipient-Address | 1201 | - | - | X | - | Grouped | V,M | P |  |  | N |
| Reply-Applic-ID | 1223 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Reporting-Reason | 872 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Requested-Party-Address | 1251 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Required-MBMS-Bearer-Capabilities | 901 | X | - | X | - | refer [207] |  |  |  |  |  |
| Role-of-Node | 829 | X | - | X | - | Enumerated | V,M | P |  |  | N |
| SDP-Media-Component | 843 | X | - | X | - | Grouped | V,M | P |  |  | N |
| SDP-Media-Description | 845 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Media-Name | 844 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SDP-Session-Description | 842 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Served-Party-IP-Address | 848 | X | - | - | - | Address | V,M | P |  |  | N |
| Server-Capabilities | 603 | X | - | - | - | refer [204] |  |  |  |  |  |
| Server-Name | 602 | X | - | - | - | refer [204] |  |  |  |  |  |
| Service-Generic-Information | 1256 | X | - | X | - | Grouped | V,M | P |  |  | N |
| Service-Id | 855 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Service-Information | 873 | X | - | X | X | Grouped | V,M | P |  |  | N |
| Service-Specific-Data | 863 | X | - | - | - | UTF8String | V,M | P |  |  | N |
| Service-Specific-Type | 1257 | X | - | - | - | Unsigned32 | V,M | P |  |  | N |
| Service-Specific-Info | 1249 | X | - | - | - | Grouped | V,M | P |  |  | N |
| SGSN-Address | 1228 | X | - | X | - | Address | V,M | P |  |  | N |
| SIP-Method | 824 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| SIP-Request-Timestamp | 834 | X | - | X | - | Time | V,M | P |  |  | N |
| SIP-Response-Timestamp | 835 | X | - | X | - | Time | V,M | P |  |  | N |
| Submission-Time | 1202 | - | - | X | - | Time | V,M | P |  |  | N |
| Talk-Burst-Exchange | 1255 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Terminating-IOI | 840 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| Time-Quota-Threshold | 868 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| Time-Stamps | 833 | X | - | X | - | Grouped | V,M | P |  |  | N |
| TMGI | 900 | X | - | X | - | refer [207] |  |  |  |  |  |
| Token-Text | 1215 | - | - | X | - | UTF8String | V,M | P |  |  | N |
| Trigger | 1264 | - | - | X | X | Grouped | V,M | P |  |  | N |
| Trigger-Type | 870 | - | - | X | X | Enumerated | V,M | P |  |  | N |
| Trunk-Group-Id | 851 | X | - | - | - | Grouped | V,M | P |  |  | N |
| Type-Number | 1204 | - | - | X | - | Enumerated | V,M | P |  |  | N |
| Unit-Quota-Threshold | 1226 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| User-Data | 606 | X | - | - | - | refer [204] | V,M | P |  |  | N |
| User–Session-Id | 830 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| VAS-Id | 1102 | - | - | X | - | refer [213] |  |  |  |  |  |
| VASP-Id | 1101 | - | - | X | - | refer [213] |  |  |  |  |  |
| Volume-Quota-Threshold | 869 | - | - | - | X | Unsigned32 | V,M | P |  |  | N |
| WAG-Address | 890 | X | - | X | - | Address | V,M | P |  |  | N |
| WAG-PLMN-Id | 891 | X | - | X | - | OctetString | V,M | P |  |  | N |
| WLAN-Information | 875 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Radio-Container | 892 | X | - | X | - | Grouped | V,M | P |  |  | N |
| WLAN-Session-Id | 1246 | X | - | X | - | UTF8String | V,M | P |  |  | N |
| WLAN-Technology | 893 | X | - | X | - | Unsigned32 | V,M | P |  |  | N |
| WLAN-UE-Local-IPAddress | 894 | X | - | X | - | Address | V,M | P |  |  | N |

Next Modified Section

### 7.2.119A	 Trigger AVP

The Trigger AVP (AVP code 1264) is of type Grouped and holds the trigger types. The presence of the Trigger AVP without any Trigger-Type AVP in a CCA allows OCS to disable all the triggers. The presence of the Trigger AVP in the CCR identifies the event(s) triggering the CCR.

It has the following ABNF grammar:

<Trigger>::		=		< AVP Header: 1264 >

* [ Trigger-Type ]

End of modifications