3GPP TSG-SA5 (Telecom Management) 	Tdoc  S5-070251

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0174 |  rev | - |  Current version: | 7.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Alignment of Online Charging Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia (benni.alexander@nokia.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Existing sections within the TS are not aligned leading to inconsistent descriptions and subsequently errors. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Table 7.1 needs alignment with previous chapters (6.3.10 and 6.4.3). |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Misalignment of information could lead to incorrect implementations of online charging client. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | R7 Mirror CR of S5-070250 |  |  |  |  |  |  |  |  |

1st Modified Section

# 7	Summary of used Attribute Value Pairs

## 7.1	Diameter AVPs

The use of the Attribute Value Pairs (AVPs) that are defined in the Diameter Protocol is specified in clause 6.2 for offline charging and in clause 6.4 for online charging. The information is summarized in the table 7.1 in alphabetical order. Detailed specification of some of these AVPs is available after the table and for the others can be found from IETF RFC 3588 [401] and IETF RFC 4006 [402].

Those Diameter AVPs that are used are marked ”M”, “OM“or “Oc” in the following table. This implies that their content can be used by the CDF for offline and by the OCF for online charging purposes. Those Diameter AVPs that are not used are marked "-" in the following table.

Table 7.1: Use Of IETF Diameter AVPs

| AVP Name | AVP Code | Used in |  |  |  | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ACR | ACA | CCR | CCA |  | Must | May | Should not | Must not | May Encr. |
| Accounting-Realtime-Required | 483 | - | - | - | - | Enumerated | - | - | - | - | - |
| Accounting-Record-Number | 485 | M | M | - | - | Unsigned32 | M | P | - | V | Y |
| Accounting-Record-Type | 480 | M | M | - | - | Enumerated | M | P | - | V | Y |
| Accounting-Sub-Session-Id | 287 | - | - | - | - | Unsigned64 | - | - | - | - | - |
| Acct-Application-Id | 259 | OC | OC | - | - | Unsigned32 | M | P | - | V | N |
| Acct-Interim-Interval | 85 | OC | OC | - | - | Unsigned32 | M | P | - | V | Y |
| Acct-Multi-Session-Id | 50 | - | - | - | - | Unsigned32 | - | - | - | - | - |
| Acct-Session-Id | 44 | - | - | - | - | OctetString | - | - | - | - | - |
| Auth-Application-Id | 258 | - | - | M | M | Unsigned32 | M | P | - | V | N |
| AVP | * | - | - | - | - | Grouped | - | - | - | - | - |
| Called-Station-Id | 30 | OC | - | OC | - | OctetString | M | P | - | V | N |
| CC-Correlation-Id | 411 | - | - | - | - | OctetString | - | - | - | - | - |
| CC-Input-Octets | 412 | - | - | OC | OC | Unsigned64 | - | P,M | - | V | Y |
| CC-Money | 413 | - | - | - | - | Grouped | - | - | - | - | - |
| CC-Output-Octets | 414 | - | - | OC | OC | Unsigned64 | M | P | - | V | Y |
| CC-Request-Number | 415 | - | - | M | M | Unsigned32 | M | P | - | V | Y |
| CC-Request-Type | 416 | - | - | M | M | Enumerated | M | P | - | V | Y |
| CC-Service-Specific-Units | 417 | - | - | OC | OC | Unsigned64 | M | P | - | V | Y |
| CC-Session-Failover | 418 | - | - | - | OC | Enumerated | - | - | - | - | - |
| CC-Sub-Session-Id | 419 | - | - | - | - | Unsigned64 | - | - | - | - | - |
| CC-Time | 420 | - | - | OC | OC | Unsigned32 | M | P | - | V | Y |
| CC-Total-Octets | 421 | - | - | OC | OC | Unsigned64 | M | P | - | V | Y |
| CC-Unit-Type | 454 | - | - | - | OC | Enumerated | - | - | - | - | - |
| Check-Balance-Result | 422 | - | - | - | - | Enumerated | - | - | - | - | - |
| Cost-Information | 423 | - | - | - | OC | Grouped | M | P | - | V | Y |
| Cost-Unit | 424 | - | - | - | OC | UTF8String | M | P | - | V | Y |
| Credit-Control | 426 | - | - | - | - | Enumerated | - | - | - | - | - |
| Credit-Control-Failure-Handling | 427 | - | - | - | OC | Enumerated | M | P | - | V | Y |
| Currency-Code | 425 | - | - | - | M | Unsigned32 | M | P | - | V | Y |
| Destination-Host | 293 | - | - | OC | - | DiamIdent | M | P | - | V | N |
| Destination-Realm | 283 | M | - | M | - | DiamIdent | M | P | - | V | N |
| Direct-Debiting-Failure-Handling | 428 | - | - | - | OC | Enumerated | M | P | - | V | Y |
| Error-Message | 281 | - | - | - | - | UTF8String | - | - | - | - | - |
| Error-Reporting-Host | 294 | - | OC | - | - | DiamIdent | - | P | - | V,M | N |
| Event-Timestamp | 55 | OC | OC | OC | - | Time | M | P | - | V | N |
| Exponent | 429 | - | - | - | OC | Integer32 | M | P | - | V | Y |
| Failed-AVP | 279 | - | - | - | OC | Grouped | M | P | - | V | N |
| Filter-Id | 11 | - | - | - | OC | UTF8String | M | P | - | V | Y |
| Final-Unit-Action | 449 | - | - | - | OC | Enumerated | M | P | - | V | Y |
| Final-Unit-Indication | 430 | - | - | - | OC | Grouped | M | P | - | V | Y |
| Granted-Service-Unit | 431 | - | - | - | OC | Grouped | M | P | - | V | Y |
| G-S-U-Pool-Identifier | 453 | - | - | - | OC | Unsigned32 | M | P | - | V | Y |
| G-S-U-Pool-Reference | 457 | - | - | - | OC | Grouped | M | P | - | V | Y |
| Location-Type | IANA | OC | - | OC | - | refer [403] |  |  |  |  |  |
| Location-Information | IANA | OC | - | OC | - | refer [403] |  |  |  |  |  |
| Multiple-Services-Credit-Control | 456 | - | - | OM | OM | Grouped | M | P | - | V | Y |
| Multiple-Services-Indicator | 455 | - | - | OM | - | Enumerated | M | P | - | V | Y |
| Operator-Name | IANA | OC | - | OC | - | refer [403] |  |  |  |  |  |
| Origin-Host | 264 | M | M | M | M | DiamIdent | M | P | - | V | N |
| Origin-Realm | 296 | M | M | M | M | DiamIdent | M | P | - | V | N |
| Origin-State-Id | 278 | OC | OC | OC | - | Unsigned32 | M | P | - | V | N |
| Proxy-Info | 284 | OC | OC | OC | OC | Grouped | M | - | - | P,V | N |
| Proxy-Host | 280 | M | M | M | M | DiamIdent | M | - | - | P,V | N |
| Proxy-State | 33 | M | M | M | M | OctetString | M | - | - | P,V | N |
| Rating-Group | 432 | - | - | OC | OC | Unsigned32 | M | P | - | V | Y |
| Redirect-Address-Type | 433 | - | - | M | M | Enumerated | M | P | - | V | Y |
| Redirect-Host | 292 | - | - | - | OC | DiamURI | M | P | - | V | N |
| Redirect-Host-Usage | 261 | - | - | - | OC | Enumerated | M | P | - | V | N |
| Redirect-Max-Cache-Time | 262 | - | - | - | OC | Unsigned32 | M | P | - | V | N |
| Redirect-Server | 434 | - | - | - | OC | Grouped | M | P | - | V | Y |
| Redirect-Server-Address | 435 | - | - | - | M | UTF8String | M | P | - | V | Y |
| Requested-Action | 436 | - | - | OC | - | Enumerated | M | P | - | V | Y |
| Requested-Service-Unit | 437 | - | - | OC | - | Grouped | M | P | - | V | Y |
| Restriction-Filter-Rule | 438 | - | - | - | OC | IPFilterRule | M | P | - | V | Y |
| Result-Code | 268 | - | M | - | M | Unsigned32 | M | P | - | V | N |
| Route-Record | 282 | OC | - | OC | OC | DiamIdent | M | - | - | P,V | N |
| Service-Context-Id | 461 | OM | - | M | - | UTF8String | M | P | - | V | Y |
| Service-Identifier | 439 | - | - | OC | OC | Unsigned32 | M | P | - | V | Y |
| Service-Parameter-Info | 440 | - | - | - | - | Grouped | - | - | - | - | - |
| Service-Parameter-Type | 441 | - | - | - | - | Unsigned32 | - | - | - | - | - |
| Service-Parameter-Value | 442 | - | - | - | - | OctetString | - | - | - | - | - |
| Session-Id | 263 | M | M | M | M | UTF8String | M | P | - | V | Y |
| Subscription-Id | 443 | OM | - | OM | - | Grouped | M | P | - | V | Y |
| Subscription-Id-Data | 444 | M | - | M | - | UTF8String | M | P | - | V | Y |
| Subscription-Id-Type | 450 | M | - | M | - | Enumerated | M | P | - | V | Y |
| Tariff-Change-Usage | 452 | - | - | OC | - | Enumerated | M | P | - | V | Y |
| Tariff-Time-Change | 451 | - |  | - | OC | Time | M | P | - | V | Y |
| Unit-Value | 445 | - |  | - | M | Grouped | M | P | - | V | Y |
| Used-Service-Unit | 446 | - |  | OC | - | Grouped | M | P | - | V | Y |
| User-Equipment-Info | 458 | - |  | OC | - | Grouped | - | P,M | - | V | Y |
| User-Equipment-Info-Type | 459 | - |  | M | - | Enumerated | - | P,M | - | V | Y |
| User-Equipment-Info-Value | 460 | - |  | M | - | OctetString | - | P,M | - | V | Y |
| User-Name | 1 | OC | OC | OC | - | UTF8String | M | P | - | V | Y |
| Value-Digits | 447 | - | - | - | M | Integer64 | M | P | - | V | Y |
| Validity-Time | 448 | - | - | - | OC | Unsigned32 | M | P | - | V | Y |
| Vendor-Id | 266 | - | - | - | - | Unsigned32 | - | - | - | - | - |
| Vendor-Specific-Application-Id | 260 | - | - | - | - | Grouped | - | - | - | - | - |

NOTE:	Result-Code AVP is defined in Diameter Base Protocol [401]. However, new values are used in offline and online charging applications. These additional values are defined below.

End of modifications

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2005 | SA_30 | SP 050654 | -- | -- | Created for reference by TISPAN | -- | 6.5.0 | 7.0.0 |
| Mar 2006 | SA_31 | SP-060079 | 0050 | -- | Corrections to LCS AVPs and to Diameter application | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0051 | -- | Correction on threshold reauthorisation trigger for service specific units | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060084 | 0052 | -- | Alignment of MBMS AVPs with 29.061 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0053 | -- | Correction to usage of Event Charging with Unit Reservation (ECUR) | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0054 | -- | Corrections of the usage of AVPs in CCR/CCA messages | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0055 | -- | Corrections to online charging description | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0057 | -- | Consistent use of the Event-Timestamp AVP in the CCR message | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0058 | -- | Correction to bindings for offline charging | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0059 | -- | Correction to bindings for online charging | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060086 | 0060 | -- | Add missing AVP codes for MMS Online charging | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060087 | 0061 | -- | Add missing AVP code definitions for PoC charging | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0063 | -- | Alignment on credit pooling with 32.240 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0064 | -- | Corrections for charging procedures description | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0065 | -- | Correction to session termination and overload protection | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0066 | -- | Correction to Re-authorization Request | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0067 | -- | Correction of AVP Code - Align with IETF RFC 3588 and RFC 4006 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060079 | 0069 | -- | Consistent use of the Cost-Information AVP in the CCA message | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0070 | -- | Correction of AVP type of Content-Length AVP | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0072 | -- | Correction of AVP code definitions for IMS charging - Align with IETF RFC 3261 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0074 | -- | Correction to Cause-Code AVP type - Align with IETF RFC 3588 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0076 | -- | Correction of timestamp data types- Align with RFC 3588 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060081 | 0078 | -- | Correction of User-Name AVP | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0080 | -- | Correction of Terminating Inter Operator Identifier (IOI) AVP | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0082 | -- | Correction of Multiple Service Indicator - Align with IETF RFC 4006 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060081 | 0084 | -- | Correction of SIP timestamps | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060081 | 0086 | -- | Correction to Calling-Party-Address AVP description | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0087 | -- | Alignment on Message Body occurrences in ACR | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060081 | 0088 | -- | Correction to Inter Operator Identifier (IOI) occurrences in ACcounting Request (ACR) | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060088 | 0089 | -- | Correction to AVP code definitions for WLAN charging | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060075 | 0091 | -- | Alignment of PS AVPs with 32.251 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060076 | 0093 | -- | Correction to PS-Furnish-Charging-Information AVP at MSCC level | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060075 | 0095 | -- | Align Service Identifier Type with IETF RFC 4006 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060080 | 0097 | -- | Correction to Diameter AVPs table assignments - Align with IETF RFC 3588 | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060087 | 0099 | -- | Correction to re-authorisation on a change in the number of participants in a PoC session | A | 7.0.0 | 7.1.0 |
| Mar 2006 | SA_31 | SP-060087 | 0101 | -- | Correction to PoC charging correlation between the different servers | A | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060241 | 0103 | -- | Correction on Proxy-Info AVP - Align with IETF RFC 3588 | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0105 | -- | Correction to online re-authorisation due to media change | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0108 | -- | Alignment of Message Body AVP with TS 32.260 | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0110 | -- | Correction to Calling-Party-Address AVP definition | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0112 | -- | Implicit registration: Add 'list of associated URIs' parameter to the IMS charging information - Align with SA2's 23.228 IMS Stage 2 | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060244 | 0114 | -- | Correct PoC specific information | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0116 | -- | Alignment with IETF RFC 3588 on use of Error-Reporting-Host AVP | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0118 | -- | Alignment on Service-Identifier AVP | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0120 | -- | Alignment with IETF RFC 4006 on use of the Direct-Debit-Failure-Handling AVP | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0122 | -- | Correction to Update-Record use in Offline Charging | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060240 | 0126 | -- | Correct information set used for PDP context related charging | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0128 | -- | Correction to Called Party Address in IMS Charging | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060241 | 0130 | -- | Correction to description of "Event-Type" AVP and "SIP-Method" AVP - Align with 32.260 | A | 7.1.0 | 7.2.0 |
| Jun 2006 | SA_32 | SP-060248 | 0106 | -- | Add a new field named PoC_user_role to PoC_Information - Align with OMA PoC 2.0 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060524 | 0132 | -- | Correction on definition of Event-Timestamp due to ambiguity in IETF definitions | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060517 | 0136 | -- | Correction on WLAN-Session-ID AVP code | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060517 | 0138 | -- | Correction on LCS AVP codes | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060517 | 0140 | -- | Correction on MMS AVP codes | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060518 | 0142 | -- | Correction to number of participants charging for PoC | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060517 | 0144 | -- | Correction on the type of the Number-of-Participants AVP | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060542 | 0145 | -- | Add P-Access-Network-Info to IMS-Information and CDRs | C | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060516 | 0147 | -- | Align description of number of participants with 32.272 (PoC-CH) | A | 7.2.0 | 7.3.0 |
| Sep 2006 | SA_33 | SP-060542 | 0148 | -- | Enhance Service-Specific-Data AVP definition | C | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060720 | 0152 | -- | Correction of AVP categories - Align with RFC 4006 | F | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060706 | 0153 | -- | Correct the usage-indicators for IMS AVPs | A | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0154 | -- | Correction on Service Information - Align with OMA Charging Enabler v1.0 | C | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060713 | 0155 | -- | Add Access Network Charging Identifier Value AVP in ACR and CCR | C | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0156 | -- | Add an identifier for the service initiating offline Diameter Accounting | B | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0157 | -- | Add an identifier for the served user in offline Diameter Accounting | B | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060706 | 0158 | -- | Correction on Multiple Service Credit Control (MSCC) - Align inside 32.299 | A | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060706 | 0159 | -- | Correct MBMS Information - Align with 23.246 | A | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0160 | -- | Correction on PS-Information AVP | F | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0161 | -- | Correction on Service Specific Data | F | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060720 | 0162 | -- | Add PoC User Access Priority into PoC-information - Align with OMA-RD-PoC-V2_0 | B | 7.3.0 | 7.4.0 |
| Dec 2006 | SA_34 | SP-060714 | 0163 | -- | Introduce support of event based charging for PCC - Align with 23.203 | B | 7.3.0 | 7.4.0 |