3GPP TSG-SA5 (Telecom Management)	S5-070312

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.642 | CR | 0020 |  rev | - |  Current version: | 7.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Revise the Object Model in UTRAN network resources IRP Network Resource Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Siemens Networks, Ericsson, Motorola, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-NIM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | - Decision to add results of CO~OP regarding HO handling into 3GPP TS32.642; - Decision to re-design the IOCs in the UTRAN NRM inorder to follow OOD principles; - Decision to add HSDPA attributes to be able to manage HSDPA features in UTRAN cells supporting HSDPA. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - IOC UtranCell and ExternalUtranCell are replaced by UtranGenericCell and ExternalUtranGenericCell and their child classes; - add necessary attributes and corresponding definitions to the IOC to be able to check HO relationships between cells across different domain managers; - add necessary attributes and corresponding definitions for HSDPA feature. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 6.2.1, 6.2.2, 6.3, 6.3.3, 6.3.5, 6.3.6, 6.4.2, 6.4.3, 6.4.4, 6.4.6, 6.4.7, 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | Results from the agreed CR S5-061505 (Siemens Networks), S5-061421 (Ericsson), S5-061471 (Motorola) and S5-70148 (CATT) |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 23.003: "Numbering, addressing and identification".

[4]	3GPP TS 25.401: "UTRAN Overall Description".

[5]	3GPP TS 25.433: "UTRAN Iub Interface NBAP Signalling".

[6]	3GPP TS 32.652: "Telecommunication management; Configuration Management (CM); GERAN network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	Void.

[8]	3GPP TS 32.672: "Telecommunication management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

[9]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[10]	Void.

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	Void.

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	3GPP TS 23.002: "Network Architecture".

[16]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[17]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) Information Service (IS)".

[18]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[19]	3GPP TR 25.802: "Remote control of electrical tilting antennas".

[20] 	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

Next Modified Section

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML.  Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The following figures show the containment/naming hierarchy and the associations of the information object classes defined in the present document. They are split in several figures only for a readability purpose.

NOTE 1:	The listed cardinality numbers represent transient as well as steady state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	The IOC AntennaFunction is required when supporting RET, For a description and clarification of RET, please refer to Annex B.

NOTE 3:	The instances of the AntennaFunction associated with a particular instance of NodeBFunction shall be contained by the same ManagedElement instance.

Figure 6.2.1.1: Transport view UTRAN NRM Containment/Naming and Association diagram

ManagedElement

(from 32.622)

<<InformationObjectClass>>

GsmCell

(from 32.652)

<<InformationObjectClass>>

ExternalGsmCell

(from 32.652)

<<InformationObjectClass>>

RncFunction

<<InformationObjectClass>>

1

0..1

1

0..1

<<names>>

UtranRelation

<<InformationObjectClass>>

GsmRelation

(from 32.652)

<<InformationObjectClass>>

0..1

0..n

0..1

+gsmRelation-GsmCell

0..n

GsmNeighbourCellRelation

GsmRelation

(from 32.652)

<<InformationObjectClass>>

0..1

0..n

0..1

+gsmRelation-ExternalGsmCell

0..n

ExternalGsmNeighbourCellRelation

UtranGenericCell

<<InformationObjectClass>>

1

0..n

1

0..n

<<names>>

0..n

1

0..n

1

<<names>>

0..1

0..n

0..1

+utranRelation-UtranGenericCell

0..n

UtranNeighbourCellRelation

0..n

1

0..n

1

<<names>>

0..n

1

0..n

1

<<names>>

SubNetwork

(from 32.622)

<<InformationObjectClass>>

ExternalRncFunction

<<InformationObjectClass>>

0..n

1

0..n

1

<<names>>

UtranRelation

<<InformationObjectClass>>

0..n

1

0..n

1

<<names>>

ExternalUtranGenericCell

<<InformationObjectClass>>

0..n

1

0..n

1

<<names>>

0..n

0..1

+controlledCellList

0..n

+controllingRnc

0..1

ExternalRncUtranCellRelation

0..n

1

0..n

1

<<names>>

0..1

0..n

0..1

+utranRelation-ExternalUtranGenericCell

0..n

ExternalUtranNeighbourCellRelation

NOTE 1:	The listed cardinality numbers represent transient as well as steady state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	The relation between GsmRelation and GsmCell is optional. It may be present if both the UtranGenericCell and the GsmCell are managed by the same management node.

NOTE 3:	The UtranRelation and GsmRelation can be name-contained under IOCs defined in other NRMs.

NOTE 4:	ExternalUtranGenericCell is contained under SubNetwork or ExternalRncFunction.

Figure 6.2.1.2: Cell view UTRAN NRM Containment/Naming and Association diagram

NOTE 1:	The listed cardinality numbers represent transient as well as steady state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

Figure 6.2.1.3: VsDataContainer Containment/Naming and Association in UTRAN NRM diagram

NOTE 1:	The ATMChannelTerminationPoint is name-contained under IOCs defined in the Transport Network NRM.

NOTE 2:	The group of ATMChannelTerminationPoints associated with an IubLink (the relation AssociatedWith1) represent the RNC end of the ATM Virtual Channel Connections (transport connection) between an RNC and a NodeB.

NOTE 3:	An ATMChannelTermiationPoint can be associated with more than one IubLink for the case of AAL2 multiplexing/switching,  i.e. to allow an ATM Channel at the RNC to be connected to multiple NodeBs.

Figure 6.2.1.4: UTRAN Transport Network NRM Containment/Naming and Association diagram

The VsDataContainer is only used for the Bulk CM IRP.

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of an IOC representing a cell could have a format like:

SubNetwork=Sweden, MeContext=MEC-Gbg-1,ManagedElement=RNC-Gbg-1, RncFunction=RF-1,UtranCell=Gbg-1.

Next Modified Section

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 6.2.2.1 shows the inheritance hierarchy for the UTRAN NRM.

Figure 6.2.2.1: UTRAN NRM Inheritance Hierarchy

Next Modified Section

### 6.3.3	UtranCell

void

#### UtranCell

#### 6.3.3.3	Attribute constraints

UtranCell

| Next Modified Section |  |  |  |
| --- | --- | --- | --- |

### 6.3.5	UtranRelation

#### 6.3.5.1	Definition

The UtranRelationIOC contains radio network related parameters for the relation to the UtranGenericCell or ExternalUtranGenericCell IOC. The UtranGenericCell and the ExternalUtranGenericCell may be an FDD mode cell, 
a lcr (low chip rate) 1.28 Mcps TDD mode cell or a hcr (high chip rate) 3.84 (7.68) Mcps TDD mode cell.

NOTE:	In handover relation terms, the cell containing the UTRAN Relation object is the source cell for the handover. The cell referred to in the UTRAN relation object is the target cell for the handover. 
This defines a one-way handover relation where the direction is from source cell to target cell.

#### 6.3.5.2	Attributes

Attributes of UtranRelation

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| utranRelationId | + | M | M | - |
| adjacentCell | + | M | M | M |

#### 6.3.5.3	Attribute constraints

void

#### 6.3.5.4	Notifications

Notifications of UtranRelation

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Next Modified Section

### 6.3.6	ExternalUtranCell

void

| Next Modified Section |  |  |  |
| --- | --- | --- | --- |

### 6.3.a	UtranGenericCell

#### 6.3.a.1	Definition

This abstract IOC represents the common properties of radio cells of different types (FDD, TDD) controlled by an RNC. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC UtranCellFDD and UtranCellTDD (1,28 Mcps TDD mode cell or a 3.84 (7.68) Mcps TDD mode cell) inherit from that abstract IOC.

The second table lists the additional attributes of UtranGenericCell for the support of State Management.

#### 6.3.a.2	Attributes

Attributes of UtranGenericCell

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| id | + | M | M | - |
| userLabel | + | M | M | M |
| cId | + | M | M | M |
| localCellId | + | M | M | M |
| retAntennaFunctionList | + | O | M | M |
| maximumTransmissionPower | + | M | M | M |
| pichPower | + | O | M | O |
| pchPower | + | O | M | O |
| fachPower | + | O | M | O |
| cellMode | + | M | M | - |
| Lac | + | M | M | M |
| Rac | + | O | M | M |
| Sac | + | M | M | M |
| uraList | + | O | M | M |
| utranGenericCell-IubLink | + | M | M | - |
| hsFlag | + | O | M | - |
| hsEnable | + | O | M | M |
| numOfHspdschs | + | O | M | M |
| numOfHsscchs | + | O | M | M |
| frameOffset | + | O | M | - |
| cellIndividualOffset | + | O | M | - |
| hcsPrio | + | O | M | - |
| maximumAllowedUlTxPower | + | O | M | - |
| snaInformation | + | O | M | - |
| qrxlevMin | + | O | M | - |
| deltaQrxlevmin | + | O | M | - |
| qhcs | + | O | M | - |
| penaltyTime | + | O | M | - |
| referenceTimeDifferenceToCell | + | O | M | - |
| readSFNIndicator | + | O | M | - |
| restrictionStateIndicator | + | O | M | - |
| dpcModeChangeSupportIndicator | + | O | M | - |
| snaInformation | + | O | M | - |

| Attribute Name | Support Qualifier | READ | WRITE |
| --- | --- | --- | --- |
| operationalState | O | M |  |
| NOTE: No state propagation shall be implied. |  |  |  |

#### 6.3.a.3	Attribute constraints

The optional attributes rac and uraList shall be included if the PLMN contains a PS CN.

The optional attribute hsFlag shall only be supported over Itf-N, if the functionality it provides is not supported by vendor specific extension mechanisms.

The optional attributes hsEnable, numOfHspdschs and numOfHsscchs shall only be supported, if the functionality they provide is not supported by vendor specific extension mechanisms while the HSDPA feature is supported in the UTRAN cell.

#### 6.3.a.4	Notifications

Notifications of UtranGenericCell

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAttributeValueChange | O |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |
| notifyStateChange | O |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

Next Modified Section

### 6.3.b	ExternalUtranGenericCell

#### 6.3.b.1	Definition

This abstract IOC represents the properties of a radio cell controlled by another IRPAgent. This IOC contains necessary attributes for inter-system and intra-system handover. It also contains a subset of the attributes of related IOCs controlled by another IRPAgent. The way to maintain consistency between the attribute values of these IOCs is outside the scope of the present document.

Inheriting from this IOC are the IOC ExternalUtranCellFDD and ExternalUtranCellTDD.

#### 6.3.b.2	Attributes

Attributes of ExternalUtranGenericCell

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| id | + | M | M | - |
| userLabel | + | M | M | M |
| cId | + | M | M | M |
| Mcc | + | M | M | M |
| Mnc | + | M | M | M |
| rncId | + | M | M | M |
| cellMode | + | M | M | - |
| Lac | + | M | M | M |
| Rac | + | O | M | M |
| controllingRnc | + | O | M | - |
| hsFlag | + | O | M | - |
| frameOffset | + | O | M | - |
| cellIndividualOffset | + | O | M | - |
| hcsPrio | + | O | M | - |
| maximumAllowedUlTxPower | + | O | M | - |
| qrxlevMin | + | O | M | - |
| deltaQrxlevmin | + | O | M | - |
| qhcs | + | O | M | - |
| penaltyTime | + | O | M | - |
| referenceTimeDifferenceToCell | + | O | M | - |
| readSFNIndicator | + | O | M | - |
| restrictionStateIndicator | + | O | M | - |
| dpcModeChangeSupportIndicator | + | O | M | - |
| snaInformation | + | O | M | - |

#### 6.3.b.3	Attribute constraints

The optional attribute rac shall be included if the external cell is connected to a PS CN.

The optional attribute hsFlag shall only be supported over Itf-N, if the functionality it provides is not supported by vendor specific extension mechanisms.

#### 6.3.b.4	Notifications

Notifications of ExternalUtranGenericCell

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Next Modified Section

### 6.3.c	UtranCellFDD

#### 6.3.c.1	Definition

This IOC represents a FDD radio cell controlled by an RNC. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC UtranCellFDD inherits from the abstract IOC UtranGenericCell.

#### 6.3.c.2	Attributes

Attributes of UtranCellFDD

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| uarfcnUl | + | O | M | M |
| uarfcnDl | + | O | M | M |
| primaryScramblingCode | + | O | M | M |
| primaryCpichPower | + | O | M | M |
| primarySchPower | + | O | M | M |
| secondarySchPower | + | O | M | M |
| bchPower | + | O | M | M |
| aichPower | + | O | M | - |
| qqualMin | + | O | M | - |
| cellCapabilityContainerFDD | + | O | M | - |
| txDiversityIndicator | + | O | M | - |
| temporaryOffset1 | + | O | M | - |
| temporaryOffset2 | + | O | M | - |
| sttdSupportIndicator | + | O | M | - |
| closedLoopMode1SupportIndicator | + | O | M | - |

#### 6.3.c.3	Attribute Constraints

The following optional attributes shall be supported over Itf-N, if they are not supported by vendor specific extension mechanisms: aichPower, pichPower, pchPower, fachPower.

Next Modified Section

### 6.3.d	UtranCellTDD

#### 6.3.d.1	Definition

This IOC is an abstract class representing the common properties of TDD high chip rate (hcr) and TDD low chip rate (lcr) radio cells controlled by an RNC. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC UtranCellTDD inherits from the abstract IOC UtranGenericCell

#### 6.3.d.2	Attributes

Attributes of UtranCellTDD

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| Uarfcn | + | O | M | M |
| cellParameterId | + | O | M | M |
| primaryCcpchPower | + | O | M | M |
| timeSlotList | + | O | M | M |
| cellCapabilityContainerTDD | + | O | M | - |
| sctdIndicator | + | O | M | - |
| dpchConstantValue | + | O | M | - |

Next Modified Section

### 6.3.e	UtranCellTDDLcr

#### 6.3.e.1	Definition

This IOC represents a TDD low chip rate (lcr) radio cell controlled by an RNC. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC UtranCellTDDLcr inherits from the abstract IOC UtranCellTDD

#### 6.3.e.2	Attributes

Attributes of UtranCellTDDLcr

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| fpachPower | + | O | M | O |
| dwPchPower | + | O | M | M |
| tstdIndicator | + | O | M | - |
| timeSlotLCR | + | O | M | - |

Next Modified Section

### 6.3.f	UtranCellTDDHcr

#### 6.3.f.1	Definition

This IOC represents a TDD high chip rate (hcr) radio cell controlled by an RNC. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC UtranCellTDDHcr inherits from the abstract IOC UtranCellTDD

#### 6.3.f.2	Attributes

Attributes of UtranCellTDDHcr

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| schPower | + | O | M | M |
| burstType | + | O | M | - |
| temporaryOffset1 | + | O | M | - |
| syncCase | + | O | M | - |
| timeSlotForSch | + | O | M | - |
| schTimeSlot | + | O | M | - |

Next Modified Section

### 6.3.g	ExternalUtranCellFDD

#### 6.3.g.1	Definition

This IOC represents a FDD radio cell controlled by another IRP agent. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC ExternalUtranCellFDD inherits from the abstract IOC ExternalUtranCell

#### 6.3.g.2	Attributes

Attributes of ExternalUtranCellFDD

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| uarfcnUl | + | O | M | M |
| uarfcnDl | + | O | M | M |
| primaryScramblingCode | + | O | M | M |
| primaryCpichPower | + | O | M | M |
| qqualMin | + | O | M | - |
| cellCapabilityContainerFDD | + | O | M | - |
| txDiversityIndicator | + | O | M | - |
| temporaryOffset1 | + | O | M | - |
| temporaryOffset2 | + | O | M | - |
| sttdSupportIndicator | + | O | M | - |
| closedLoopMode1SupportIndicator | + | O | M | - |

Next Modified Section

### 6.3.h	ExternalUtranCellTDD

#### 6.3.h.1	Definition

This IOC is an abstract class representing the common properties of TDD high chip rate (hcr) and TDD low chip rate (lcr) radio cells controlled by another IRP agent. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC ExternalUtranCellTDD inherits from the abstract IOC ExternalUtranGenericCell.

#### 6.3.h.2	Attributes

Attributes of ExternalUtranCellTDD

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| uarfcn | + | O | M | M |
| cellParameterId | + | O | M | M |
| primaryCcpchPower | * | O | O | O |
| timeSlotList | + | O | M | - |
| cellCapabilityContainerTDD | + | O | M | - |
| sctdIndicator | + | O | M | - |
| dpchConstantValue | + | O | M | - |

Next Modified Section

### 6.3.i	ExternalUtranCellTDDHcr

#### 6.3.i.1	Definition

This IOC represents a TDD high chip rate (hcr) radio cell controlled by another IRPagent. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC ExternalUtranCellTDDLcr inherits from the abstract IOC ExternalUtranCellTDD.

#### 6.3.i.2	Attributes

Attributes of ExternalUtranCellTDDHcr

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| burstType | + | O | M | - |
| temporaryOffset1 | + | O | M | - |
| syncCase | + | O | M | - |
| timeSlotForSch | + | O | M | - |
| schTimeSlot | + | O | M | - |

Next Modified Section

### 6.3.j	ExternalUtranCellTDDLcr

#### 6.3.j.1	Definition

This IOC represents a TDD low chip rate (lcr) radio cell controlled by another IRPagent. For more information about radio cells, see 3GPP TS 23.002 [15].

The IOC ExternalUtranCellTDDHcr inherits from the abstract IOC ExternalUtranCellTDD.

#### 6.3.j.2	Attributes

Attributes of ExternalUtranCellTDDLcr

| Attribute name | Visibility | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- | --- |
| tstdIndicator | + | O | M | - |
| timeSlotLCR | + | O | M | - |

Next Modified Section

### 6.4.2	AssociatedWith (M)

#### 6.4.2.1	Definition

This represents a bi-directional relation between the IubLink and UtranGenericCell. The role of the relation shall be mapped to a reference attribute of the IOC. The name of the reference attribute shall be the role name.

#### 6.4.2.2	Roles

Roles of the relation AssociatedWith

| Name | Definition |
| --- | --- |
| iubLink-UtranGenericCell | This role (when present) represents the IubLink capability to identify the set of related child classes of UtranGenericCell. IubLink.iubLink-UtranGenericCell shall carry the set of DN(s) of the child classes of UtranGenericCell. |
| utranGenericCell-IubLink | This role (when present) represents UtranGenericCellcapability to identify one related IubLink. When the role is absent, the UtranGenericCell.utranGenericCell-IubLink shall contain no information. When it is present, it shall contain one IubLink DN. |

#### 6.4.2.3	Constraints

When a particular IubLink identifies a particular child class of UtranGenericCell, that particular UtranGenericCell must have identified the particular IubLink.

Next Modified Section

### 6.4.3	ExternalUtranNeighbourCellRelation (M)

#### 6.4.3.1	Definition

This represents a unidirectional relation from UtranRelation to the ExternalUtranGenericCell. The role of the relation shall be mapped to a reference attribute, named adjacentCell, of the IOC.

#### 6.4.3.2	Roles

Roles of the relation ExternalUtranNeighbourCellRelation

| Name | Definition |
| --- | --- |
| utranRelation-externalUtranNeighbourCell | This role (when present) represents the UtranRelation capability to identify one child class of ExternalUtranGenricCell. When this role is present, the UtranRelation.adjacentCell shall contain a DN of one child class of ExternalUtranNeighbourCell. |

#### 6.4.3.3	Constraints

- This role (for a particular UtranRelation) shall be present if the UtranNeighbourCellRelation of this particular UtranRelation is absent.  This role shall be absent if the UtranNeighbourCellRelation of this particular UtranRelation is present.

Next Modified Section

### 6.4.4	UtranNeighbourCellRelation (M)

#### 6.4.4.1	Definition

This represents the unidirectional relation from the UtranRelation to UtranGenericCell. The role of the relation shall be mapped to a reference attribute, named adjacentCell, of the IOC.

#### 6.4.4.2	Roles

Roles of the relation UtranNeighbourCellRelation

| Name | Definition |
| --- | --- |
| utranRelation-utranNeighbourCell | This role (when present) represents the UtranRelation capability to identify one UtranGenericCell. When this role is present, the UtranRelation.adjacentCell shall contain one DN of a child class of UtranGenericCell. |

#### 6.4.4.3	Constraints

This role (for a particular UtranRelation) shall be present if the ExternalUtranNeighbourCellRelation of this particular UtranRelation is absent.  This role shall be absent if the ExternalUtranNeighbourCellRelation of this particular UtranRelation is present.

Next Modified Section

### 6.4.6	AssociatedWith2 (O)

#### 6.4.6.1	Definition

This represents a bi-directional relation between a UtranGenericCell and an AntennaFunction. The roles of the relation shall be mapped to a reference attribute of the IOCs. The name of the reference attribute shall be the role name

#### 6.4.6.2	Roles

Roles of the relation AssociatedWith2

| Name | Definition |
| --- | --- |
| retAntennaFunctionList | This role (when present) allows navigation from a UtranGenericCell to the AntennaFunction(s) which are supporting the UtranGenericCell. |
| retUtranCellList | This role (when present) allows navigation from an AntennaFunction to the UtranGenericCell(s) it is supporting. |

#### 6.4.6.3	Constraints

| Name | Definition |
| --- | --- |
| inv_antennaInstance | The referential attributes retAntennaFunctionList and retUtranCellList are to be populated when instances of the IOC AntennaFunction are instantiated. |

Next Modified Section

### 6.4.7	ExternalRncUtranCellRelation (O)

#### 6.4.7.1	Definition

This represents the bi-directional relation between the ExternalUtranGenericCell and ExternalRncFunction. The roles of the relation shall be mapped to a reference attribute of the IOCs. The name of the reference attribute shall be the role name.

#### 6.4.7.2	Roles

Roles of the relation ExternalRncUtranCellRelation

| Name | Definition |
| --- | --- |
| controllingRnc | This role (when present) represents the ExternalUtranGenericCell capability to identify one related ExternalRncFunction. When it is present, it shall contain one ExternalRncFunction DN. |
| controlledCellList | This role (when present) represents the ExternalRncFunction capability to identify the set of related child classes of ExternalUtranCell. When it is present, it shall contain the set of DNs of child classes of ExternalUtranGenericCell. |

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

The following table defines the attributes that are present in several Information Object Classes (IOCs) of the present document.

Attributes

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| adjacentCell | It carries the DN of the UtranGenericCell or the ExternalUtranGenericCell. |  |
| aichPower | The Power of the aich channel in a FDD cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-22..+5 dB) Steps of 1dB |
| antennaFunctionId | An attribute whose "name+value" can be used as an RDN (according to the rules in TS 32.300 [13]) when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| baseElevation | The elevation in meters above sea level at the base of the antenna structure. This value, when subtracted from height, provides the height of the antenna above the ground. | An integral value representing a number of meters in 0.1 meter increments. |
| bchPower | The power of the broadcast channel in the FDD mode cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-35..+15 dB) Steps of 0.1dB |
| bearing | The bearing in degrees that the antenna is pointing in. Note that bearing is the “true” heading (the compass heading offset by a true north variation). | A single integral value corresponding to an angle in degrees between 0 and 360. |
| burstType | Use for Timeslot ISCP measurements only. Default value is "Type1" (3.84 Mcps TDD only) | ENUMERATED |
| cellCapabilityContainerFDD | Defined in 3GPP TS25.423 Each bit indicates whether a cell supports a particular functionality. | BITSTRING(32) |
| - cellCapabilityContainerTDD | Defined in 3GPP TS25.423 Each bit indicates whether a cell supports a particular functionality. | BITSTRING(32) |
| cellIndividualOffset | Defined in 3GPP TS25.331 (25.423). Attribute relevant for HO decision Used to offset measured quantity value. | REAL(-10..10 by step of 0.5 |
| cellMode | An attribute that identifies the cell mode. | Type: Enumerated value Range: (“FDD mode”, “1.28McpsTDD mode”, “3.84 (7.68)McpsTDD mode”) |
| cellParameterId | For IOCs UtranGenericCell and ExternalUtranGenericCell,this attribute identifies unambiguously the TDD mode cell (Ref. TS 25.433 [5]): - 3.84 Mcps TDD - Code Groups, Scrambling Codes, Midambles and Toffset, or - 1.28 Mcps TDD - SYNC-DL and SYNC-UL sequences, the scrambling codes and the midamble codes. | Type: Integral numeric value Range: (0..127) |
| cId | The attribute is the identifier of a cell in one RNC (Ref. 3GPP TS 25.401 [4], 3GPP TS 25.433 [5]). | Type: Integral numeric value Range: (0..65535) |
| closedLoopModelSupportIndicator | Power control , defined in 3GPP TS25.423 The Closed Loop Mode1 Support Indicator indicates whether the particular cell is capable to support Closed loop mode1 or not | ENUMERATED |
| dpcModeChangeSupportIndicator | Power control, defined in 3GPP TS25.423 The DPC Mode Change Support Indicator IE indicates that the particular cell is capable to support DPC mode change. | ENUMERATED |
| deltaQrxlevmin | Cell (re)selection Defined in 3GPP TS25.331 If present, the actual value of Qrxlevmin = Qrxlevmin + DeltaQrxlevmin | INTEGER(-4..-2 by step of 2 |
| dpchConstantValue | DPCH Constant Value is the power margin in dB used by a UE to set the proper uplink power. | INTEGER(-10…10) |
| dwPchPower | DwPCH Power is the power that shall be used for transmitting the DwPCH in a 1.28 Mcps TDD cell (Ref. 3GPP TS 25.433 [5] ). | Type: Numeric value Range: (-15..+40 dBm) Steps of 0.1dB |
| externalRncFunctionId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| externalUtranCellId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| fachPower | The power of the fach transport channel in the cell (Ref 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-35.. +15 dB) Steps of 0.1dB |
| fpachPower | The Power of the fpach channel in TDD cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-15.. +40 dBm) Steps of 0.1dB |
| frameOffset | Neighbouring cells monitoring (synchro), defined in 3GPP TS25.423 Frame Offset is the required offset between the dedicated channel downlink transmission frames (CFN, Connection Frame Number) and the broadcast channel frame offset (Cell Frame Number). The Frame Offset is used in the translation between Connection Frame Number (CFN) on Iub/Iur and least significant 8 bits of SFN (System Frame Number) on Uu. The Frame Offset is UE and cell specific. | INTEGER(0..255) |
| hcsPrio | Cell (re)selection for HCS Defined in 3GPP TS25.331 (TS 25.304) This specifies the HCS priority level (0-7) for serving cell and neighbouring cells. HCS priority level 0 means lowest priority and HCS priority level 7 means highest priority. | INTEGER (0..7) |
| height | The height of an antenna above sea level. Planning permission (for a cell site) is normally granted on the antenna height. This parameter also determines the site coverage and feeds into the planning tool. | An integral value representing a number of metres in 0.1 meter increments. |
| horizBeamwidth | The 3 dB power beamwidth of the antenna pattern in the horizontal plane. A value of 360 indicates an omni-directional antenna. | A single integral value corresponding to an angle in degrees between 0 and 360. |
| id | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| iubLinkId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| lac | IOCs UtranGenericCell and ExternalUtranGenericCell: Location Area Code, LAC (Ref. 3GPP TS 23.003 [3]). | Type: Integral numeric value Range: (1..65533, 65535) |
| latitude | The latitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. | Valid values described in 3GPP TS23.032 [20]. |
| localCellId | Local Cell id is used to uniquely identify the set of resources defined in a Node B to support a cell (as defined by a Cid Ref. 3GPP TS 25.401 [4], 3GPP TS 25.433 [5]). It must be unique in Node B at a minimum, but may be unique in UTRAN. It can be used to tie the cell in the RNC to a specific set of resources in the Node B. | Type: Integral numeric value Range: (0..268435455) |
| longitude | The longitude of the antenna location based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. | Valid values described in 3GPP TS23.032.[20] |
| maximumAllowedUlTxPower | Cell (re)selection, defined in 3GPP TS25.331 This information element indicates the maximum allowed uplink transmit power. | Type: Numeric value Unit: dBm |
| maxAzimuthValue | The maximum amount of change of azimuth the RET system can support. This is the change in degrees clockwise from bearing. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees, see Note. |
| maxTiltValue | The maximum amount of tilt the RET system can support. This helps in preventing the user from entering any unrealistic value for retTiltValue and hence prevents the motors on the RET unit from getting jammed / burnt out. | A single integral value corresponding to an angle in degrees between 0 and 360. In 0.1 degree increments (see TR.25.802 [19] clause 7.7.5.11 RET. |
| maximumTransmissionPower | The maximum transmission power of a cell. It is the maximum power for all downlink channels added together, that is allowed to be used simultaneously in a cell. (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (0..50 dBm) Steps of 0.1 dB |
| mcc | Mobile Country Code, MCC (part of the PLMN Id, Ref. 3GPP TS 23.003 [3]). |  |
| mechanicalOffset | This is a value representing a non-adjustable tilt value, which is imparted to the antenna due to the physical installation. The actual tilt at any point in time is the summation of mechanicalOffset and retTiltValue . | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees, see Note. |
| minAzimuthValue | The minimum amount of change of azimuth the RET system can support. This is the change in degrees counter-clockwise from bearing. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees, see Note. |
| minTiltValue | The minimum amount of tilt the RET system can support. This helps in preventing the user from entering any unrealistic value for retTiltValue and hence prevents the motors on the RET unit from getting jammed / burnt out. | A single integral value corresponding to an angle in degrees between 0 and 360 with a resolution of 0.1 degrees, see Note (see also 3GPP TR.25.802 [19] clause 7.7.5.11 RET Note 1). |
| mnc | Mobile Network Code, MNC (part of the PLMN Id, Ref. 3GPP TS 23.003 [3]). |  |
| pchPower | The power of pch transport channel in the cell (Ref 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-35.. +15 dB) Steps of 0.1dB |
| penaltyTime | Cell (re)selection for HCS Defined in 3GPP TS25.331 (TS 25.304) This specifies the time duration for which the TEMPORARY_OFFSET is applied for a neighbouring cell. | - INTEGER(0, 10, 20, 30 ,40, 50, 60); Default is 0. |
| pichPower | The Power of the pich channel in the cell (Ref 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-10..+5 dB) Steps of 1dB |
| primaryCcpchPower | IOCs UtranGenericCell and ExternalUtranGenericCell: The power of the primary CCPCH channel in the TDD cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-15..+40 dBm) Steps of 0.1dB |
| nodeBFunctionId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| patternLabel | The pattern name is a textual, alpha-numeric string to allow identification of the antenna pattern along with the antenna vendor information such as model number, etc. |  |
| primaryCpichPower | IOCs UtranCell and ExternalUtranCell: The power of the primary CPICH channel in the FDD mode cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-10..50 dBm) Steps of 0.1 dB |
| primarySchPower | The power of the primary synchronisation channel in the FDD mode cell, DL Power (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-35..+15 dB) Steps of 0.1dB |
| primaryScramblingCode | IOCs UtranCell and ExternalUtranCell: The primary DL scrambling code used by the FDD mode cell (Ref. 3GPP TS 25.433 [5]). | Type: Integral numeric value Range: (0..511) |
| Qhcs | Cell (re)selection for HCS, defined in 3GPP TS25.331 (TS 25.304) This specifies the quality threshold levels for applying prioritised hierarchical cell re-selection | INTEGER(0..99) |
| qqualMin | Cell (re)selection, defined in 3GPP TS25.331 (TS25.304) This specifies the minimum required quality level in the cell in dB. It is only applicable for FDD cells. | INTEGER(-24..0) |
| qrxlevMin | Cell (re)selection, defined in 3GPP TS25.331 (TS25.304) This specifies the minimum required RX level in the cell in dBm. | INTEGER(-115..-25 by step of 2) |
| rac | Routing Area Code, RAC (Ref. 3GPP TS 23.003 [3]). | Type: Integral numeric value Range: (0..255) |
| readSFNIndicator | Neighbouring cells monitoring (synchro), defined in 3GPP TS25.331 TRUE indicates that read of SFN is requested for the target cell | BOOLEAN |
| referenceTimeDifferenceToCell | Neighbouring cells monitoring (synchro), defined in 3GPP TS25.331 In the System Information message, the reference time difference to cell indicates the timing difference between the primary CCPCH of the current cell and the primary CCPCH of a neighbouring cell. In the Measurement Control message, the reference time difference to cell indicates the timing difference between UE uplink transmission timing and the primary CCPCH of a neighbouring cell. | INTEGER |
| restrictionStateIndicator | Cell Access Control, defined in 3GPP TS25.423 The Restriction state indicator is the identifier indicates whether the cell is "Cell Reserved for Operator Use" or not. It is provided by DRNS and reported to SRNC. | ENUMERATED |
| retAntennaFunctionList | This is a referential attribute to list the DNs of AntennaFunction(s) that support the UtranGenericCell. | A list of DNs as defined in TS 32.300 [13]. |
| retGroupName | The group name is a textual, alpha-numeric string to define a logical grouping of antennas which may be in different cells. This attribute permits the definition of a logical grouping of the antennas. This may be defined either at installation time, or by management activity to provisioning the group name via the Itf-N. | Type: string bounded to 80 characters. |
| retTiltValue | Gives you the tilt value of the antenna that has been made using electrical means (i.e. using RET). This attribute gives the operator an indication of the current setting of the antenna and is at the centre of the RET feature. | A single integral value corresponding to an angle in degrees between 0 and 360in 0.1 degree increments (see Note). |
| retUtranCellList | This is a list of UtranGenericCell DNs to record the relationship between the AntennaFunction instance and the UtranGenericCell(s) which are supported by the antenna. | A list of DNs as defined in TS 32.300 [13]. |
| rncFunctionId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| rncId | IOC ExternalUtranGenericCell: Unique RNC ID for the associated RNC (Ref. 3GPP TS 23.003 [3]). IOC RncFunction: Unique RNC ID (Ref. 3GPP TS 23.003 [3]). |  |
| sac | Service Area Code, SAC (Ref. 3GPP TS 23.003 [3]). | Type: Integral numeric value Range: (0.. 65535) |
| schPower | The power of the synchronisation channel in 3.84 Mcps TDD cell (Ref. 3GPP TS 25.433 [5]). | Type: Numeric Value Range: (-35..+15 dB) Steps of 0.1dB |
| schTimeSlot | The SCH Time Slot IE represents the first time slot (k) of a pair of time slots inside a Radio Frame that is assigned to the Physical Channel SCH. The SCH Time Slot IE is only applicable if the value of Sync Case IE is Case 2 since in this case the SCH is allocated in TS#k and TS#k+8. | INTEGER(0…6) |
| sctdIndicator | This attribute indicates whether SCTD is used. | BOOLEAN TRUE indicates that SCTD is used |
| secondarySchPower | The power of the secondary synchronisation channel in the FDD mode cell, DL Power (Ref. 3GPP TS 25.433 [5]). | Type: Numeric value Range: (-35..+15 dB) Steps of 0.1dB |
| snaInformation | Shared Networks Access Control, defined in 3GPP TS25.423. This information element contains a list of Shared Network Areas, identified by the Shared Network Area Code (SNAC) which a certain cell belongs to. |  |
| sttdSupportIndicator | Power control , defined in 3GPP TS25.423 The STTD Support Indicator indicates whether the STTD can be applied to DL DPCH and F-DPCH in the cell or not. | ENUMERATED |
| syncCase | The SCH and PCCPCH in a TDD cell are mapped on one or two downlink slots per frame. There are two cases of Sync Case as follows: - SCH and PCCPCH allocated in a single TS#k - SCH allocated in two TS: TS#k and TS#k+8. PCCPCH allocated in TS#k There is no Sync Case indication needed for 1.28Mcps TDD. | INTEGER |
| temporaryOffset1 | Cell (re)selection for HCS Defined in 3GPP TS25.331 (TS 25.304) This specifies the offset applied to the H and R criteria for a neighbouring cell for the duration of PENALTY_TIME. It is used for TDD and GSM cells and for FDD cells in case the quality measure for cell selection and re-selection is set to CPICH RSCP | INTEGER(3, 6, 9, 12, 15, 18, 21, inf) |
| temporaryOffset2 | Cell (re)selection for HCS Defined in 3GPP TS25.331 (TS 25.304) This specifies the offset applied to the H and R criteria for a neighbouring cell for the duration of PENALTY_TIME. It is used for FDD cells in case the quality measure for cell selection and re-selection is set to CPICH Ec/No. | INTEGER(2, 3, 4, 6, 8, 12, inf) |
| timeSlotForSch | The Time Slot represents the time interval assigned to a Physical Channel referred to the start of a Radio Frame. | INTEGER(0…14) |
| timeSlotLCR | The Time Slot LCR is the number of the traffic time slot within a 5 ms subframe of LCR TDD (Dl only). | INTEGER(0…6) |
| timeSlotList | This attribute defines the time slot configuration information in the TDD cell. It is a list which contains 7 (for 1.28 Mcps TDD cell) or 15 (for 3.84 Mcps TDD cell) items. Within each item there are three parts: timeSlotId, timeSlotDirection, timeSlotStatus (Ref. 3GPP TS 25.433 [5]). | timeSlotId: when applied to 1.28 Mcps TDD cell: Type: Integral numeric value Range: (0..6); when applied to 3.84 (7.68) Mcps TDD cell: Type: Integral numeric value Range: (0..14); timeSlotDirection: Type: Enumerated value Range: (Ul, Dl); timeSlotStatus: Type: Enumerated value Range: (Active, Not active) |
| txDiversityIndicator | Defined in 3GPP TS25.331 (25.423) The txDiversityIndicator indicates whether following conditions are satisfied: - Primary CPICH is broadcast from two antennas - STTD is applied to Primary CCPCH - TSTD is applied to Primary SCH and Secondary SCH | BOOLEAN |
| tstdIndicator | This attribute indicates whether TSTD is used (1.28 Mcps TDD only). | BOOLEAN default value is TRUE |
| uarfcn | IOCs UtranGenericCell and ExternalUtranGenericCell: The UTRA absolute Radio Frequency Channel number for TDD mode cell, UARFCN (Ref. 3GPP TS 25.433 [5]). | Type : Integral numeric Value Range: (0..16383) |
| uarfcnDl | IOCs UtranGenericCell and ExternalUtranGenericCell: The DL UTRA absolute Radio Frequency Channel number for FDD mode cell, UARFCN (Ref. 3GPP TS 25.433 [5]). | Type: Integral numeric value Range: (0..16383) |
| uarfcnUl | IOCs UtranGenericCell and ExternalUtranGenericCell: The UL UTRA absolute Radio Frequency Channel number for FDD mode cell, UARFCN (Ref. 3GPP TS 25.433 [5]). | Type: Integral numeric value Range: (0..16383) |
| uraList | A list of UTRAN Registration Area, URA (Ref. 3GPP TS 25.331 (subclause 10.3.10)[9]), that an UtranCell can belong to. | Type: A list of Integral numeric values Range: (0..65535) for each integral numeric value. |
| userLabel | A user-friendly (and user assigned) name of the associated object. Inherited from ManagedFunction. |  |
| utranRelationId | An attribute whose "name+value" can be used as an RDN when naming an instance of the object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| vertBeamwidth | The 3 dB power beamwidth of the antenna pattern in the vertical plane. | A single integral value corresponding to an angle in degrees between 0 and 180. |
| NOTE: If an antenna vendor can only support a granularity of tilt value in 5 degree increments, it means that the value of tilt over the Itf-N would be 0, 50, 100, 150 etc, corresponding to an integral number of 0.1 degree values. |  |  |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Jun 2001 | SA_12 | SP-010283 | -- | -- | Approved at TSG SA #12 and placed under Change Control | -- | 2.0.0 | 4.0.0 |
| Jun 2002 | SA_16 | SP-020303 | 0001 | -- | Corrections of reference in figure 6.2 and of attribute descriptions in UtranRelation in 32.642 (UTRAN network resources IRP: NRM) | F | 4.0.0 | 4.1.0 |
| Jun 2002 | SA_16 | SP-020304 | 0002 | -- | Correction of supported IRP in system context | F | 4.0.0 | 4.1.0 |
| Sep 2002 | SA_17 | SP-020490 | 0003 | -- | UML corrections | F | 4.1.0 | 4.2.0 |
| Sep 2002 | SA_17 | SP-020492 | 0004 | -- | Add the new IRP IS methodology defined in 32.102 | F | 4.2.0 | 5.0.0 |
| Sep 2002 | SA_17 | SP-020492 | 0005 | -- | Add State Management | B | 4.2.0 | 5.0.0 |
| Dec 2002 | SA_18 | SP-020748 | 0006 | -- | Inclusion of valid values and ranges for UTRAN Cell parameters | F | 5.0.0 | 5.1.0 |
| Jan 2003 | -- | -- | -- | -- | Accepted all revision marks | -- | 5.1.0 | 5.1.1 |
| Jun 2003 | SA_20 | SP-030282 | 0008 | -- | Include notification tables | A | 5.1.1 | 5.2.0 |
| Jun 2003 | SA_20 | SP-030282 | 0010 | -- | Correction of UML diagram vsDataContainer Containment/Naming and Association in UTRAN NRM | A | 5.1.1 | 5.2.0 |
| Jun 2003 | SA_20 | SP-030283 | 0012 | -- | Deletion of UTRAN attribute relationType | A | 5.1.1 | 5.2.0 |
| Dec 2003 | SA_22 | SP-030715 | 0014 | -- | Correction in attribute description for “maximumTransmissionPower” to remove dual interpretation - Align with RAN3's 25.433 | A | 5.2.0 | 5.3.0 |
| Dec 2003 | SA_22 | SP-030646 | 0016 | -- | Correction of the number of possible URAs from 1 to 8 | A | 5.2.0 | 5.3.0 |
| Dec 2003 | SA_22 | SP-030641 | 0017 | -- | Add missing notification notifyPotentialFaultyAlarmlist | F | 5.2.0 | 5.3.0 |
| Dec 2003 | SA_22 | SP-030643 | 0018 | -- | Remove redundant VsDataContainer Containment UML - Now covered by 32.622 | F | 5.2.0 | 5.3.0 |
| Mar 2004 | SA_23 | SP-040129 | 0019 | -- | Addition of new attributes for support of both FDD and TDD modes | B | 5.3.0 | 6.0.0 |
| Jun 2004 | SA_24 | SP-040254 | 0021 | -- | Correction of the supported UMTS frequencies | A | 6.0.0 | 6.1.0 |
| Sep 2004 | SA_25 | SP-040584 | 0022 | -- | Add support for the state change notification in UTRAN network resources IRP NRM | B | 6.1.0 | 6.2.0 |
| Sep 2004 | SA_25 | SP-040595 | 0023 | -- | Include ATM in CM UTRAN network resources IRP NRM | B | 6.1.0 | 6.2.0 |
| Sep 2004 | SA_25 | SP-040585 | 0026 | -- | Align with the IRP IS template (32.151) and IRP IS UML repertoire (32.152) | F | 6.1.0 | 6.2.0 |
| Sep 2004 | SA_25 | SP-040587 | 0027 | -- | Add support for Remote control of Electrical Tilting (RET) antenna CR not implementable (UML conflict) New CR028 SA#26 approved | B | 6.1.0 | 6.2.0 |
| Dec 2004 | SA_26 | SP-040810 | 0028 | -- | Add AntennaFunction class and attributes to support RET (Remote control of Electrical Tilting) | B | 6.2.0 | 6.3.0 |
| Dec 2004 | SA_26 | SP-040810 | 0029 | -- | Add support for the state change notification | B | 6.2.0 | 6.3.0 |
| Mar 2005 | SA_27 | SP-050048 | 0032 | -- | Align with SA2’s 23.221, for allowing only CS CN in a PLMN | A | 6.3.0 | 6.4.0 |
| Mar 2005 | SA_27 | SP-050048 | 0033 | -- | Add missing definition of IOC ExternalRncFunction | F | 6.3.0 | 6.4.0 |
| Mar 2005 | SA_27 | SP-050048 | 0034 | -- | Amendments to UTRAN NRM for RET | F | 6.3.0 | 6.4.0 |
| Sep 2005 | SA_29 | SP-050461 | 0035 | -- | Add attributes for RET antennas systems - Align with TR 32.804 & RAN specs | F | 6.4.0 | 6.5.0 |
| Mar 2006 | SA_31 | SP-060102 | 0036 | -- | Correct relationships for external Information Object Classes (IOCs) | F | 6.5.0 | 6.6.0 |
| Jun 2006 | SA_32 | SP-060415 | 0037 | 1 | Correct the definition of longitude | F | 6.6.0 | 6.7.0 |
| Jun 2006 | SA_32 | SP-060259 | 0038 | -- | Add configuration parameters for radio channel power | B | 6.6.0 | 7.0.0 |