3GPP TSG-SA5 (Telecom Management)	S5-081544

Meeting SA5#61, 13-17 Oct 2008, Dalian, China

- Source:	Qualcomm Europe, Alcatel-Lucent

- Title:	Requirements for Physical Cell ID assignment

- Document for:	Discussion, Decision and Approval

Agenda Item:	6.07

# 1	Decision/action requested

SA5 is asked to discuss and approve the following text for inclusion in TS 32.500.

# 2	References

[1]    R3-082359	LS on Configuring a list of PCI, RAN3.

[2]	R3-082357	Agreement on framework for PCI selection, Qualcomm Europe, NSN, Orange, Vodafone, Telecom Italia, T-Mobile, Alcatel-Lucent, Huawei.

[3] 	TS 32.762		Telecommunications management;	Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM), Integration Reference Point (IRP): Information Service (IS) (Release 8).

[4]	TS  32.500	Self-organizing Networks (SON): Conepts and Requirements.

# 3	Rationale

In [1], RAN3 has requested SA5 to provide signaling to allow the OAM to sepcify a range of physical cell identities (PCI) at a network element. According to [1]:

- OAM shall provide a list of valid PCIs, or a specific PCI value, that the eNB can choose from.

- The eNB shall base the selection of its PCI either on a centralized or distributed PCI assignment algorithm:

  - [Centralized PCI assignment] The OAM signals a specific PCI value. The eNB shall select this value as its PCI.

  - [Distributed PCI assignment] The OAM signals a list of PCI values. The eNB shall select a PCI value from this list, which may be refined by the algorithms described in [2].

In order to support the above feature, configuration of the list of PCIs needs to be supported over the management interface. The network element can then choose an appropriate PCI from within this list.

### 3.1 New atrributes for Physical Cell Identity List

To support the requirement for specifying a valid list of physical cell identities, we will propose a new attribute in the appropriate cell object. The type of this attribute shall be a list of PCIs.

### 3.2 Existing physicalCellIdentity attribute

There already exists a physicalCellIdentity attribute in [3] that specifies the physical cell identity used by the eNB. This attribute should remain unchanged, and define the exact PCI being used by the network element.

Any change in this attribute (e.g. caused by a local PCI selection algorithm) should be reported over the management interface.

# 4	Detailed proposal

It is proposed to add the following text to [4].

6.1      Requirement

6.1.7 	Continuous Optimisation due to Dynamic Changes in the Network

REQ-xxx-xxm1: A list of valid physical cell identifiers shall be configured by IRPManager, in order to allow the eNB to choose an appropriate PCI for a cell from within this list. The list of PCIs shall be cell-specific.

REQ-xxx-xxm2: A valid PCI shall be configured by IRPManager to support centralized PCI assignment. The PCI shall be cell-specific.

REQ-xxx-xxn IRPAgent shall inform IRPManager about the PCIs that are selected for each cell.

REQ-xxx-xxo: IRPAgent should inform IRPManager about the reason for changing the PCI for a cell.

REQ-xxx-xxp: IRPAgent shall inform IRPManager if a valid physical cell ID cannot be found in the list of PCIs configured by IRPManager.