3GPP TSG-SA5 (Telecom Management)	S5-081722

Meeting SA5#61, 13 - 17 October 2008, Dalian, China

- Source:	Qualcomm, Vodafone, T-Mobile, Telefonica

- Title:	Performance Measurements for Inter-RAT Handover Monitoring

- Document for:	Discussion & approval

Agenda Item:	6.05.3

# 1	Decision/action requested

Agree on PM for inter-RAT handover monitoring

# 2	References

[1]	TS 36.331	Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification

[2]	TS 32.762	E-UTRAN NRM

[3]	TS 36.413	S1 Application protocol (S1AP)

[4]	TS 32.425	E-UTRAN Performance Measurements

# 3	Rationale

In order to gain insight into the performance of IRAT handovers, the operator needs to know the success rate of IRAT handovers along with the number of triggered IRAT handovers. Additionally, in order to be able to break down the HO performance per target RAT and per neighbour relation, the handover success and attempt rate need to be available per target RAT and per neighbour relation basis. Finally, the performance measurements indicating the causes for IRAT handover triggering are required in order to properly monitor handovers initiated by different causes.

# 4				Description

The following performance measurements based on UE measurement reports should be introduced to facilitate monitoring of IRAT HO performance:

## Inter-RAT handover related measurements

### Attempted outgoing inter-RAT handovers in active mode per cause

- This measurement provides the number of attempted outgoing inter-RAT handovers per cause.

- CC.

- Transmission of the MobilityFromEUTRACommand message or the HandoverFromEUTRAPreparationRequest message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to UTRAN or to GERAN or to CDMA2000 with a specific cause (see TS 36.331 [1]).

- All IRAT handovers to the cells measured are counted.

- A single integer value.

- IRATHO.AttOut.UTRAN.Quality

- IRATHO.AttOut.UTRAN.Other

- IRATHO.AttOut.GERAN.Quality
IRATHO.AttOut.GERAN.Other

- IRATHO.AttOut.CDMA2000.Quality

- IRATHO.AttOut.CDMA2000.Other

- GSMRelation

- UTRANRelation

- CDMA200Relation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Attempted outgoing inter-RAT handovers in active mode per serving-target cell relation per cause

- This measurement provides the number of attempted outgoing inter-RAT handovers per serving-target cell relation per cause.

- CC.

- Transmission of the MobilityFromEUTRACommand message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to a specific UTRAN or GERAN cell with a specific cause or transmission of the HandoverFromEUTRAPreparationRequest message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to a specific CDMA2000 cell with a specific cause (see TS 36.331 [1]).

- All IRAT handovers to the cells measured are counted.

- A single integer value.

- IRATHO.AttOut.Quality.<TargetCellID>

- IRATHO.AttOut.Other.<TargetCellID>

- GSMNeighborCellRelation

- UTRANNeighborCellRelation

- CDMA2000NeighborCellRelation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Successful outgoing inter-RAT handovers in active mode per cause

- This measurement provides the number of successful outgoing inter-RAT handovers per cause.

- CC.

- Receipt of a S1AP message SAE BEARER RELEASE sent from the MME to the serving eNB, indicating a successful IRAT handover initiated due to a specific cause (see TS 36.413 [3]).

- All IRAT handovers to the cells measured are counted.

- A single integer value.

- IRATHO.SuccOut.UTRAN.Quality
IRATHO.SuccOut.UTRAN.Other

- IRATHO.SuccOut.GERAN.Quality

- IRATHO.SuccOut.GERAN.Other

- IRATHO.SuccOut.CDMA2000.Quality

- IRATHO.SuccOut.CDMA2000.Other

- GSMRelation

- UTRANRelation

- CDMA200Relation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Successful outgoing inter-RAT handovers in active mode per serving-target cell relation per cause

- This measurement provides the number of successful outgoing inter-RAT handovers per serving-target cell relation per cause.

- CC.

- Receipt of a S1AP message SAE BEARER RELEASE sent from the MME to the serving eNB, indicating a successful IRAT handover to a specific target cell initiated due to a specific cause (see TS 36.413 [3])..

- All IRAT handovers to the cells measured are counted.

- A single integer value.

- IRATHO.SuccOut.Quality.<TargetCellID>

- IRATHO.SuccOut.<TargetCellID>

- GSMNeighborCellRelation

- UTRANNeighborCellRelation

- CDMA2000NeighborCellRelation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

## Inter-RAT handover related measurements with measurement gaps

### Attempted outgoing inter-RAT handovers in active mode with measurement gaps per cause

- This measurement provides the number of attempted outgoing inter-RAT handovers when measurement gaps are used per cause.

- CC.

- Transmission of the MobilityFromEUTRACommand message or the HandoverFromEUTRAPreparationRequest message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to UTRAN or to GERAN or to CDMA2000 with a specific cause (see TS 36.331 [1]).

- Only IRAT handovers to the cells measured using measurement gaps configured by EUTRAN are counted.

- A single integer value.

- IRATHO.AttOutGaps.UTRAN.Quality

- IRATHO.AttOutGaps.UTRAN.Other

- IRATHO.AttOutGaps.GERAN.Quality
IRATHO.AttOutGaps.GERAN.Other

- IRATHO.AttOutGaps.CDMA2000.Quality

- IRATHO.AttOutGaps.CDMA2000.Other

- GSMRelation

- UTRANRelation

- CDMA200Relation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Attempted outgoing inter-RAT handovers in active mode with measurement gaps per serving-target cell relation per cause

- This measurement provides the number of attempted outgoing inter-RAT handovers when measurement gaps are used per serving-target cell relation per cause.

- CC.

- Transmission of the MobilityFromEUTRACommand message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to a specific UTRAN or GERAN cell with a specific cause or transmission of the HandoverFromEUTRAPreparationRequest message from the serving eNB to the UE indicating the attempt of an outgoing handover from EUTRAN to a specific CDMA2000 cell with a specific cause (see TS 36.331 [1]).

- Only IRAT handovers to the cells measured using measurement gaps configured by EUTRAN are counted.

- A single integer value.

- IRATHO.AttOutGaps.Quality.<TargetCellID>

- IRATHO.AttOutGaps.Other.<TargetCellID>

- GSMNeighborCellRelation

- UTRANNeighborCellRelation

- CDMA2000NeighborCellRelation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Successful outgoing inter-RAT handovers in active mode with measurement gaps per cause

- This measurement provides the number of successful outgoing inter-RAT handovers when measurement gaps are used per cause.

- CC.

- Receipt of a S1AP message SAE BEARER RELEASE sent from the MME to the serving eNB, indicating a successful IRAT handover initiated due to a specific cause (see TS 36.413 [3]).

- Only IRAT handovers to the cells measured using measurement gaps configured by EUTRAN are counted.

- A single integer value.

- IRATHO.SuccOutGaps.UTRAN.Quality
IRATHO.SuccOutGaps.UTRAN.Other

- IRATHO.SuccOutGaps.GERAN.Quality

- IRATHO.SuccOutGaps.GERAN.Other

- IRATHO.SuccOutGaps.CDMA2000.Quality

- IRATHO.SuccOutGaps.CDMA2000.Other

- GSMRelation

- UTRANRelation

- CDMA200Relation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

### Successful outgoing inter-RAT handovers in active mode with measurement gaps per serving-target cell relation per cause

- This measurement provides the number of successful outgoing inter-RAT handovers when measurement gaps are used per serving-target cell relation per cause.

- CC.

- Receipt of a S1AP message SAE BEARER RELEASE sent from the MME to the serving eNB, indicating a successful IRAT handover to a specific target cell initiated due to a specific cause (see TS 36.413 [3])..

- Only IRAT handovers to the cells measured using measurement gaps configured by EUTRAN are counted.

- A single integer value.

- IRATHO.SuccOutGaps.Quality.<TargetCellID>

- IRATHO.SuccOutGaps.Other.<TargetCellID>

- GSMNeighborCellRelation

- UTRANNeighborCellRelation

- CDMA2000NeighborCellRelation 
Editor’s note: the measurement object class definition is pending the completion of TS 32.762 [2].

- N/A

- LTE

## Summary of the proposed PM

Inter-RAT handover related measurements (without and with measurement gaps):

| Measurement Type | Description |
| --- | --- |
| - IRATHO.AttOut.UTRAN.Quality - IRATHO.AttOut.UTRAN.Other - IRATHO.AttOut.GERAN.Quality IRATHO.AttOut.GERAN.Other - IRATHO.AttOut.CDMA2000.Quality - IRATHO.AttOut.CDMA2000.Other | - Total number of attempted outgoing inter-RAT handovers with specific cause. |
| IRATHO.AttOut.Quality.<targetCellId> IRATHO.AttOut.Other.<targetCellId> | - Total number of attempted inter-RAT handovers to specific target cell with specific cause. |
| - IRATHO.SuccOut.UTRAN.Quality - IRATHO.SuccOut.UTRAN.Other - IRATHO.SuccOut.GERAN.Quality - IRATHO.SuccOut.GERAN.Other - IRATHO.SuccOut.CDMA2000.Quality - IRATHO.SuccOut.CDMA2000.Other | - Total number of successful outgoing inter-RAT handovers with specific cause. |
| IRATHO.SuccOut.Quality.<targetCellId> IRATHO.SuccOut.Other.<targetCellId> | - Total number of successful inter-RAT handovers to specific target cell with specific cause. |

Inter-RAT handover related measurements with measurement gaps:

| Measurement Type | Description |
| --- | --- |
| - IRATHO.AttOutGaps.UTRAN.Quality - IRATHO.AttOutGaps.UTRAN.Other - IRATHO.AttOutGaps.GERAN.Quality IRATHO.AttOutGaps.GERAN.Other - IRATHO.AttOutGaps.CDMA2000.Quality - IRATHO.AttOutGaps.CDMA2000.Other | - Number of attempted outgoing inter-RAT handovers when measurement gaps are used with specific cause. |
| IRATHO.AttOutGaps.Quality.<targetCellId> IRATHO.AttOutGaps.Other.<targetCellId> | - Number of attempted inter-RAT handovers when measurement gaps are used to specific target cell with specific cause. |
| - IRATHO.SuccOutGaps.UTRAN.Quality IRATHO.SuccOutGaps.UTRAN.Other - IRATHO.SuccOutGaps.GERAN.Quality - IRATHO.SuccOutGaps.GERAN.Other - IRATHO.SuccOutGaps.CDMA2000.Quality - IRATHO.SuccOutGaps.CDMA2000.Other | - Number of successful outgoing inter-RAT handovers when measurement gaps are used with specific cause. |
| IRATHO.SuccOutGaps.Quality.<targetCellId> IRATHO.SuccOutGaps.Other.<targetCellId> | - Number of successful inter-RAT handovers when measurement gaps are used to specific target cell with specific cause. |

# Detailed proposal

It is proposed to agree on the proposed performance measurements and to introduce them into TS 32.425 [4] (exact location TBD by the rapporteur).