3GPP TSG-SA5 (Telecom Management)	S5-0902770r2

Meeting SA5#66, 29 June – 3 July 2009, Dublin, Ireland

- Source:	Alcatel-Lucent

- Title:	Add HeNB Procedures for Alarm Management

- Document for:	Discussion and Approval

Agenda Item:	6.4.3

# 1	Decision/action requested

The group is asked to discuss the proposed Procedures for Alarm Management, and introduce the agreed part into the draft TS 32.593.

# 2	References

# [1] TS 32.593 v0.1.0

# 3	Rationale

Procedure flows for Alarm Management needs to be described in TS 32.593

# 4	Detailed proposal

1st Modified Section

# 5    Procedure Flows

## 5.5	Alarm Reporting

### 5.5.1	Alarm Reporting Mechanism Configuration

This procedure allows TR-069 Manager to use SetParameterValues method to determine which alarm attributes, such as  perceived severity, alarm type, etc., of the HeNB alarms shall be used to select alarms to be reported to TR-069 Manager in the HeMS.

Alarm Reporting Mechanism Configuration

HeNB

TR-069 Manager

1. SetParameterValues (ReportingMechanism)

2. SetParameterValuesResponse

TR-069 connection establishment

Figure 5.5.1-1: Alarm Reporting Mechanism Configuration

A TR-069 connection is established between the HeNB and the HeMS.

- TR-069 Manager initiates the configuration of the alarm reporting mechanism by using SetParameterValues. The parameter ReportingMechanism is defined in TS 32.592 [10] Section 6.2.5.

- HeNB responds to TR-069 Manager by using SetParameterValues Response message to indicate success or failure of the procedure. In case of failure the response message returns an error code.

For the detailed procedure of SetParameterValues refer to TR-069 Amendment 2 A.3.2.1

### 5.5.2	Alarm Reporting Procedure for expediated and Queued alarms (by RPC method)

When an alarm occurs, depending on the ReportingMechanism (defined in TS 32.592 [10] Section 6.2.5)  of the alarm, the HeNB reports the alarm to the TR-069 Manager. This procedure is applicable to only the alarms that classified as expedited and queued.

Alarm Reporting

HeNB

TR-069 Manager

1. Inform

2. InformResponse

TR-069 connection establishment

Figure 5.2.2-1: Alarm Reporting Procedure Flow

A TR-069 connection is established between the HeNB and the HeMS.

- HeNB reports the alarm directly to TR-069 Manager by using Inform method.

- When TR-069 Manager receives the alarm, it responds to HeNB with InformResponse message.

For the detailed procedure of the inform method refer to TR-069 Amendment 2 A.3.3.1.

End of 1st modifications