3GPP TSG-SA5 (Telecom Management)	S5-094340

Meeting SA5#68, 9 Nov-13 Nov 2009, Shanghai, China

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.582 | CR | CRNum |  rev | - |  Current version: | 8.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Delete LIPA definition from 32.582 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ATT, Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM8 |  |  |  |  |  |  | Date:  |  | 011/13/2009 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | To remove an item outside of Rel 8 scope. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Deletion of LIPA definition. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Confusion about Rel 8 scope. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.1.1, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2    References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.401: “Telecommunication management; Performance Management (PM); Concept and requirements”

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 25.331: “Radio Resource Control (RRC); Protocol specification”

[7] 	TR-069 Amendment 2, HNB WAN Management Protocol v1.1, Broadband Forum, viewable at http://www.broadband-forum.org/technical/download/TR-069Amendment2.pdf

[8]	3GPP TR 25.820: “3G Home NodeB Study Item Technical Report”

[9]	3GPP TS 25.413: “UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling”

[10]	3GPP TS 25.401” “ Radio Access Network UTRAN Overall Description”

[11]	3GPP TS 25.433: “UTRAN Iub interface Node B Application Part (NBAP) signalling”

[12]	3GPP TR 32.821: "Study of Self-Organizing Network(SON) related OAM for Home NodeB".

[13] 	3GPP TS 25.467: "UTRAN architecture for 3G Home NodeB, stage 2".

[14]	TR-106, Broadband Forum

[15]	TR-196, Femto Access Point Device Data Model”, Broadband Forum, 2009

[16] 	3GPP TS 32.432 “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Telecommunication management; Performance measurement: File format definition

[17]	3GPP TS 32.111-2: “Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)”

### 6.1.1	HNB Access Network

HNB Access Network Related Parameter Types

| Parameter Type | Description | Reference |
| --- | --- | --- |
| HNB-GW Gateway Identity | Identity of the HNB-GW the HNB connects to. The type of identity may be either “name” or IP address. | 25.467 |
| Security Gateway Identity | Identity of the Security GW the HNB connects to. The type of identity may be either “name” or IP address. | 25.467 |
| Access Mode | Indicates the type of access mode the HNB is to operate; it can be either “Open Access,” “Closed Access,” or “Hybrid Access.” | 22.011 |
| ACL list | Defines the ACL, which consists of one or more IMSI of the UE that the access to the HNB is allowed. It is applicable to either “Closed Access” and “Hybrid Access.” | 22.011 |
| CSG Identity | Defines the CSG ID to which the HNB is a member of. It consists of one or more than one CSG ID. | 22.011 |
| CSG specific info | Defines the CSG specific split information including Primary Scrambling Code (PSC) and UARFCN dedicated to the CSG. This information is intended in the BCCH SIB information for the UEs under the HNB. | 25.331, 10.3.2.8,10.3.2.9, 10.2.48.8.14a |
| HNB ID | Home NodeB Identifier broadcasted when it operates as CSG cell and is to be displayed on the UE. | 22.011 |
| Home Zone Name | Character string that defines the “home zone name” that is to be displayed to the UE that camps on to the HNB. | n/a |
| Local IP Access | Defines the Local IP Access behaviour of the HNB for the associated UE(s) from which the traffic originates from. This includes the destination IP address and subnet mask, protocol type(s), forwarding behaviour (Local IP Access or not). | n/a |
| URA list | List of one or more URA that the HNB belongs to. | 25.331, sec.10.3.2.6 |
| RNC ID | RNC-ID of the HNB. | 25.413, sec.9.2.1.39, 9.2.1.39a |
| RANAP Timers | RANAP message related timers, typically named as “T-xxx” | 25.413, sec.9.5 |
| Cell ID | Cell Identity, bit string (28). | 25.331, sec.10.3.2.2 |
| HSDPA related parameter | HSDPA specific parameters including: 1) HSDPA is used in the cell or not, 2) number of codes at the defined spreading factor within the code tree, 3) number of HS-SCCHs used in the HNB. | 32.642, sec.6.3.9 |
| Measurement related parameter | Measurement related parameters including: 1) measurement quantity, 2) minimum Rx signal level, 3) offset to the minimum Rx level, 4) cell re-selection hysteresis, 5) timers for re-selection, 6) threshold for re-selection. Note that separate sets of parameters exist for intra-frequency, inter-frequency, and inter-RAT frequency measurements. | 25.331, sec.10.3.2.3, 10.3.2.4 |
| HCS related parameter | Measurement related parameters including: 1) timers, threshold, hysteresis for HCS to determine when the UE enters or exists the high-mobility state, 2) quality threshold level for HCS, 3) flag to indicate HCS is used or not in the HNB, 4) HCS priority value for the HNB if the HCS is used. | 25.331, sec.10.3.2.3, 10.3.7.12, 10.3.7.47, 10.3.7.54a |
| RACH related parameter for UE | RACH related parameters for UE when accessing the PRACH, including: 1) Maximum Tx power level a UE may use, 2) initial Tx power, 3) ramp-up step, 4) maximum preamble retransmission value, 5) maximum RACH preamble cycles, 6) back-off period lowerbound/upperbound. | 25.331, sec.10.3.2.3, 10.3.6.11, 10.3.6.54, 10.3.6.67 25.304, sec.5.2.3 |
| DRX related parameter | DRX related paramters including: DRX cycle length coefficient. | 25.331, sec.10.3.3.6, 10.3.3.49 |

End of 1st Modified Section

2nd Modified Section

Annex A (normative): 3GPP Home Node B Type 1 Data Model

## For 3GPP Home Node B Type 1 Data Model the following specification shall be used:

## TR-196 [15].

The following exception shall apply for this release:

- All parameters and objects under TR-196 .FAPService.{i}.AccessMgmt.LocalIPAccess is not applicable for 3GPP and shall not be required to be used nor implemented.

End of 2nd Modified Section