3GPP TS 32.393 V8.1.0 (2010-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Delta Synchronization Integration Reference Point (IRP);

Common Object Request Broker Architecture (CORBA)

Solution Set (SS)

(Release 8)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

management, CORBA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Architectural features	6

5	Mapping	7

5.1	General mappings	7

5.2	Operation and notification mapping	7

5.3	Operation parameter mapping	8

5.4	Notification parameter mapping	10

Annex A (normative):	IDL specifications	14

A.1	IDL specification (file name "DeltaSynchronizationConstDefs.idl")	14

A.2	IDL specification (file name "DeltaSynchronizationSystem.idl")	19

A.3	IDL specification (file name "DeltaSynchronizationNotifications.idl")	22

Annex B (informative):	Change history	24

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

32.391:	Delta Synchronization Integration Reference Point (IRP); Requirements.

32.392:	Delta Synchronization Integration Reference Point (IRP); Information Service (IS).

32.393:	Delta Synchronization Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set.

32.395:	Delta Synchronization Integration Reference Point (IRP); eXtensible Markup Language (XML) file format definition.

The Itf-N interface is built up by a number of IRPs and a related Name Convention, which realise the functional capabilities over this interface. The basic structure of the IRPs is defined in 3GPP TS 32.101 [1] and 3GPP TS 32.102 [2].

IRPManagers (typically Network Management Systems) and IRPAgents (typically EMs or NEs) synchronize their data concerning alarms or configuration data. In certain scenarios this synchronization is lost or not done. This IRP provides functionality to significantly reduces the amount of data which needs to be transferred in order to re-establish synchronization.

# 1	Scope

The purpose of Delta Synchronization IRP is to define an interface through which an IRPManager can request only those data which changed (i.e. changed, were created or deleted) from a synchronization point onwards.

The present document is the "CORBA Solution Set" of Delta Synchronization IRP for the IRP whose semantics is specified in Delta Synchronization IRP: Information Service (3GPP TS 32.392 [5]).

# 2	References

The following documents contain provisions that, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.391: "Configuration Management (CM); Delta Synchronization Integration Reference Point (IRP): Requirements".

[5]	3GPP TS 32.392: "Configuration Management (CM); Delta Synchronization Integration Reference Point (IRP): Information Service (IS)".

[6]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.312: "Telecommunication management; Generic Integration Reference Point (IRP) management; Information Service (IS)".

[8]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic CM Integration Reference Point (IRP) management; Information Service (IS)".

[9]	3GPP TS 32.303: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[10]	OMG TC Document telecom/98-11-01: "OMG Notification Service". http://www.omg.org/technology/documents/

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2] and 3GPP TS 32.391 [4] apply.

IRP: See 3GPP TS 32.101 [1].

IRPAgent: See 3GPP TS 32.102 [2].

IRPManager: See 3GPP TS 32.102 [2].

Changed: See 3GPP TS 32.391 [4].

Changed instance: See 3GPP TS 32.391 [4].

Delta Synchronisation: See 3GPP TS 32.391 [4].

Delta Synchronisation Point: See 3GPP TS 32.391 [4].

Full Synchronisation: See 3GPP TS 32.391 [4].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

EM	Element Manager

IRP	Integration Reference Point

IS	Information Service (see 3GPP TS 32.101 [1])

Itf-N	Interface N

NE	Network Element

SS	Solution Set

# 4	Architectural features

The overall architectural feature of Delta Synchronization IRP is specified in 3GPP TS 32.382 [5].

# 5	Mapping

## 5.1	General mappings

Not applicable.

## 5.2	Operation and notification mapping

The Delta Synchronization IS defines semantics of operations visible across the Itf-N. Table 5.2-1 indicates mapping of these operations and notifications to their equivalents defined in this SS.

Table 5.2-1: Mapping from IS Operation to SS equivalents

| IS Operation / Notification (3GPP TS 32.392) | SS Method | Qualifier |
| --- | --- | --- |
| manageDeltaSynchronization | manageDeltaSynchronization | M |
| getAvailableDeltaSynchPoints | getAvailableDeltaSynchPoints | O |
| triggerDeltaSynchOfCMData | triggerDeltaSynchOfCMData | O |
| triggerDeltaSynchOfAlarms | triggerDeltaSynchOfAlarms | O |
| notifyStatusOfDeltaSynchronization | notifyStatusOfDeltaSynchronization | M |
| notifyNewDeltaSynchPoint | notifyNewDeltaSynchPoint | O |

## 5.3	Operation parameter mapping

The Delta Synchronization IS defines semantics of parameters carried in operations across the Itf-N. The following tables indicate the mapping of these parameters, as per operation, to their equivalents defined in this SS.

Table 5.3-1: Mapping from IS manageDeltaSynchronization parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| managerReference | DeltaSynchronizationConstDefs::ManagerReference | M |
| manageDeltaSynchForAlarmData | DeltaSynchronizationConstDefs::ManageDeltaSynchForXDataConditional | CM |
| manageDeltaSynchForCMData | DeltaSynchronizationConstDefs::ManageDeltaSynchForXDataConditional | CM |
| status | Exceptions: DeltaSynchronizationConstDefs::ManageDeltaSynchronization, GenericIRPManagementSystem::ParameterNotSupported, GenericIRPManagementSystem::InvalidParameter, GenericIRPManagementSystem::ValueNotSupported, GenericIRPManagementSystem::OperationNotSupported | M |

Table 5.3-2: Mapping from IS getAvailableDeltaSynchPoints parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| managerReference | DeltaSynchronizationConstDefs::ManagerReferenceOpt | O |
| synchPointsForCMDataRequested | DeltaSynchronizationConstDefs::SynchPointsRequestedConditional | CM |
| synchPointsForAlarmDataRequested | DeltaSynchronizationConstDefs::SynchPointsRequestedConditional | CM |
| synchPointListForAlarms | DeltaSynchronizationConstDefs::SynchPointListConditional | CM |
| synchPointListForCMData | DeltaSynchronizationConstDefs::SynchPointListConditional | CM |
| status | Exceptions: DeltaSynchronizationConstDefs::DeltaSynchNotSupportedForCMData, DeltaSynchronizationConstDefs::DeltaSynchNotSupportedForAlarmData, DeltaSynchronizationConstDefs::DeltaSynchNotActive, DeltaSynchronizationConstDefs::DeltaSynchForCMDataDeactivated, DeltaSynchronizationConstDefs::DeltaSynchForAlarmDataDeactivated, GenericIRPManagementSystem::ParameterNotSupported, GenericIRPManagementSystem::InvalidParameter, GenericIRPManagementSystem::ValueNotSupported, GenericIRPManagementSystem::OperationNotSupported | M |

Table 5.3-3: Void

Table 5.3-4: Mapping from IS triggerDeltaSynchOfCMData parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| managerReference | DeltaSynchronizationConstDefs::ManagerReferenceOpt | O |
| dataRequested | DeltaSynchronizationConstDefs::CMDataRequestedOpt | O |
| baseMOInstance | KernelCmConstDefs::DNOpt | O |
| scope | KernelCmConstDefs::ScopeTypeOpt | O |
| synchPoint | DeltaSynchronizationConstDefs::SynchPoint | M |
| deltaLists | DeltaSynchronizationConstDefs::DeltaListsConditional | CM |
| newSynchPoint | DeltaSynchronizationConstDefs::SynchPointConditional | CM |
| status | Exceptions: DeltaSynchronizationConstDefs::TriggerDeltaSynchOfCMData, DeltaSynchronizationConstDefs::SynchronizationPointTooLongAgo, DeltaSynchronizationConstDefs::TooManyChangesFullSynchronizationRecommended DeltaSynchronizationConstDefs::SynchPointUnknown, DeltaSynchronizationConstDefs::DeltaSynchNotSupportedForCMData, DeltaSynchronizationConstDefs::DeltaSynchForCMDataDeactivated, DeltaSynchronizationConstDefs::DeltaSynchNotActive, GenericIRPManagementSystem::ParameterNotSupported, GenericIRPManagementSystem::InvalidParameter, GenericIRPManagementSystem::ValueNotSupported, GenericIRPManagementSystem::OperationNotSupported | M |

Table 5.3-5: Mapping from IS triggerDeltaSynchOfAlarms parameters to SS equivalents

| IS Operation parameter | SS Method parameter | Qualifier |
| --- | --- | --- |
| managerReference | DeltaSynchronizationConstDefs::ManagerReference | M |
| dataRequested | DeltaSynchronizationConstDefs::AlarmDataRequested | M |
| baseMOInstance | KernelCmConstDefs::DN | O |
| scope | KernelCmConstDefs::ScopeTypeOpt | O |
| synchPoint | DeltaSynchronizationConstDefs::SynchPoint | M |
| deltaLists | DeltaSynchronizationConstDefs::DeltaListsConditional | CM |
| newSynchPoint | DeltaSynchronizationConstDefs::SynchPointConditional | CM |
| status | Exceptions: DeltaSynchronizationConstDefs::TriggerDeltaSynchOfAlarms, DeltaSynchronizationConstDefs::SynchronizationPointTooLongAgo, DeltaSynchronizationConstDefs::TooManyChangesFullSynchronizationRecommended DeltaSynchronizationConstDefs::SynchPointUnknown, DeltaSynchronizationConstDefs::DeltaSynchNotSupportedForAlarms, DeltaSynchronizationConstDefs::DeltaSynchForAlarmsNotActive, DeltaSynchronizationConstDefs::DeltaSynchNotActive, GenericIRPManagementSystem::ParameterNotSupported, GenericIRPManagementSystem::InvalidParameter, GenericIRPManagementSystem::ValueNotSupported, GenericIRPManagementSystem::OperationNotSupported | M |

## 5.4	Notification parameter mapping

The delta synchronization Information Service defines semantics of parameters carried in notifications. The following table indicates the mapping of these parameters to their OMG CORBA Structured Event (defined in OMG Notification Service [n2!!]) equivalents. The composition of OMG Structured Event, as defined in the OMG Notification Service [n2!!], is:

Header

Fixed Header

domain_name

type_name

event_name

Variable Header

Body

filterable_body_fields

remaining_body

The following tables list all OMG Structured Event attributes in the second column. The first column identifies the delta synchronization Information Service defined notification parameters.

Table 5.4-1: Mapping for notifyStatusOfDeltaSynchronization

| IS Parameters | OMG CORBA Structured Event attribute | Qualifier | Comment |
| --- | --- | --- | --- |
| There is no corresponding IS attribute. | domain_name | M | It carries the IRP document version number string. See clause 3.1. It indicates the syntax and semantics of the Structured Event as defined by the present document. |
| notificationType | type_name | M | This is the NotifyDeltaSynchOfInstancesDeactivated of module DeltaSynchronizationNotifications. |
| There is no corresponding IS attribute. | event_name | M | It carries no information. |
| There is no corresponding IS attribute. | Variable Header |  |  |
| objectClass, objectInstance | One NV pair of filterable_body_fields | M | NV stands for name-value pair. Order arrangement of NV pairs is not significant. The name of NV-pair is always encoded in string. Name of this NV pair is the MANAGED_OBJECT_INSTANCE of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| notificationId | One NV pair of remaining_body | M | Name of NV pair is the NOTIFICATION_ID of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a long. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| eventTime | One NV pair of filterable_body_fields | M | Name of NV pair is the EVENT_TIME of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is IRPTime. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| systemDN | One NV pair of filterable_body_fields | M | Name of NV pair is the SYSTEM_DN of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| managerReference | One NV pair of remaining_body | M | Name of NV pair is the MANAGER_REFERENCE of interface notifyDeltaSynchOfInstancesDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is ManagerReference of module DeltaSynchronizationConstDefs. |
| deltaSynchStatusForCMData | One NV pair of remaining_body | M | Name of NV pair is the DELTA_SYNCH_STATUS_FOR_CMDATA of interface notifyDeltaSynchOfInstancesDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is DeltaSynchStatus of module DeltaSynchronizationConstDefs. |
| deltaSynchStatusForAlarmData | One NV pair of remaining_body | M | Name of NV pair is the DELTA_SYNCH_STATUS_FOR_ALARM_DATA of interface notifyDeltaSynchOfInstancesDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is DeltaSynchStatus of module DeltaSynchronizationConstDefs. |

Table 5.4-2: Mapping for notifyNewDeltaSynchPoint

| IS Parameters | OMG CORBA Structured Event attribute | Qualifier | Comment |
| --- | --- | --- | --- |
| There is no corresponding IS attribute. | domain_name | M | It carries the IRP document version number string. See subclause 3.1. It indicates the syntax and semantics of the Structured Event as defined by the present document. |
| notificationType | type_name | M | This is the NotifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. |
| There is no corresponding IS attribute. | event_name | M | It carries no information. |
| There is no corresponding IS attribute. | Variable Header |  |  |
| objectClass, objectInstance | One NV pair of filterable_body_fields | M | NV stands for name-value pair. Order arrangement of NV pairs is not significant. The name of NV-pair is always encoded in string. Name of this NV pair is the MANAGED_OBJECT_INSTANCE of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| notificationId | One NV pair of remaining_body | M | Name of NV pair is the NOTIFICATION_ID of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a long. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| eventTime | One NV pair of filterable_body_fields | M | Name of NV pair is the EVENT_TIME of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is IRPTime. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| systemDN | One NV pair of filterable_body_fields | M | Name of NV pair is the SYSTEM_DN of interface AttributeNameValue of module NotificationIRPConstDefs. Value of NV pair is a string. See corresponding table in Notification IRP: CORBA SS (3GPP TS 32.303 [1]). |
| newSynchPoint | One NV pair of remaining_body | M | Name of NV pair is the NEW_DELTA_SYNCH_POINT of interface notifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is SynchPoint of module DeltaSynchronizationConstDefs. |
| requestedSynchPoint | One NV pair of remaining_body | M | Name of NV pair is the REQUESTED_SYNCH_POINT of interface notifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is SynchPoint of module DeltaSynchronizationConstDefs. |
| deltaSynchPointType | One NV pair of remaining_body | M | Name of NV pair is the DELTA_SYNCH_POINT_TYPE of interface notifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is DeltaSynchPointType of module DeltaSynchronizationConstDefs. |
| triggeredByAgentOrManager | One NV pair of remaining_body | M | Name of NV pair is the TRIGGERED_BY_AGENT_OR_MANAGER of interface notifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is TriggeredByAgentOrManager of module DeltaSynchronizationConstDefs. |
| agentOrManagerReference | One NV pair of remaining_body | M | Name of NV pair is the AGENT_OR_MANAGER_REFERENCE of interface notifyDeltaSynchOfAlarmsDeactivated of module DeltaSynchronizationNotifications. Value of NV pair is AgentOrManagerReference of module DeltaSynchronizationConstDefs. |

Annex A (normative):
IDL specifications

# A.1	IDL specification (file name "DeltaSynchronizationConstDefs.idl")

// File: DeltaSynchronizationConstDefs.idl

#ifndef _DELTA_SYNCHRONIZATION_CONST_DEFS_IDL_

#define _DELTA_SYNCHRONIZATION_CONST_DEFS_IDL_

#include <TimeBase.idl>

#include <DeltaSynchronizationConstDefs.idl>

#include <GenericIRPManagementConstDefs.idl>

#include <KernelCmConstDefs.idl>

#include <FileTransferIRPConstDefs.idl>

#include <AlarmIRPConstDefs.idl>

//FileTransferIRPConstDefs::FileLocation value;

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

/* ## Module: DeltaSynchronizationConstDefs */

module DeltaSynchronizationConstDefs

{

/*****************************************************************************/

/* definition of types used in several operations for Delta Synchronization: */

/*****************************************************************************/

/* types used in several operations: */

typedef string ManagerReference;

typedef string AgentOrManagerReference;

/*

ManagerReferenceConditional is a type carrying an optional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case the value is present. Otherwise the value is be absent.

*/

union ManagerReferenceConditional switch (boolean)

{

case TRUE: ManagerReference value;

};

/*

ManagerReferenceOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value is present. Otherwise the value is absent.

*/

union ManagerReferenceOpt switch (boolean)

{

case TRUE: ManagerReference value;

};

enum ManageDeltaSynchForXData {ACTIVATE, DEACTIVATE};

/*

ManageDeltaSynchForXDataConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case the value is present. Otherwise the value is absent.

*/

union ManageDeltaSynchForXDataConditional switch (boolean)

{

case TRUE: ManageDeltaSynchForXData value;

};

typedef TimeBase::UtcT SynchPoint;

/*

SynchPointConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case the value is present. Otherwise the value is absent.

*/

union SynchPointConditional switch (boolean)

{

case TRUE: SynchPoint value;

};

/*

SynchPointOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value is present. Otherwise the value is absent.

*/

union SynchPointOpt switch (boolean)

{

case TRUE: SynchPoint value;

};

/*

ScopeTypeOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value is present. Otherwise the value is absent.

*/

union ScopeTypeOpt switch (boolean)

{

case TRUE: KernelCmConstDefs::ScopePara value;

};

/*

BaseMOInstanceOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value is present. Otherwise the value is absent.

*/

union BaseMOInstanceOpt switch (boolean)

{

case TRUE: GenericIRPManagementConstDefs::DN value;

};

enum Status {SUCCESS, FAILURE};

/********************************************************************/

/* types used in operation manageDeltaSynchronization               */

/********************************************************************/

enum ActivatedStatus {ACTIVATED, DEACTIVATED };

typedef ActivatedStatus ManageDeltaSynchMode;

/********************************************************************/

/* types used in operation getAvailableDeltaSynchPoints             */

/********************************************************************/

typedef boolean SynchPointsRequested;

/*

SynchPointsRequestedConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. Otherwise the value may be absent.

*/

union SynchPointsRequestedConditional switch (boolean)

{

case TRUE: SynchPointsRequested value;

};

typedef sequence <SynchPoint> SynchPointList;

/*

SynchPointListConditional is a type carrying an optional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case the value is present. Otherwise the value is be absent.

*/

union SynchPointListConditional switch (boolean)

{

case TRUE: SynchPointList value;

};

/********************************************************************/

/* types used in operation triggerDeltaSynchOfCMData and            */

/*            in operation triggerDeltaSynchOfAlarmData             */

/********************************************************************/

/*

AttributeListConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the operation’s dnsOnly=FALSE. In this case the value is present.

Otherwise the value is absent.

*/

union AttributeListConditional switch (boolean)

{

case TRUE: GenericIRPManagementConstDefs::MOAttributeSet value;

};

struct ListedInstance

{

GenericIRPManagementConstDefs::DN moInstance;   /* DN is a string; */

AttributeListConditional attributeList;

};

typedef sequence <ListedInstance> ListOfInstances;

struct DeltaListsWithRealLists

{

TimeBase::UtcT startTime;

TimeBase::UtcT endTime;

ListOfInstances listOfCreatedInstances;

ListOfInstances listOfChangedInstances;

ListOfInstances listOfDeletedInstances;

};

struct AlarmDeltaListsWithRealLists

{

TimeBase::UtcT startTime;

TimeBase::UtcT endTime;

AlarmIRPConstDefs::AlarmInformationSeq listOfNewAlarms;

AlarmIRPConstDefs::AlarmInformationSeq listOfChangedAlarms;

AlarmIRPConstDefs::AlarmInformationIdSeq listOfDeletedAlarms;

};

typedef sequence <FileTransferIRPConstDefs::FileLocation> FileLocationList;

struct DeltaListsWithFileReferences

{

TimeBase::UtcT startTime;

TimeBase::UtcT endTime;

FileLocationList fileList;

/* if several files are used, then they shall be processed by the IRPmanager in sequence, i.e. first file first, second file as second, … */

};

enum DeltaListContentChoice {REAL_LISTS, FILE_REFERENCES};

//The CmDeltaList may contain a list of ListOfInstances or a list of filenames

union CmDeltaLists switch (DeltaListContentChoice)

{

case REAL_LISTS: DeltaListsWithRealLists deltaListRealLists;

case FILE_REFERENCES: DeltaListsWithFileReferences deltaListFileReferences;

};

//The AlarmDeltaLists may contain a list of ListOfInstances or a list of filenames

union AlarmDeltaLists switch (DeltaListContentChoice)

{

case REAL_LISTS: AlarmDeltaListsWithRealLists deltaListRealLists;

case FILE_REFERENCES: DeltaListsWithFileReferences deltaListFileReferences;

};

/*

CmDeltaListsConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case

the value is present. Otherwise the value is be absent.

*/

union CmDeltaListsConditional switch (boolean)

{

case TRUE: CmDeltaLists value;

};

/*

AlarmDeltaListsConditional is a type carrying a conditional parameter.

The boolean shall be TRUE, if the condition described in TS 32.392 is fulfilled. In this case

the value is present. Otherwise the value is absent.

*/

union AlarmDeltaListsConditional switch (boolean)

{

case TRUE: AlarmDeltaLists value;

};

/********************************************************************/

/* types used in operation triggerDeltaSynchOfAlarmData             */

/********************************************************************/

enum AlarmDataRequested { ALARM_IDS_ONLY, COMPLETE_ALARM_INFORMATION };

/*

AlarmDataRequestedOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value

is present. Otherwise the value is absent.

*/

union AlarmDataRequestedOpt switch (boolean)

{

case TRUE: AlarmDataRequested value;

};

/********************************************************************/

/* types used in operation triggerDeltaSynchOfCMData                */

/********************************************************************/

enum CMDataRequested { DNS_ONLY, COMPLETE_DATA_SET };

/*

CMDataRequestedOpt is a type carrying an optional parameter.

The boolean shall be TRUE, if the operation request uses this parameter. In this case the value

is present. Otherwise the value is absent.

*/

union CMDataRequestedOpt switch (boolean)

{

case TRUE: CMDataRequested value;

};

/*****************************************************************************/

/* definition of types in notifications for Delta Synchronization            */

/*****************************************************************************/

enum DeltaSynchPointType { DELTA_SYNCH_POINT_FOR_ALARM, DELTA_SYNCH_POINT_FOR_CM_DATA };

typedef ActivatedStatus DeltaSynchStatus;

enum TriggeredBy { IRP_AGENT, IRP_MANAGER };

/*********************************************************************************/

/* Definition of parameters specified in notifications for Delta Synchronization */

/*********************************************************************************/

interface AttributeNameValue

{

const string MANAGER_REFERENCE = "MANAGER_REFERENCE";

const string AGENT_OR_MANAGER_REFERENCE = "AGENT_OR_MANAGER_REFERENCE";

const string DELTA_SYNCH_STATUS_FOR_CMDATA = "DELTA_SYNCH_STATUS_FOR_CMDATA";

const string DELTA_SYNCH_STATUS_FOR_ALARM_DATA = "DELTA_SYNCH_STATUS_FOR_ALARM_DATA";

const string NEW_DELTA_SYNCH_POINT = "NEW_DELTA_SYNCH_POINT";

const string DELTA_SYNCH_POINT_TYPE = "DELTA_SYNCH_POINT_TYPE";

const string REQUESTED_SYNCH_POINT = "REQUESTED_SYNCH_POINT";

const string TRIGGERED_BY_AGENT_OR_MANAGER = "TRIGGERED_BY_AGENT_OR_MANAGER";

};

};

#endif // _DELTA_SYNCHRONIZATION_CONST_DEFS_IDL_

# A.2	IDL specification (file name "DeltaSynchronizationSystem.idl")

//File: DeltaSynchronizationSystem.idl

#ifndef _DELTA_SYNCHRONIZATION_SYSTEM_IDL_

#define _DELTA_SYNCHRONIZATION_SYSTEM_IDL_

#include <KernelCmConstDefs.idl>

#include <DeltaSynchronizationConstDefs.idl>

#include <GenericIRPManagementSystem.idl>

#include <AlarmIRPConstDefs.idl>

#include <AlarmIRPSystem.idl>

#include <NotificationLogIRPSystem.idl>

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

/* ## Module: DeltaSynchronizationSystem */

module DeltaSynchronizationSystem

{

/*

If the system fails to complete an operation, then it can provide a reason

to qualify the exception. The semantics carried in this reason are outside

the scope of the present document.

*/

exception ManageDeltaSynchronization { string reason; };

exception GetAvailableDeltaSynchPoints { string reason; };

exception TriggerDeltaSynchOfCMData { string reason; };

exception TriggerDeltaSynchOfAlarms { string reason; };

exception SynchronizationPointTooLongAgo { string reason; };

exception TooManyChangesFullSynchronizationRecommended { string reason; };

exception DeltaSynchNotSupportedForCMData { string reason; };

exception DeltaSynchNotSupportedForAlarmData { string reason; };

exception DeltaSynchNotActive { string reason; };

exception DeltaSynchForCMDataDeactivated { string reason; };

exception DeltaSynchForAlarmDataDeactivated { string reason; };

exception SynchPointTooLongAgo{ string reason; };

exception SynchPointUnknown { string reason; };

exception DeltaSynchNotSupportedForAlarms { string reason; };

exception DeltaSynchForAlarmsNotActive { string reason; };

interface DeltaSynchGenericParts

{

DeltaSynchronizationConstDefs::Status manageDeltaSynchronization

/* for the purpose of this operation see 3GPP TS 32.392 */

(

in DeltaSynchronizationConstDefs::ManagerReference managerReference,

in DeltaSynchronizationConstDefs::ManageDeltaSynchForXDataConditional

manageDeltaSynchForAlarmData,

in DeltaSynchronizationConstDefs::ManageDeltaSynchForXDataConditional

manageDeltaSynchForCMData

)

raises

(

ManageDeltaSynchronization,

GenericIRPManagementSystem::ParameterNotSupported,

GenericIRPManagementSystem::InvalidParameter,

GenericIRPManagementSystem::ValueNotSupported,

GenericIRPManagementSystem::OperationNotSupported

);

DeltaSynchronizationConstDefs::Status getAvailableDeltaSynchPoints

/* for the purpose of this operation see 3GPP TS 32.392 */

(

in DeltaSynchronizationConstDefs::ManagerReferenceOpt managerReference,

in DeltaSynchronizationConstDefs::SynchPointsRequestedConditional

synchPointsForCMDataRequested,

in DeltaSynchronizationConstDefs::SynchPointsRequestedConditional

synchPointsForAlarmDataRequested,

out DeltaSynchronizationConstDefs::SynchPointListConditional synchPointListForAlarms,

out DeltaSynchronizationConstDefs::SynchPointListConditional synchPointListForCMData

)

raises

(

GetAvailableDeltaSynchPoints,

DeltaSynchNotSupportedForCMData,

DeltaSynchNotSupportedForAlarmData,

DeltaSynchNotActive,

DeltaSynchForCMDataDeactivated,

DeltaSynchForAlarmDataDeactivated,

GenericIRPManagementSystem::ParameterNotSupported,

GenericIRPManagementSystem::InvalidParameter,

GenericIRPManagementSystem::ValueNotSupported,

GenericIRPManagementSystem::OperationNotSupported

);

};

interface DeltaSynchOfCMData

{

DeltaSynchronizationConstDefs::Status triggerDeltaSynchOfCMData

/* for the purpose of this operation see 3GPP TS 32.392 */

(

in DeltaSynchronizationConstDefs::ManagerReferenceOpt managerReference,

in DeltaSynchronizationConstDefs::CMDataRequestedOpt cmDataRequested,

in DeltaSynchronizationConstDefs::BaseMOInstanceOpt baseMOInstance,

in DeltaSynchronizationConstDefs::ScopeTypeOpt scope,

in DeltaSynchronizationConstDefs::SynchPoint synchPoint,

out DeltaSynchronizationConstDefs::CmDeltaListsConditional deltaLists,

out DeltaSynchronizationConstDefs::SynchPointConditional newSynchPoint

)

raises

(

TriggerDeltaSynchOfCMData,

SynchronizationPointTooLongAgo,

TooManyChangesFullSynchronizationRecommended,

SynchPointUnknown,

DeltaSynchNotSupportedForCMData,

DeltaSynchForCMDataDeactivated,

DeltaSynchNotActive,

GenericIRPManagementSystem::ParameterNotSupported,

GenericIRPManagementSystem::InvalidParameter,

GenericIRPManagementSystem::ValueNotSupported,

GenericIRPManagementSystem::OperationNotSupported

);

};

interface DeltaSynchOfAlarmData

{

DeltaSynchronizationConstDefs::Status triggerDeltaSynchOfAlarms

/* for the purpose of this operation see 3GPP TS 32.392 */

(

in DeltaSynchronizationConstDefs::ManagerReferenceOpt managerReference,

in DeltaSynchronizationConstDefs::AlarmDataRequestedOpt alarmDataRequested,

in DeltaSynchronizationConstDefs::BaseMOInstanceOpt baseMOInstance,

in DeltaSynchronizationConstDefs::ScopeTypeOpt scope,

in DeltaSynchronizationConstDefs::SynchPoint synchPoint,

out DeltaSynchronizationConstDefs::AlarmDeltaListsConditional deltaLists,

out DeltaSynchronizationConstDefs::SynchPointConditional newSynchPoint

)

raises

(

TriggerDeltaSynchOfAlarms,

SynchronizationPointTooLongAgo,

TooManyChangesFullSynchronizationRecommended,

SynchPointUnknown,

DeltaSynchNotSupportedForAlarms,

DeltaSynchForAlarmsNotActive,

DeltaSynchNotActive,

GenericIRPManagementSystem::ParameterNotSupported,

GenericIRPManagementSystem::InvalidParameter,

GenericIRPManagementSystem::ValueNotSupported,

GenericIRPManagementSystem::OperationNotSupported

);

};

interface DeltaSynchIRPSystem : DeltaSynchGenericParts, DeltaSynchOfCMData,

DeltaSynchOfAlarmData, GenericIRPManagementSystem::GenericIRPManagement{};

};

#endif // _DELTA_SYNCHRONIZATION_SYSTEM_IDL_

# A.3	IDL specification (file name "DeltaSynchronizationNotifications.idl")

//File: DeltaSynchronizationNotifications.idl

#ifndef _DELTA_SYNCHRONIZATION_NOTIFICATIONS_IDL_

#define _DELTA_SYNCHRONIZATION_NOTIFICATIONS_IDL_

#include <DeltaSynchronizationConstDefs.idl>

#include <NotificationIRPNotifications.idl>

// This statement must appear after all include statements

#pragma prefix "3gppsa5.org"

/* ## Module: DeltaSynchronizationNotifications

This contains the specification of notifications of Delta Synchronization.

==========================================================================

*/

module DeltaSynchronizationNotifications

{

/* Constant definitions for the NotifyDeltaSynchOfInstancesDeactivated notification */

interface notifyDeltaSynchOfInstancesDeactivated: NotificationIRPNotifications::Notify

{

const string EVENT_TYPE = "notifyStatusOfDeltaSynchronization";

/**

* This constant defines the name of the ManagerReference property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::ManagerReferenceConditional.

*/

const string MANAGER_REFERENCE =

DeltaSynchronizationConstDefs::AttributeNameValue::MANAGER_REFERENCE;

/**

* This constant defines the name of the DeltaSynchStatusForCMData property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::DeltaSynchStatus.

*/

const string DELTA_SYNCH_STATUS_FOR_CMDATA =

DeltaSynchronizationConstDefs::AttributeNameValue::DELTA_SYNCH_STATUS_FOR_CMDATA;

/**

* This constant defines the name of the DeltaSynchStatusForAlarmData property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::DeltaSynchStatus.

*/

const string DELTA_SYNCH_STATUS_FOR_ALARM_CMDATA =

DeltaSynchronizationConstDefs::AttributeNameValue::DELTA_SYNCH_STATUS_FOR_ALARM_DATA;

};

/* Constant definitions for the notifyNewDeltaSynchPoint notification */

interface notifyNewDeltaSynchPoint: NotificationIRPNotifications::Notify

{

const string EVENT_TYPE = "notifyNewDeltaSynchPoint";

/**

* This constant defines the name of the AgentOrManagerReference property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::AgentOrManagerReference.

*/

const string AGENT_OR_MANAGER_REFERENCE =

DeltaSynchronizationConstDefs::AttributeNameValue::AGENT_OR_MANAGER_REFERENCE;

/**

* This constant defines the name of the NewDeltaSynchPoint property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::SynchPoint.

*/

const string NEW_DELTA_SYNCH_POINT =

DeltaSynchronizationConstDefs::AttributeNameValue::NEW_DELTA_SYNCH_POINT;

/**

* This constant defines the name of the RequestedSynchPoint property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::SynchPoint.

*/

const string REQUESTED_SYNCH_POINT =

DeltaSynchronizationConstDefs::AttributeNameValue::REQUESTED_SYNCH_POINT;

/**

* This constant defines the name of the DeltaSynchPointType property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs:: DeltaSynchPointType.

*/

const string DELTA_SYNCH_POINT_TYPE =

DeltaSynchronizationConstDefs::AttributeNameValue::DELTA_SYNCH_POINT_TYPE;

/**

* This constant defines the name of the TriggeredByAgentOrManager property,

* which is transported in the filterable_body fields.

* The data type for the value of this property is

* DeltaSynchronizationConstDefs::TriggeredBy.

*/

const string TRIGGERED_BY_AGENT_OR_MANAGER =

DeltaSynchronizationConstDefs::AttributeNameValue::TRIGGERED_BY_AGENT_OR_MANAGER;

};

};

#endif // _DELTA_SYNCHRONIZATION_NOTIFICATIONS_IDL_

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Mar 2007 | SA_35 | SP-070054 | -- | -- | Submitted to SA#35 for Information | -- | 1.0.0 | 2.0.0 |
| Jun 2007 | SA_36 | SP-070285 | -- | -- | Submitted to SA#36 for Approval | -- | 2.0.0 | 7.0.0 |
| Sep 2007 | SA_37 | SP-070612 | 0001 | -- | Remove operation setDeltaSynchPoint - Align with IS in 32.392 | F | 7.0.0 | 7.1.0 |
| Sep 2007 | SA_37 | SP-070612 | 0002 | -- | Corrections to IDL Definitions | F | 7.0.0 | 7.1.0 |
| Dec 2008 | SA_42 | -- | -- | -- | Upgrade to Release 8 | -- | 7.1.0 | 8.0.0 |
| Dec 2010 | SA_50 | SP-100744 | 0003 | -- | Correcting IDL syntax errors | F | 8.0.0 | 8.1.0 |