3GPP TSG-SA5 (Telecom Management)	S5-110094

SA5#75, 24 - 28 Jan 2011; Sorrento, ITALY	revision of S5-11abcd

# Source:	Nokia Siemens Networks

# Title:	Inventory object model enhancements

# Document for:	Approval

Agenda Item:	6.8.1 Study on Alignment of 3GPP Generic NRM IRP and TMF SID (460037)

# 1	Decision/action requested

Agree to the proposed pCR for TR 32.828

# 2	References

[1] 		3GPP TR 32.828 V1.4.0 (2010-12) Alignment of 3GPP Generic NRM IRP and TMF SID

[2] 		S5-103247 Inventory object model enhancements

# 3	Rationale

This contribution provides update to inventory recommendation on inventory object model.

Contribution content, object model, was discussed and agreed in SA5#74, 15 - 19 Nov 2010; Jacksonville meeting [2]

# 4	Detailed proposal

Begin of Change

# 6	Recommendations for 3GPP and TMF Resource Model Alignment

## 6.1	Inventory recommendations

### 6.1.1	General

This chapter identified the recommended changes based on comparison in chapter 4.3 and alignment chapter 5.3.

The recommendation is to update 3GPP inventory NRM to enable one specification capturing all necessary inventory items regardless of the technology. 3GPP IRP concept is also clearly separating the interface and resource model definitions, which is a key concept for enabling technology agnostic inventory management.

NGNM top 10 requirements [4] identifies inventory as one of the key functionalities. The content of the requirement is to have inventory information at inventory management system and changes in the inventory information should be available to related systems (“Notification of any change to a passive or active element or its configuration relevant to a business process must be possible” [4]).

### 6.1.2	Inventory model and information

Network element consists of one or several logical functionalities. The logical functionalities are realized by one or several HW units, related SW and controlling licences combinations. Current 3GPP inventory NRM captures already the NE level information in ManagedElement object. NE may contain one or several logical entities which may again contain either logical and/or physical entities. Current model enables only HW related split. To address this hierarchy of the logical and physical entities, the recommendation is to add following object

Editors Note:	The M/O qualifiers used below are FFS.

- inventoryUnit NE representing and realizing logical and physical structure of the NE. InventoryUnit NE should contain following attributes:

  - id: vendor defined unique identifier of a logical or physical network element unit

  - customerIdentifier: Unique identification of a vendors’ customer

  - productName: NE name classifying a vendor’s product family or function

  - vendorName: Name of inventory unit vendor (or vendors may provide manufacturer name)

  - productType: Identifier of the platform, in case the product can be based on different HW/SW platforms (not used for logical NEs)

  - salesUniqueId (O): Unique identifier used by vendor (used e.g. for ordering a new unit)

  - operatorUniqueName (O): Unique NE identifier used by operator

  - siteId (O):	NE site in customer network

  - additionalInformation (O): Supplementary information about NE inventory data (if any)

  - NEcomponents (O): contained logical or physical NE units with above attributes

InventoryUnit NE consists of HW units, SW units and licenses (LIC) controlling the functionalities. The SW and LIC may be the same for several entities of same type within one InventoryUnit NE. Same applies to HW in case of shared resources. To address properly the HW, SW and license items, the recommendation is to add following objects

- InventoryUnit HW (hardware) containing following attributes for least replaceable units

  - serialNo: Serial number given from factory (empty value in case of NE as highest equipment holder)

  - hwType: Type of the HW unit e.g. equipment holder, carriage

  - hwName (O):	Mnemonic of hw inventory unit family type (e.g. Fan, PSU) assigned by vendor.

  - hwVersion: Version / revision no. of current unit e.g. firmware version (empty value possible in case no versioning is available)

  - vendorName (O): Name of inventory unit vendor (or vendors may provide manufacturer name)

  - salesUniqueId (O): Unique identifier used by vendor (used e.g. for ordering a new unit)

  - hwUnitLocation: Unique physical / logical location identifier within NE

  - model: Equipment configuration, e.g. standard hw unit or a variant that may contain additional disk capacity (empty value possible))

  - hwCapability (O): Hardware capability e.g. capacity, size (empty value is possible)

  - modificationDate (O): Date/time stamp of last change (e.g. repair action)

  - additionalInformation (O): Supplementary information about HW inventory data (if any)

  - manualDataEntry: Indicates whether unit is passive or active

  - HardwareComponents (O): contained HW units (active or passive) with above attributes

- InventoryUnit SW (software) containing following attributes

  - id:	Unique identifier of a SW unit

  - swName (O):	SW release name used

  - swVersion: Version identifier of the SW unit

  - vendorName (O): Name of inventory unit vendor (or vendors may provide manufacturer name)

  - salesUniqueId (O): Unique identifier used by vendor (used e.g. for ordering a new unit)

  - classification: Name of installed SW (e.g. SW release, SW build, SW patches), empty value possible

  - swStatus: Status of the SW unit (e.g. installed, archived)

  - installationTime (O): Date/time stamp of SW installation

  - additionalInformation (O): Supplementary information about SW inventory data (if any)

  - SoftwareComponents (O): contained SW units with above attributes

- InventoryUnit LIC (license) containing following attributes

  - id: Unique identifier of a license (e.g. name, code)

  - lictype (O): Describing type of current license (e.g. capacity, particular feature,	no. of subscribers)

  - vendorName (O): Name of inventory unit vendor (or vendors may provide manufacturer name)

  - licActivationDate (O):	Date/time stamp of license activation

  - Validity (O):	License validity which may include one of the elements duration, end (expiration date) or forever

  - key (O): License activation key according to the used licensing system

  - licStatus (O): License status – applicable only for managed licenses (e.g. scheduled, valid, expired, invalid, capacity violated)

  - additionalInformation (O): Supplementary information about license inventory data (if any)

  - salesUniqueId: Unique identifier used by vendor (used e.g. for ordering a new unit)

In order to minimize the amount of data in inventory data file, the redundant information should be minimized. It is recommended that object shall contain relationships to other objects.

Element/domain management system is seen as part of network from network management perspective thus recommendation is to include EM/DM to inventory entities also.

Inventory data consist of information of the whole network managed by the operator. The information is related to network structure and entities (logical and physical entities), hardware components, software and licenses controlling the functionalities.

The inventory data amount may be huge. A large network consists of tens of thousands of network elements. Considering that network element hardware consist of racks, shelves, slots, cards and physical ports etc, the number of objects only for hardware inventory is enormous. Software items can also be seen/defined in a similar structured way than hardware, even if the SW structure is more vendor-specific than hardware.  SW items consist of releases, builds, modules etc. Thus the number of SW inventory items is also large. An estimation of 100 inventory items per network element may lead to millions of inventory items per network.

SW inventory items are to be linked to some network elements and/or hardware thus we need to define the SW information for every NE/HW entity (vendor dependent). Though in several cases, the same type of network elements run the same SW. Or a license applies to several network elements. Thus repeating the SW information for every NE/HW entity leads to huge amount of redundant information. License information can be compared to SW information thus creating additional redundant information. The target should be to eliminate “waste”, redundant information, without loosing any information. SW and license inventory items should appear only as “independent” inventory items and hardware and network element inventory items should have a relationship to these SW and license inventory items. An estimation of save in “space” is a factor 10. Also the logical and physical structure on inventory network element can be separated from hardware inventory items.

Figure 6.1.2.1: Inventory unit structure and some visualization of the nested structure

The example shown below shows how relationships between the different Inventory items could be defined (using an XML structure).

Figure 6.1.2.2: Relationships between the different Inventory items (example)

The figure below shows another example for NE inventory file.

Figure 6.1.2.3: Relationships between the different Inventory items (example 2)

The structure above would enable inventory data collection per network element resulting to an output file per network element. The final definition of network element inventory unit logical and other inventory units are defined at contract phase depending on the operator needs.

### 6.1.3 Inventory interface enhancement

Currently two methods are defined in 3GPP for uploading inventory information:

- Explicitly via Bulk CM IRP / Inventory NRM IRP

- Implicitly via FT IRP / Inventory NRM IRP (with the notification capabilities of the FT IRP to inform an IRP Manager about the availability of changed inventory information)

It is proposed to establish the method “FT IRP / Inventory NRM IRP” as a viable alternative for transferring inventory information. Reason: The inventory information is either static or semi static meaning that the information does not change (static) or changes very seldom (semi static). For this reason, inventory information does not need to be updated and transferred to network management system in real time nor as frequently as configuration management information. Depending on the case the update may happen e.g. once per day or once per week or per need basis. The inventory data could be stored per network element in an inventory file. The files would be available for upload per request or per schedule using existing File Transfer IRP [3] capabilities, which already allows for inventory data transfer – though a File Format Definition will have to be defined based on the enhanced inventory information outlined in this study. File Transfer IRP contains also the notification mechanism (notifyFileReady).

### 6.1.4	Inventory object model

InventoryUnitNE IOC definition

This IOC represents the logical and physical structure of the NE.

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M |  |
| neId | M | M |  |
| customerIdentifier | O | M | - |
| productName | M | M | - |
| vendorName | M | M | - |
| productType | M | M | - |
| salesUniqueId | O | M | - |
| operatorUniqueName | O | M | - |
| siteId | O | M | - |
| additionalInformation | O | M | - |
| neComponents | O | M | - |

InventoryUnitHw IOC definition

This IOC represents the hardware components.

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| serialNo | M | M | - |
| hwType | M | M | - |
| hwName | O | M | - |
| hwVersion | M | M | - |
| vendorName | O | M | - |
| salesUniqueId | O | M | - |
| hwUnitLocation | M | M | - |
| model | M | M | - |
| hwCapability | O | M | - |
| modificationDate | O | M | - |
| additionalInformation | O | M | - |
| manualDataEntry | M | M | - |
| hardwareComponents | O | M | - |

InventoryUnitSw IOC definition

This IOC represents the software components.

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| swId | M | M | - |
| swName | O | M | - |
| swVersion | O | M | - |
| vendorName | O | M | - |
| salesUniqueId | O | M | - |
| classification | M | M | - |
| swStatus | O | M | - |
| installationTime | O | M | - |
| additionalInformation | O | M | - |
| softwareComponents | O | M | - |

InventoryUnitLic IOC definition

This IOC represents the licence components.

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| licId | M | M | - |
| licType | O | M | - |
| vendorName | O | M | - |
| licActivationDate | O | M | - |
| validity | O | M | - |
| key | O | M | - |
| licStatus | O | M | - |
| salesUniqueId | O | M | - |
| additionalInformation | O | M | - |

Information attribute definitions

| Attribute Name | Definition | Information Type / Legal Values |
| --- | --- | --- |
| additionalInformation | Supplementary information about inventory data (if any) |  |
| classification | Name of installed SW (e.g. SW release, SW build, SW patches), empty value possible |  |
| customerIdentifier | Unique identification of a vendors’ customer |  |
| hwCapability | Hardware capability e.g. capacity, size (empty value is possible) |  |
| hwName | Mnemonic of hw inventory unit family type (e.g. Fan, PSU) assigned by vendor. |  |
| hwType | Type of the HW unit e.g. equipment holder, carriage |  |
| hwUnitLocation | Unique physical / logical location identifier within NE |  |
| hwVersion | Version / revision no. of current unit e.g. firmware version (empty value possible in case no versioning is available) |  |
| id | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of this object class. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| installationTime | Date/time stamp of SW installation |  |
| key | License activation key according to the used licensing system |  |
| licActivationDate | Date/time stamp of license activation |  |
| licId | Unique identifier of a license (e.g. name, code) |  |
| licStatus | License status – applicable only for managed licenses (e.g. scheduled, valid, expired, invalid, capacity violated) |  |
| licType | Describing type of current license (e.g. capacity, particular feature, no. of subscribers) |  |
| manualDataEntry | Indicates whether unit is passive (manual insertion of inventory data is needed) or active (inventory data can be read from the unit) |  |
| model | Equipment configuration, e.g. standard hw unit or a variant that may contain additional disk capacity (empty value possible) |  |
| modificationDate | Date/time stamp of last change (e.g. repair action) |  |
| neId | vendor defined unique identifier of a logical or physical network element unit |  |
| operatorUniqueName | Unique NE identifier used by operator |  |
| productName | NE name classifying a vendor’s product family or function |  |
| productType | Identifier of the e.g. platform, in case the product can be based on different HW/SW platforms (not used for logical NEs) |  |
| salesUniqueId | Unique identifier used by vendor (used e.g. for ordering a new unit) |  |
| serialNo | Serial number given from factory. |  |
| siteId | NE site in customer network |  |
| swId | Unique identifier of a SW unit |  |
| swName | SW release name used |  |
| swStatus | Status of the SW unit (e.g. installed, archived) |  |
| swVersion | Version identifier of the SW unit |  |
| validity | License validity which may include one of the elements duration, end (expiration date) or forever |  |
| vendorName | Name of inventory unit vendor (or vendors may provide manufacturer name) |  |

End of Change