3GPP TS 32.732 V11.0.0 (2011-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

IP Multimedia Subsystem (IMS)

Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

(Release 11)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2011, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	System overview	10

4.1	Compliance rules	10

5	Modelling approach	10

6	Information Object Classes	10

6.1	Imported information entities and local labels	10

6.2	Class diagram	11

6.2.1	Attributes and relationships	11

6.2.2	Inheritance	16

6.3	Information object class definitions	20

6.3.1	ASFunction	20

6.3.1.1	Definition	20

6.3.1.2	Attributes	20

6.3.1.3	Notifications	20

6.3.2	BGCFFunction	20

6.3.2.1	Definition	20

6.3.2.2	Attributes	20

6.3.2.3	Notifications	20

6.3.3	CAMELIMSSFASFunction	20

6.3.3.1	Definition	20

6.3.3.2	Attributes	20

6.3.4	CSCFFunction	21

6.3.4.1	Definition	21

6.3.4.2	Attributes	21

6.3.4.3	Notifications	21

6.3.5	HSSFunction	21

6.3.5.1	Definition	21

6.3.5.2	Attributes	21

6.3.5.3	Notifications	21

6.3.6	ICSCFFunction	21

6.3.6.1	Definition	21

6.3.6.2	Attributes	21

6.3.6.3	Notifications	21

6.3.7	IMSMGWFunction	21

6.3.7.1	Definition	21

6.3.7.2	Attributes	22

6.3.7.3	Notifications	22

6.3.8	MGCFFunction	22

6.3.8.1	Definition	22

6.3.8.2	Attributes	22

6.3.8.3	Notifications	22

6.3.9	MRFCFunction	22

6.3.9.1	Definition	22

6.3.9.2	Attributes	22

6.3.9.3	Notifications	22

6.3.10	MRFPFunction	23

6.3.10.1	Definition	23

6.3.10.2	Attributes	23

6.3.10.3	Notifications	23

6.3.11	OSASCSASFunction	23

6.3.11.1	Definition	23

6.3.11.2	Attributes	23

6.3.12	PCSCFFunction	23

6.3.12.1	Definition	23

6.3.12.2	Attributes	23

6.3.12.3	Notifications	23

6.3.13	SCSFFunction	23

6.3.13.1	Definition	23

6.3.13.2	Attributes	23

6.3.13.3	Notifications	23

6.3.14	SIPASFunction	24

6.3.14.1	Definition	24

6.3.14.2	Attributes	24

6.3.15	SLFFunction	24

6.3.15.1	Definition	24

6.3.15.2	Attributes	24

6.3.15.3	Notifications	24

6.3.16	ECSCFFunction	24

6.3.16.1	Definition	24

6.3.16.2	Attributes	24

6.3.16.3	Notifications	24

6.3.50	Reserved for Future Use	24

6.3.51	Void	24

6.3.52	Link_AS_ICSCF	24

6.3. 52.1	Definition	24

6.3.53	Link_AS_SCSCF	25

6.3.53.1	Definition	25

6.3.54	Link_AS_SLF	25

6.3.54.1	Definition	25

6.3.55	Link_BGCF_BGCF	25

6.3.55.1	Definition	25

6.3.56	VoidVoid	25

6.3.57	Link_BGCF_MGCF	25

6.3.57.1	Definition	25

6.3.58	Link_BGCF_SCSCF	25

6.3.58.1	Definition	25

6.3.59	Void	25

6.3.60	Link_HSS_ICSCF	25

6.3.60.1	Definition	25

6.3.61	Link_ICSCF_SCSCF	25

6.3.61.1	Definition	25

6.3.62	Link_ICSCF_MGCF	25

6.3.62.1	Definition	25

6.3.64	Link_ICSCF_PCSCF	26

6.3.64.1	Definition	26

6.3.65	Link_PCSCF_SCSCF	26

6.3.65.1	Definition	26

6.3.67	Link_CAMELIMSSFAS_HSS	26

6.3.67.1	Definition	26

6.3.68	Link_HSS_OSASCSAS	26

6.3.68.1	Definition	26

6.3.69	Link_HSS_SCSCF	26

6.3.69.1	Definition	26

6.3.70	Link_HSS_SIPAS	26

6.3.70.1	Definition	26

6.3.71	Link_ICSCF_SLF	26

6.3.71.1	Definition	26

6.3.72	Link_IMSMGW_MGCF	26

6.3.72.1	Definition	26

6.3.73	Link_MGCF_SCSCF	27

6.3.73.1	Definitions	27

6.3.74	Link_MRFC_MRFP	27

6.3.74.1	Definition	27

6.3.75	Link_MRFC_SCSCF	27

6.3.75.1	Definition	27

6.3.76	Link_SCSCF_SCSCF	27

6.3.76.1	Definition	27

6.3.77	Link_SCSCF_SLF	27

6.3.77.1	Definition	27

6.3.78	Link_PCSCF_ECSCF	27

6.3.78.1	Definition	27

6.3.79	Link_BGCF_ECSCF	27

6.3.79.1	Definition	27

6.3.80	Link_MGCF_ECSCF	27

6.3.80.1	Definition	27

6.4	Information relationship definitions	27

6.5	Information attribute definitions	28

6.5.1	Definition and legal values	28

6.5.2	Constraints	28

6.6	Common notifications	29

6.7	Particular information configurations	29

Annex A (informative):	Change history	30

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

32.731:	"IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements".

32.732:	"IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

32.736:	"IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Solution Set (SS) definitions".

# 1	Scope

The present document is part of an Integration Reference Point (IRP) named IMS Network Resource Model (NRM) IRP, through which an IRPAgent can communicate configuration management information to one or several IRPManagers concerning IMS resources. The IMS NRM IRP comprises a set of specifications defining Requirements, a protocol neutral Information Service and one or more Solution Set(s).

The present document specifies the protocol neutral IMS NRM IRP: Information Service (IS). It reuses relevant parts of the Generic NRM IRP: IS in 3GPP TS 32.622 [9], either by direct reuse or sub-classing, and in addition to that defines IMS specific Information Object Classes.

Finally, in order to access the information defined by this NRM, an Interface IRP is needed, such as the Basic CM IRP in 3GPP TS 32.602 [10]. However, which Interface IRP that is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM IRP Information Service (IS)".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and definitions".

[5]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point: Information Service (IS)".

[6]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[7]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[8]	3GPP TS 23.002: "Network architecture".

[9]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[10]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic Configuration Management Integration Reference Point (IRP): Information Service (IS)".

[11]	3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [7] and the following apply:

Association: In general it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

(1)	name bindings;

(2)	reference attributes; and

(3)	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Information Object Class (IOC): See 3GPP TS 32.150 [4].

Managed Element (ME): an instance of the Information Object Class ManagedElement defined in 3GPP TS 32.622 [9].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. The MO is an instance of a MO class defined in an NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Network Resource Model (NRM): a model representing the actual managed telecommunications network resources that a System is providing through the subject IRP
An NRM identifies and describes IOCs, their associations, attributes and operations.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AUC	AUthentication Centre

AS	Application Server

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Billing System

CBC	Cell Broadcast Center

CGF	Charging Gateway Functionality

CN	Core Network

DN	Distinguished Name (see 3GPP TS 32.300 [6])

ECSCF	Emergency Call Session Control Function

EIR	Equipment Identity Register

EM	Element Manager

FM	Fault Management

FNR	Flexible Number Register

GDMO	Guidelines for the Definition of Managed Objects

GGSN	Gateway GPRS Support Node

GMLC	Gateway Mobile Location Center

GMSC Server	Gateway MSC Server

GMSC	Gateway MSC

GPRS	General Packet Radio System

ICSCF	Interrogating Call Session Control Function

IDL	Interface Definition Language

IMS	IP Multimedia Subsystem

IMSMGW	IMS Media Gateway

IOC	Information Object Class

IRP	Integration Reference Point

ISO	International Standards Organization

IWF	InterWorking Function

ME	Managed Element

MGCF	Media Gateway Control Function

MGW	Media GateWay

MNP-SRF	Mobile Number Portability-Signalling Relay Function

MO	Managed Object

MOI	Managed Object Instance

MRFC	Multimedia Resource Function Controller

MRFP	Call Session Control Function Processor

MSC Server	Mobile Services Switching Centre Server

MSC	Mobile Services Switching Centre

NE	Network Element

NM	Network Manager

NPDB	Number Portability DataBase

NR	Network Resource

NRM	Network Resource Model

OSI	Open Systems Interconnection

PCSCF	Proxy Call Session Control Function

PM	Performance Management

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [6])

SCF	Service Control Function

SCSCF	Serving Call Session Control Function

SGSN	Serving GPRS Support Node

SGW	Signalling GateWay

SLF	Subscription Locator Function

SMLC	Serving Mobile Location Center

SMS	Short Message Service

SMS-GMSC	SMS Gateway MSC

SMS-IWMSC	SMS InterWorking MSC

SRF	Specialized Resource Function

SSF	Service Switching Function

TMN	Telecommunications Management Network

UML	Unified Modelling Language

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

VLR	Visitor Location Register

# 4	System overview

## 4.1	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional information object classes, attributes and associations without requiring the IRPManager to have any knowledge of the extensions.

Given that:

- rules for vendor-specific extensions remain to be fully specified; and

- many scenarios under which IRPManager and IRPAgent interwork may exist;

it is recognized that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach is described in the Generic Network Resources IRP: NRM (3GPP TS 32.622 [9]).

It should be noted that this model allows for combined managed element functionality, where more than one ‘function IOCs' (inherited from ManagedFunction) modelling more specific managed element functionality may be contained in the ManagedElement IOC.

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 32.622 [9], information object class, Link | Link |
| 32.622 [9], information object class, ManagedElement | ManagedElement |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The figures below show the containment/naming hierarchy and the associations of the information object classes defined in the present document.

NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios in all figures.

Figure 6.2.1: IMS NRM Containment/Naming relationships

Note:	If this NRM has interfaces (Link IOCs) modelled to IOCs in other NRM(s), the Link IOC definitions may be defined in the other NRM(s) and need to be considered in implementations.

Figure 6.2.1.1: IMS NRM Link Associations 1

Figure 6.2.1.2: IMS NRM Link Associations 2

Figure 6.2.1.3: IMS NRM Link Associations 3

Figure 6.2.1.4: IMS NRM Link Associations 4

Figure 6.2.1.5: IMS NRM Link Associations 5

Figure 6.2.1.6: IMS NRM Link Associations 6

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between  IOCs.

Figure 6.2.2.1: IMS NRM Inheritance Hierarchy 1

Figure 6.2.2.2: IMS NRM Inheritance Hierarchy 2

Figure 6.2.2.3: IMS NRM Inheritance Hierarchy 3

Figure 6.2.2.4: IMS NRM Inheritance Hierarchy 4

Figure 6.2.2.5: IMS NRM Inheritance Hierarchy 5

Figure 6.2.2.6: IMS NRM Inheritance Hierarchy 6

Figure 6.2.2.7: IMS NRM Inheritance Hierarchy 7

## 6.3	Information object class definitions

### 6.3.1	ASFunction

#### 6.3.1.1	Definition

This IOC represents AS functionality. For more information about the AS, see 3GPP TS 23.002 [8]. AsFunction may be instantiated when a specific Application Server type is not already represented with an IOC subclassed from this IOC in this specification. If the Application Server type is already represented by an IOC in this specification, then it should instead be used. In the case a subclassed IOC is utilized, the AsFunction relationships (including link objects) are carried forward to the sub-classed IOC through inheritance.

#### 6.3.1.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| asFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.1.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.2	BGCFFunction

#### 6.3.2.1	Definition

This IOC represents BGCF functionality. For more information about the BGCF, see 3GPP TS 23.002 [8].

#### 6.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| bgcfFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.2.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.3	CAMELIMSSFASFunction

#### 6.3.3.1	Definition

This IOC represents CAMEL IM-SSF AS functionality. For more information about the CAMEL IM-SSF AS, see 3GPP TS 23.002 [8].

#### 6.3.3.2	Attributes

Void.

### 6.3.4	CSCFFunction

#### 6.3.4.1	Definition

This IOC represents CSCF functionality. For more information about the CSCF, see 3GPP TS 23.002 [8].

This IOC cannot be instantiated. It is defined for sub-classing purposes.

#### 6.3.4.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| cscfFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.4.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.5	HSSFunction

#### 6.3.5.1	Definition

This IOC represents HSS functionality. For more information about the HSS, see 3GPP TS 23.002 [8].

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| hssFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.5.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.6	ICSCFFunction

#### 6.3.6.1	Definition

This IOC represents I-CSCF functionality. For more information about the I-CSCF, see 3GPP TS 23.002 [8].

#### 6.3.6.2	Attributes

Void.

#### 6.3.6.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.7	IMSMGWFunction

#### 6.3.7.1	Definition

This IOC represents IMS-MGW functionality. For more information about IMS-MGW, see 3GPP TS 23.002 [8].

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| imsMgwFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.7.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.8	MGCFFunction

#### 6.3.8.1	Definition

This IOC represents MGCF functionality. For more information about the MGCF, see 3GPP TS 23.002 [8].

#### 6.3.8.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| mgcfFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.8.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.9	MRFCFunction

#### 6.3.9.1	Definition

This IOC represents MRFC functionality. For more information about the MRFC, see 3GPP TS 23.002 [8].

#### 6.3.9.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| mrfcFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.9.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.10	MRFPFunction

#### 6.3.10.1	Definition

This IOC represents MRFP functionality. For more information about the MRFP, see 3GPP TS 23.002 [8].

#### 6.3.10.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| mrfpFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.10.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.11	OSASCSASFunction

#### 6.3.11.1	Definition

This IOC represents OSA Application Server (Service Capability Server) functionality. For more information about the OSA Service Capability Server, see 3GPP TS 23.002 [8].

#### 6.3.11.2	Attributes

Void.

### 6.3.12	PCSCFFunction

#### 6.3.12.1	Definition

This IOC represents P-CSCF functionality. For more information about the P-CSCF, see 3GPP TS 23.002 [8].

#### 6.3.12.2	Attributes

Void.

#### 6.3.12.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.13	SCSFFunction

#### 6.3.13.1	Definition

This IOC represents S-CSCF functionality. For more information about the S-CSCF, see 3GPP TS 23.002 [8].

#### 6.3.13.2	Attributes

Void.

#### 6.3.13.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.14	SIPASFunction

#### 6.3.14.1	Definition

This IOC represents SIP AS functionality. For more information about the SIP AS, see 3GPP TS 23.002 [8].

#### 6.3.14.2	Attributes

Void.

### 6.3.15	SLFFunction

#### 6.3.15.1	Definition

This IOC represents SLF functionality. For more information about the SLF, see 3GPP TS 23.002 [8].

#### 6.3.15.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| slfFunctionId | M | M | - |
| linkList | O | M | - |

#### 6.3.15.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.16	ECSCFFunction

#### 6.3.16.1	Definition

This IOC represents E-CSCF functionality. For more information about the E-CSCF, see 3GPP TS 23.002 [8].

#### 6.3.16.2	Attributes

Void.

#### 6.3.16.3	Notifications

The common notifications defined in subclause 6.6 are valid for this IOC, without exceptions or additions.

### 6.3.50	Reserved for Future Use

### 6.3.51	Void

### 6.3.52	Link_AS_ICSCF

#### 6.3. 52.1	Definition

This IOC models the Ma reference point as defined in TS 23.002 [8].

### 6.3.53	Link_AS_SCSCF

#### 6.3.53.1	Definition

This IOC models the Isc reference point as defined in TS 23.002 [8].

### 6.3.54	Link_AS_SLF

#### 6.3.54.1	Definition

This models the Dh reference point as defined in TS 23.002 [8].

### 6.3.55	Link_BGCF_BGCF

#### 6.3.55.1	Definition

This models the Mk reference point as defined in TS 23.002 [8].

### 6.3.56	VoidVoid

### 6.3.57	Link_BGCF_MGCF

#### 6.3.57.1	Definition

This models the Mj reference point as defined in TS 23.002 [8].

### 6.3.58	Link_BGCF_SCSCF

#### 6.3.58.1	Definition

This models the Mi reference point as defined in TS 23.002 [8].

### 6.3.59	Void

### 6.3.60	Link_HSS_ICSCF

#### 6.3.60.1	Definition

This IOC models the Cx reference point as defined in TS 23.002 [8].

### 6.3.61	Link_ICSCF_SCSCF

#### 6.3.61.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

### 6.3.62	Link_ICSCF_MGCF

#### 6.3.62.1	Definition

This IOC models the Mg reference point as defined in TS 23.002 [8].

6.3.63	VoidVoid

### 6.3.64	Link_ICSCF_PCSCF

#### 6.3.64.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

### 6.3.65	Link_PCSCF_SCSCF

#### 6.3.65.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

6.3.66	 Void

### 6.3.67	Link_CAMELIMSSFAS_HSS

#### 6.3.67.1	Definition

This IOC models the Si reference point between CAMEL Application Server (IM-SSF) and HSS as defined in TS 23.002 [8].

### 6.3.68	Link_HSS_OSASCSAS

#### 6.3.68.1	Definition

This IOC models the Sh reference point between OSA Application Server (Service Capability Server) and HSS as defined in TS 23.002 [8].

### 6.3.69	Link_HSS_SCSCF

#### 6.3.69.1	Definition

This IOC models the Cx reference point as defined in TS 23.002 [8].

### 6.3.70	Link_HSS_SIPAS

#### 6.3.70.1	Definition

This IOC models the Sh reference point between SIP Application Server and HSS as defined in TS 23.002 [8].

### 6.3.71	Link_ICSCF_SLF

#### 6.3.71.1	Definition

This models the Dx reference point as defined in TS 23.002 [8].

### 6.3.72	Link_IMSMGW_MGCF

#### 6.3.72.1	Definition

This models the Mn reference point as defined in TS 23.002 [8].

### 6.3.73	Link_MGCF_SCSCF

#### 6.3.73.1	Definitions

This models the Mg reference point as defined in TS 23.002 [8].

### 6.3.74	Link_MRFC_MRFP

#### 6.3.74.1	Definition

This IOC models the Mp reference point as defined in TS 23.002 [8].

### 6.3.75	Link_MRFC_SCSCF

#### 6.3.75.1	Definition

This IOC models the Mr reference point as defined in TS 23.002 [8].

### 6.3.76	Link_SCSCF_SCSCF

#### 6.3.76.1	Definition

This models the Mw reference point as defined in TS 23.002 [8].

### 6.3.77	Link_SCSCF_SLF

#### 6.3.77.1	Definition

This IOC models the Dx reference point as defined in TS 23.002 [8].

### 6.3.78	Link_PCSCF_ECSCF

#### 6.3.78.1	Definition

This IOC models the Mw reference point as defined in TS 23.002 [8].

### 6.3.79	Link_BGCF_ECSCF

#### 6.3.79.1	Definition

This IOC models the Mi reference point as defined in TS 23.002 [8].

### 6.3.80	Link_MGCF_ECSCF

#### 6.3.80.1	Definition

This IOC models the Mg reference point as defined in TS 23.002 [8].

## 6.4	Information relationship definitions

In this version of the present document there are no information relationship definitions.

## 6.5	Information attribute definitions

### 6.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| asFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| bgcfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| cscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| icscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| imsMgwFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| linkList | An attribute containing all of the related link object Distinguished Names for the object instance, if any. |  |
| mgcfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mrfcFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| mrfpFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| pcscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| scscfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| slfFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |
| hssFunctionId | An attribute whose ‘name+value’ can be used as an RDN when naming an instance of the IOC. This RDN uniquely identifies the object instance within the scope of its containing (parent) object instance. |  |

### 6.5.2	Constraints

| Name | Definition |
| --- | --- |
| - | - |

## 6.6	Common notifications

This subclause contains a list of notifications that can be referred to by any IOC defined by this IRP specification. 
These notifications are only applicable to IOCs referring to this subclause.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [5]) |  |
| notifyObjectCreation | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |
| notifyObjectDeletion | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |
| notifyAttributeValueChange | See Kernel CM IRP (3GPP TS 32.662 [3]) |  |

## 6.7	Particular information configurations

Not applicable.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | R | Subject/Comment | Cat | Old | New |
| Sep 2006 | SA_33 | SP-060563 | -- | -- | Submitted to TSG SA #33 for Information | -- | -- | 1.0.0 |
| Dec 2006 | SA_34 | SP-060750 | -- | -- | Submitted to TSG SA #34 for Approval | -- | 2.0.0 | 7.0.0 |
| Mar 2007 | SA_35 | SP-070047 | 0001 | -- | Add HssFunction to IMS NRM | F | 7.0.0 | 7.1.0 |
| Jun 2007 | SA_36 | SP-070276 | 0002 | -- | Change in cardinalities of IMS managed functions | F | 7.1.0 | 7.2.0 |
| Jun 2007 | SA_36 | SP-070276 | 0003 | -- | Correct definitions of AsFunctions - Align with 23.002 | F | 7.1.0 | 7.2.0 |
| Jun 2007 | SA_36 | SP-070276 | 0004 | -- | Add missing Link_As_Icscf to IMS NRM - Align with 23.002 | F | 7.1.0 | 7.2.0 |
| Sep 2007 | SA_37 | SP-070612 | 0006 | -- | Add missing link attributes to IMS NRM - Align with 3GPP2 | F | 7.2.0 | 7.3.0 |
| Sep 2007 | SA_37 | SP-070610 | 0005 | -- | Correct description of common notifications - Align with Rel-8 32.151 IRP IS Template | F | 7.3.0 | 8.0.0 |
| Jun 2008 | SA_40 | SP-080329 | 0007 | -- | Correction of the CamelImSsfAsFunction definition | F | 8.0.0 | 8.1.0 |
| Dec 2009 | - | - | - | - | Update to Rel-9 version | -- | 8.1.0 | 9.0.0 |
| Dec 2010 | SA_50 | SP-100758 | 0008 | 5 | Correcting CscfFunction definition of IMS NRM - Align with SA2 TS 23.002 | F | 9.0.0 | 9.1.0 |
| Mar 2011 | - | - | - | - | Update to Rel-10 version (MCC) | - | 9.1.0 | 10.0.0 |
| Sep 2011 | SA_53 | SP-110536 | 0010 | - | Add ECSCFFunction to IMS NRM | B | 10.0.0 | 11.0.0 |