3GPP TS 32.752 V10.2.0 (2012-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Evolved Packet Core (EPC)

Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

EPC, E-UTRAN, MME, management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2012, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	System overview	8

4.1	Compliance rules	8

5	Modelling approach	8

6	Information Object Classes	9

6.1	Imported information entities and local labels	9

6.2	Class diagram	9

6.2.1	Attributes and relationships	9

6.2.2	Inheritance	14

6.3	Information Object Class (IOC) definitions	16

6.3.1	EPDGFunction	16

6.3.1.1	Definition	16

6.3.2	MMEFunction	16

6.3.2.1	Definition	16

6.3.2.2	Attributes	16

6.3.2.3	Attribute constraints	17

6.3.3	PCRFFunction	17

6.3.3.1	Definition	17

6.3.4	PGWFunction	17

6.3.4.1	Definition	17

6.3.5	ServingGWFunction	17

6.3.5.1	Definition	17

6.3.5.2	Attributes	17

6.3.6	MMEPool	17

6.3.6.1	Definition	17

6.3.6.2	Attributes	17

6.3.6.3	Attribute constraints	18

6.3.7	MMEPoolArea	18

6.3.7.1	Definition	18

6.3.7.2	Attributes	18

6.3.7.3	Attribute constraints	18

6.3.8	Link_ENB_MME	18

6.3.8.1	Definition	18

6.3.9	Link_ENB_ServingGW	18

6.3.9.1	Definition	18

6.3.10	Link_EPDG_PCRF	18

6.3.10.1	Definition	18

6.3.11	Link_EPDG_PGW	18

6.3.11.1	Definition	18

6.3.12	Link_HSS_MME	19

6.3.12.1	Definition	19

6.3.13	Link_MME_MME	19

6.3.13.1	Definition	19

6.3.14	Link_MME_SGSN	19

6.3.14.1	Definition	19

6.3.15	Link_MME_ServingGW	19

6.3.15.1	Definition	19

6.3.16	Link_PCRF_ServingGW	19

6.3.16.1	Definition	19

6.3.17	Link_PCRF_PGW	19

6.3.17.1	Definition	19

6.3.18	Link_PGW_ServingGW	19

6.3.18.1	Definition	19

6.3.19	Link_SGSN_ServingGW	19

6.3.19.1	Definition	19

6.3.20	EP_RP_EPS	20

6.3.20.1	Definition	20

6.3.20.2	Attributes	20

6.3.20.3	Attribute constraints	20

6.3.20.4	Notifications	20

6.3.21	ExternalServingGWFunction	20

6.3.21.1	Definition	20

6.3.21.2	Attributes	20

6.3.22	ExternalMMEFunction	20

6.3.22.1	Definition	20

6.3.22.2	Attributes	21

6.3.23	IOC QCISet	21

6.3.23.1	Definition	21

6.3.23.2	Attributes	21

6.3.23.3	Attribute constraints	21

6.3.23.4	Notifications	21

6.3.24	MBMSGWFunction	21

6.3.24.1	Definition	21

6.3.24.2	Attributes	21

6.3.25	Link_MBMSGW_ENB	21

6.3.25.1	Definition	21

6.4	Information Relationship definitions	21

6.4.1	MMEPoolArea-MMEPoolRelation (M)	21

6.4.1.1	Definition	21

6.4.1.2	Roles	22

6.4.1.3	Constraints	22

6.4.2	MMEPool-MMERelation (M)	22

6.4.2.1	Definition	22

6.4.2.2	Roles	22

6.4.2.3	Constraints	22

6.4.3	MMEPool-ExternalMMERelation (M)	22

6.4.3.1	Definition	22

6.4.3.2	Roles	22

6.4.3.3	Constraints	22

6.5	Information Attribute definitions	23

6.5.1	Definition and legal values	23

6.6	Common notifications	24

Annex A (informative):	Change history	25

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Telecommunication management; as identified below:

32.751:	"Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Requirements";

32.752:	"Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)";

32.753:	"Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Common Object Request Broker Architecture (CORBA) Solution Set (SS)";

32.755:	"Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Bulk CM eXtensible Markup Language (XML) file format definition".

Configuration Management (CM), in general, provides the operator with the ability to assure correct and effective operation of the 3G network as it evolves. CM actions have the objective to control and monitor the actual configuration on the Network Elements (NEs) and Network Resources (NRs), and they may be initiated by the operator or by functions in the Operations Systems (OSs) or NEs.

CM actions may be requested as part of an implementation programme (e.g. additions and deletions), as part of an optimization programme (e.g. modifications), and to maintain the overall Quality of Service (QoS). The CM actions are initiated either as single actions on single NEs of the 3G network, or as part of a complex procedure involving actions on many resources/objects in one or several NEs.

# 1	Scope

The present document specifies the protocol neutral EPC Network Resource Model IRP Information Service, through which an 'IRPAgent' (typically an Element Manager or Network Element) can communicate information to one or several 'IRPManagers' (typically Network Managers) concerning EPC resources. It reuses relevant parts of the Generic NRM in 3GPP TS 32.622 [6], either by direct reuse or sub-classing, and in addition to that defines EPC specific Information Object Classes.

In order to access the information defined by this NRM, an Interface IRP such as the "Basic CM IRP" is needed (3GPP TS 32.602 [7]). However, which Interface IRP is applicable is outside the scope of the present document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[5]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[6]	3GPP TS 32.622: "Telecommunication management; Configuration Management (CM); Generic network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[7]	3GPP TS 32.602: "Telecommunication management; Configuration Management (CM); Basic Configuration Management Integration Reference Point (IRP): Information Service (IS)".

[8]		3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[9]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[10]	3GPP TS 32.732: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[11]	3GPP TS 32.762: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) network resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[12]	3GPP TS 23.402: "Architecture Enhancements for non-3GPP accesses".

[13]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM; Information service (IS)".

[14]	3GPP TS 23.003: " Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[15]	3GPP TR 32.816-160: "Telecommunication management; Study on management of Evolved Universal Terrestrial Radio Access Network (E-UTRAN) and Evolved Packet Core (EPC) ".

[16]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[17]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[18]	3GPP TS 36.331: " Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); S1 Application Protocol (S1AP)".

[19]	3GPP TS 23.203: "Policy and charging control architecture”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.600 [5] and the following apply:

Association: In general it is used to model relationships between Managed Objects. 
Associations can be implemented in several ways, such as:

(1)	name bindings;

(2)	reference attributes; and

(3)	association objects.

This IRP stipulates that containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams).

Managed Element (ME): an instance of the Information Object Class ManagedElement defined in 3GPP TS 32.622 [6].

Managed Object (MO): in the context of the present document, a Managed Object (MO) is a software object that encapsulates the manageable characteristics and behaviour of a particular Network Resource. 
The MO is instance of a MO class defined in a MIM/NRM. This class, called Information Object Class (IOC) has attributes that provide information used to characterize the objects that belong to the class (the term "attribute" is taken from TMN and corresponds to a "property" according to CIM). Furthermore, the IOC can have operations that represent the behaviour relevant for that class (the term "operation" is taken from TMN and corresponds to a "method" according to CIM). The IOC may support the emission of notifications that provide information about an event occurrence within a network resource.

Management Information Model (MIM): also referred to as NRM - see the definition below.

Network Resource Model (NRM): a model representing the actual managed telecommunications network resources that a System is providing through the subject IRP. 
An NRM identifies and describes IOCs, their associations, attributes and operations. 
The NRM is also referred to as "MIM" (see above), which originates from the ITU-T TMN.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CIM	Common Information Model

EM	Element Manager

eNodeB	evolved NodeB

EPC	Evolved Packet Core

ePDG	evolved Packet Data Gateway

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

GPRS	General Packet Radio System

IOC	Information Object Class

IRP	Integration Reference Point

IS	Information Service

ME	Managed Element

MIM	Management Information Model

MME	Mobility Management Entity

MO	Managed Object

NE	Network Element

NR	Network Resource

NRM	Network Resource Model

PCRF	Policy and Charging Rules Function

P-GW	PDN Gateway

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [4])

S-GW	Serving Gateway

TMN	Telecom Management Network

UML	Unified Modelling Language

# 4	System overview

## 4.1	Compliance rules

The following defines the meaning of Mandatory and Optional IOC attributes and associations between IOCs, in Solution Sets to the IRP defined by the present document:

- The IRPManager shall support all mandatory attributes/associations. 
The IRPManager shall be prepared to receive information related to mandatory as well as optional attributes/associations without failure; however the IRPManager does not have to support handling of the optional attributes/associations.

- The IRPAgent shall support all mandatory attributes/associations. It may support optional attributes/associations.

An IRPAgent that incorporates vendor-specific extensions shall support normal communication with a 3GPP SA5-compliant IRPManager with respect to all Mandatory and Optional IOCs, attributes and associations without requiring the IRPManager to have any knowledge of the extensions.

Given that:

- rules for vendor-specific extensions remain to be fully specified; and

- many scenarios under which IRPManager and IRPAgent interwork may exist;

it is recognized that the IRPManager, even though it is not required to have knowledge of vendor-specific extensions, may be required to be implemented with an awareness that extensions can exist and behave accordingly.

# 5	Modelling approach

The modelling approach is described in the "Generic network resources IRP NRM" (3GPP TS 32.622 [6]).

It should be noted that this model allows for combined Managed Element functionality, where more than one 'function IOCs' (inherited from ManagedFunction) modelling more specific managed element functionality may be contained in the ManagedElement IOC.

# 6	Information Object Classes

## 6.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 32.762 [11], information object class, ENBFunction | ENBFunction |
| TS 32.732 [10], information object class, HssFunction | HssFunction |
| TS 32.622 [6], information object class, Link | Link |
| TS 32.622 [6], information object class, ManagedElement | ManagedElement |
| TS 32.622 [6], information object class, ManagedFunction | ManagedFunction |
| TS 32.632 [8], information object class, SgsnFunction | SgsnFunction |
| TS 32.622 [6], information object class, SubNetwork | SubNetwork |
| TS 32.622 [6], information object class, EP_RP | EP_RP |

## 6.2	Class diagram

### 6.2.1	Attributes and relationships

This clause depicts the set of IOCs that encapsulate information relevant for this service. This clause provides the overview of all information object classes in UML. Subsequent clauses provide more detailed specification of various aspects of these information object classes.

The figures below show the containment/naming hierarchy and the associations of the information object classes defined in the present document.

- NOTE:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios in all figures.

Each IOC is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [4] that expresses its containment hierarchy. As an example, the DN of an IOC representing a MME (3GPP TS 23.401 [9]) could have a format like:

- SubNetwork=China, MeContext =MEC-Gbg-1, ManagedElement =MME-Gbg-1, MMEFunction=MME-1.

- Figure 6.2.1.1: EPC NRM Containment/Naming Relationships

- Figure 6.2.1.2: EPC NRM Containment/Naming and Association_1

Figure 6.2.1.3: EPC NRM Containment/Naming and Association_2

- Figure 6.2.1.4: EPC NRM Containment/Naming and Association3

Figure 6.2.1.5 MME Pool Object Model of EPC NRM

1

SubNetwork

(from 32.622)

<<InformationObjectClass>>

QCISet

<<InformationObjectClass>>

0..1

NOTE:	QCISet shall be contained in the root SubNetwork instance.

- Figure 6.2.1.6: EPC NRM Containment/Naming and Association 3

Figure 6.2.1.7: EPC NRM Containment/Naming and Association 4

### 6.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

The figures below show the inheritance hierarchy for the EPC NRM.

Figure 6.2.2.1: EPC NRM Inheritance Hierarchy_1

Figure 6.2.2.2: EPC NRM Inheritance Hierarchy_2

EP_RP

(from 32.622)

<<InformationObjectClass>>

EP_RP_EPS

<<InformationObjectClass>>

Figure 6.2.2.3: EPC NRM Inheritance Hierarchy_3

ManagedFunction

(from 32.622)

<<InformationObjectClass>>

QCISet

<<InformationObjectClass>>

Figure 6.2.2.4: EPC NRM Inheritance Hierarchy_4

Figure 6.2.2.5: E-UTRAN NRM Inheritance Hierarchy_5

### 6.3	Information Object Class (IOC) definitions

### 6.3.1	EPDGFunction

#### 6.3.1.1	Definition

This IOC represents ePDG functionality. For more information about the ePDG, see 3GPP TS 23.402 [12].

### 6.3.2	MMEFunction

#### 6.3.2.1	Definition

This IOC represents MME functionality. For more information about the MME, see 3GPP TS 23.401 [9].

#### 6.3.2.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| pLMNIdList | M | M | - |
| mMEC | M | M | - |
| mMEPool | M | M | - |

#### 6.3.2.3	Attribute constraints

None.

### 6.3.3	PCRFFunction

#### 6.3.3.1	Definition

This IOC represents PCRF functionality. For more information about the PCRF, see 3GPP TS 23.401 [9].

### 6.3.4	PGWFunction

#### 6.3.4.1	Definition

This IOC represents PDN Gateway functionality. For more information about the PDN Gateway, see 3GPP TS 23.401 [9].

### 6.3.5	ServingGWFunction

#### 6.3.5.1	Definition

This IOC represents Serving Gateway functionality. For more information about the Serving Gateway, see 3GPP TS 23.401 [9].

#### 6.3.5.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| pLMNIdList | M | M | - |
| tACList | M | M | - |

### 6.3.6	MMEPool

#### 6.3.6.1	Definition

This IOC represents MME Pool. For more information about the MME Pool, see 3GPP TS 23.401 [9].Key concepts related to MME Pool are:

-	An MME Pool consists of one or more MME nodes.  A particular node can be a member of one and only one MME Pool.

-	One MME Pool serves at most one MME Pool Area.  One MME Pool Area can be served by at most one MME Pool.

#### 6.3.6.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| mMEGI | M | M | - |
| mMEPoolMemberList | M | M | M |
| mMEPoolArea | M | M | M |

#### 6.3.6.3	Attribute constraints

None.

### 6.3.7	MMEPoolArea

#### 6.3.7.1	Definition

This IOC represents MME Pool Area. For more information about the MME Pool Area, see 3GPP TS 23.401 [9]. Key concepts related to MME Pool Area are:

-	An MME Pool Area is defined as an area within which an UE may be served without the need to change the serving MME.  It is a collection of complete Tracking Areas (TAs).

-	A particular TA can be a member of one or more MME Pool Areas.  In the latter case, the MME Pool Areas involved are called “overlapping MME Pool Areas”.

#### 6.3.7.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| mMEPool | M | M | M |
| tACList | M | M | - |
| pLMNIdList | O | M | - |

#### 6.3.7.3	Attribute constraints

None.

### 6.3.8	Link_ENB_MME

#### 6.3.8.1	Definition

This IOC models the S1-MME reference point as defined in TS 23.401 [9].

### 6.3.9	Link_ENB_ServingGW

#### 6.3.9.1	Definition

This IOC models the S1-U reference point as defined in TS 23.401 [9].

### 6.3.10	Link_EPDG_PCRF

#### 6.3.10.1	Definition

This IOC models the Gxb reference point as defined in TS 23.402 [12].

### 6.3.11	Link_EPDG_PGW

#### 6.3.11.1	Definition

This IOC models the S2b reference point as defined in TS 23.402 [12].

### 6.3.12	Link_HSS_MME

#### 6.3.12.1	Definition

This IOC models the S6a reference point as defined in TS 23.401 [9].

### 6.3.13	Link_MME_MME

#### 6.3.13.1	Definition

This IOC models the S10 reference point as defined in TS 23.401 [9].

### 6.3.14	Link_MME_SGSN

#### 6.3.14.1	Definition

This IOC models the S3 reference point as defined in TS 23.401 [9].

### 6.3.15	Link_MME_ServingGW

#### 6.3.15.1	Definition

This IOC models the S11 reference point as defined in TS 23.401 [9].

### 6.3.16	Link_PCRF_ServingGW

#### 6.3.16.1	Definition

This IOC models the Gxc reference point as defined in TS 23.402 [12].

### 6.3.17	Link_PCRF_PGW

#### 6.3.17.1	Definition

This IOC models the Gx reference point as defined in TS 23.401 [9].

### 6.3.18	Link_PGW_ServingGW

#### 6.3.18.1	Definition

This IOC models the S5 reference point as defined in TS 23.401 [9].

### 6.3.19	Link_SGSN_ServingGW

#### 6.3.19.1	Definition

This IOC models the S4 reference point as defined in TS 23.401 [9].

### 6.3.20	EP_RP_EPS

#### 6.3.20.1	Definition

This IOC represents an end point of reference point in EPS as defined in TS 23.401 [9].

#### 6.3.20.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| farEndNeIpAddr | O | M | CM |

#### 6.3.20.3	Attribute constraints

| Name | Definition |
| --- | --- |
| farEndNeIpAddr’s write qualifier | When the EP_RP_EPS object belongs to the different Domain Manager as the NE pointed by the farEndNeIpAddr attribute, the Write Qualifier of farEndNeIpAddr attribute is needed. |

#### 6.3.20.4	Notifications

The common notifications defined in subclause 6.1.6 of 3GPP TS 32.622[6] are valid for this IOC, without exceptions or additions.

### 6.3.21	ExternalServingGWFunction

#### 6.3.21.1	Definition

This IOC represents SGW functionality controlled by another IRPAgent. For more information about the SGW, see 3GPP TS 23.401 [9].

#### 6.3.21.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| pLMNIdList | M | M | M |
| tACList | M | M | M |

### 6.3.22	ExternalMMEFunction

#### 6.3.22.1	Definition

This IOC represents MME functionality controlled by another IRPAgent. For more information about the MME, see 3GPP TS 23.401 [9].

#### 6.3.22.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| pLMNIdList | M | M | M |
| mMEC | M | M | M |
| mMEPool | M | M | M |

### 6.3.23	IOC QCISet

#### 6.3.23.1	Definition

This IOC represents a set of QCI as defined in section 6.1.7.2 of TS 23.203 [19].

#### 6.3.23.2	Attributes

| Attribute Name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |
| qCIList | M | M | M |

#### 6.3.23.3	Attribute constraints

Null.

#### 6.3.23.4	Notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |
| notifyObjectCreation | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |
| notifyObjectDeletion | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |

### 6.3.24	MBMSGWFunction

#### 6.3.24.1	Definition

This IOC represents MBMS GW functionality. For more information about the MBMS GW, see 3GPP TS 36.300 [16].

#### 6.3.24.2	Attributes

| Attribute name | Support Qualifier | Read Qualifier | Write Qualifier |
| --- | --- | --- | --- |
| id | M | M | - |

### 6.3.25	Link_MBMSGW_ENB

#### 6.3.25.1	Definition

This IOC represents the M1 reference point as defined in 3GPP TS 36.300 [16].

## 6.4	Information Relationship definitions

### 6.4.1	MMEPoolArea-MMEPoolRelation (M)

#### 6.4.1.1	Definition

This association represents a bi-directional relationship between the MMEPoolArea and MMEPool.

#### 6.4.1.2	Roles

| Name | Definition |
| --- | --- |
| mMEPool | This role represents the associated mMEpool of a MMEPoolArea-MMEPoolRelation. |
| mMEPoolArea | This role represents the associated mMEPoolArea of a MMEPoolArea-MMEPoolRelation. |

#### 6.4.1.3	Constraints

None

### 6.4.2	MMEPool-MMERelation (M)

#### 6.4.2.1	Definition

This association represents a bi-directional relationship between the MMEFunction and MMEPool.

#### 6.4.2.2	Roles

| Name | Definition |
| --- | --- |
| mMEPool | This role represents the associated mMEPool of a MMEPool-MMERelation. |
| mMEPoolMemberList | This role represents all the associated mMEFunctions of a MMEPool-MMERelation. |

#### 6.4.2.3	Constraints

Associations MMEPool-MMERelation and MMEPool-ExternalMMERelation are mutually exclusive.

### 6.4.3	MMEPool-ExternalMMERelation (M)

#### 6.4.3.1	Definition

This association represents a bi-directional relationship between the MMEPool and ExternalMMEFunciton.

#### 6.4.3.2	Roles

| Name | Definition |
| --- | --- |
| mMEPool | This role represents the associated mMEPool of a MMEPool-ExternalMMERelation. |
| mMEPoolMemberList | This role represents all the associated ExternalMMEFunctions of a MMEPool-ExternalMMERelation. |

#### 6.4.3.3	Constraints

Associations MMEPool-MMERelation and MMEPool-ExternalMMERelation are mutually exclusive.

## 6.5	Information Attribute definitions

### 6.5.1	Definition and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| farEndNeIpAddr | The IP address(s) of the far end network entity to which the reference point is related. The IP address can be either IPv4 or IPv6. |  |
| mMEC | MME Identifier (MMEI) is constructed from an MME Group ID (MMEGI) and an MME Code (MMEC). The MMEC is unique within the MME pool area and, if overlapping pool areas are in use, unique within the area of overlapping MME pools. (Ref. 3GPP TS 23.003[14]) |  |
| mMEGI | MME Identifier (MMEI) is constructed from an MME Group ID (MMEGI) and an MME Code (MMEC). The MMEGI is the unique identity of MME Pool within the context of PLMN. ( Ref.3GPP TS 23.003[14]). Note: An UE, supported by a cell, can connect to one out of a group of MMEs. The group consists of the MMEs supporting the tracking area for the cell limited to those that are connected to the serving eNB. The MME is identified by the combination PLMNID-MMEGI-MMEC. The combination is called GUMMEI. |  |
| mMEPool | It is the DN of a MMEPool instance. |  |
| mMEPoolArea | It is the DN of a MMEPoolArea instance. |  |
| mMEPoolMemberList | It is the list of DNs of MMEFunction instances of the MMEPool. |  |
| pLMNIdList | It is a list of PLMN-Id, PLMN-Id= Mobile Country Codes (MCC)¦¦ Mobile Network Codes(MNC) (Ref. 3GPP TS 23.003[14]) The MMEPoolArea.pLMNIdList purpose is to identify the PLMNs (related to MMEFunction) the MME Pool is serving. The MMEEunction.pLMNIdList purpose is as following. One operator may have several PLMN Ids and accordingly RAN broadcasts these Ids to enable UEs of different PLMN (i.e, UEs with different MNC in their IMSIs) to access its network. If CN node does not know this PLMN list, UEs of different PLMN than the one combined in MME might be treated as UEs from other operators. This will affect Location Update and Inter-MME handover procedures, and also the changing rate. |  |
| qCIList | It is a list of QCI characteristic, which is a structure including the element QCI, Resource_Type, Priority, Packet_Delay_Budget and Packet_Error_Loss_Rate; Wherein - QCI representing the number of the QCI, is an integer; - Resource_Type representing the Resource Type(GBR or Non-GBR) of the QC; - Priority representing the Priority of QCI, is an integer; - Packet_Delay_Budget representing the Packet Delay Budget of the QCI, is an integer with the unit of millisecond(ms); - Packet_Error_Loss_Rate representing the Packet Error Loss Rate of the QCI, is a real. ( Ref.3GPP TS 23.203[x]) | Refer to 3GPP TS 23.203[x] |
| tACList | It is the list of TAC of the MMEPoolArea that is used for traffic handling. Each TAC is provisioned over the S1 interface from the eNodeB, Ref 3GPP TS 36.413[18]. Note: A cell can only broadcast one TAC. See TS 36.300 v8.4.0 [16], section 10.1.7 (PLMNID and TAC relation). The Tracking Area Identity is constructed from the MCC (Mobile Country Code), MNC (Mobile Network Code) and TAC (Tracking Area Code). ( Ref.3GPP TS 23.401[9]) |  |

## 6.6	Common notifications

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [3]) |  |
| notifyAttributeValueChange | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |
| notifyObjectCreation | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |
| notifyObjectDeletion | See Kernel CM IRP (3GPP TS 32.662 [13]) |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Dec 2008 | SP-42 | SP-080752 |  |  | Presentation to SA for information |  | --- | 1.0.0 |
| Mar 2009 | SP-43 | SP-090071 | -- | -- | Presentation to SA for approval | -- | 2.0.0 | 8.0.0 |
| Jun 2009 | SP-44 | SP-090289 | 002 | -- | Add ExternalSGWFunction and attributes for SGWFunction (6.5.1 not deleted, due to editorial error in the CR). | F | 8.0.0 | 8.1.0 |
| Jun 2009 | SP-44 | SP-090289 | 001 | -- | Add ExternalMMEFunction and relations for MMEPools | F | 8.0.0 | 8.1.0 |
| Sep 2009 | SP-45 | SP-090534 | 003 | -- | Removing changes from S5-092073 | F | 8.1.0 | 8.2.0 |
| Sep 2009 | SP-45 | SP-090534 | 004 | -- | Removal of the Write Qualifier from the TACList attribute for the MMEPoolArea | F | 8.1.0 | 8.2.0 |
| Sep 2009 | SP-45 | SP-090627 | 005 | -- | Add QCI model | B | 8.2.0 | 9.0.0 |
| Dec 2009 | SP-46 | SP-090719 | 006 | -- | Correct the definition of IOC EP_RP_EPS | F | 9.0.0 | 9.1.0 |
| Mar 2010 | SP-47 | SP-100035 | 008 | -- | Removal of sgwAddress and sgsnAddress in MMEFunction | F | 9.1.0 | 9.2.0 |
| Mar 2010 | SP-47 | SP-100035 | 009 | -- | Clarification on the cardinality of the relationships MMEFunction-MMEpool and ExternalMMEFunction-MMEPool | F | 9.1.0 | 9.2.0 |
| Sep 2010 | SP-49 | SP-100488 | 012 | -- | Changing the name of SGWFunction IOC in order not to conflict with the SGWFunction IOC in TS 32.632 | F | 9.2.0 | 9.3.0 |
| Sep 2010 | SP-49 | SP-100489 | 010 | -- | Add missing IOC MBMSGWFunction | B | 9.3.0 | 10.0.0 |
| Sep 2010 | SP-49 | SP-100489 | 011 | -- | Add the associations for MME Pool Object Model and inheritance for ExternalMMEFunction | F | 9.3.0 | 10.0.0 |
| Dec 2010 | SP-50 | SP-100833 | 013 | -- | Add missing inheritance relationship of Link_MBMSGW_ENB | F | 10.0.0 | 10.1.0 |
| March 2012 | SP-55 | SP-120045 | 014 | -- | Correction of name of IOC SGWFunction | F | 10.1.0 | 10.2.0 |