# 3GPP TSG SA WG5 (Telecom Management) Meeting #89	S5-130914

# 27-31 May 2013;Sophia Antipolis (France)	revision of S5-130abc

# Source:	China Mobile

# Title:	Add attributes to PdgFunction in 28.602

# Document for:	Approval

Agenda Item:	6.4.1

1	Decision/action requested

SA5 is asked to discuss new added attributes and converged into TS 28.602.

2	References

[1]  3GPP TS 32.672: "Telecommunication Management; Configuration Management (CM); State Management Integration Reference Point (IRP): Information Service (IS)".

3	Rationale

This pCR is targeted to add additional attributes of PdgFunction to TS 28.602. A useful feature in PDG follows the attribute recommendation of SGSN and GGSN defined in TS 28.702, i.e.  proceduralStatus. It indicates the procedural status of the object instance. This attribute provides a subset of capabilities of procedural status defined in [1].

4	Detailed proposal

Text Proposal for TS 28.602

1st Modified Section

6.3	Information object class definitions

6.3.2	PdgFunction

6.3.2.1	Definition

This IOC represents PDG functionality. For more information about the PDG, see 3GPP TS 23.234[5].

6.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| proceduralStatus(Note) | O | - | - | - | M |
| Note: This proceduralStatus is not settable or readable via any Interface IRP except conveyed by notifyStateChange notifications. |  |  |  |  |  |

6.3.2.3	Attribute constraints

None.

6.3.2.4	Notifications

The common notifications defined in subclause 6.5 are valid for this IOC, without exceptions or additions.

End of 1st Modified Section