# 3GPP TSG SA WG5 (Telecom Management) Meeting #89	S5-130918

# 27-31 May 2013;Sophia Antipolis (France)	revision of S5-130abc

# Source:	China Mobile

# Title:	Add AAA Authentication measurements to TS 28.402

# Document for:	Approval

Agenda Item:	6.5.1

1	Decision/action requested

SA5 is asked to discuss the proposals and converged into TS 28.402

2	References

[1]  3GPP TS 23.402: Architecture enhancements for non-3GPP accesses

3	Rationale

TS 23.402[1] described the basic authentication process in WLAN interworking with EPC. It’s a important role in the system. This pCR is targeted to add AAA authentication mesurment to TS 28.402.

4	Detailed proposal

Text Proposal for TS 28.402

1st Modified Section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[2]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[3]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[4]	3GPP TS 23.402: " Architecture enhancements for non-3GPP accesses "

2nd Modified Section

4	Measurements related to ePDG

4.1	AAA Authentication measurements

The performance counters presented in this subclause are mainly intended to:

- monitor the AAA server performance;

- monitor the network connectivity.

These counters are associated to the AAA server, which is playing a important role in WLAN interworking with EPC environment, and is defined in TS 23.401 [4].

The figure below presents the message sequence during an AAA authentication procedure initiated by the PDG.

# Figure: AAA authentication procedure initiated by ePDG

5.6.1	Attempted Authentication Procedures, per W-APN

- This measurement provides the number of attempted authentication procedures per W-APN.

- CC.

- Transmission by the ePDG of an AAA authentication request message to the AAA server.

- A single integer value.

- AAA.AttAuthQuery.W-Apn.

- PGWFunction, per W-APN.

- Valid for packet switched traffic.

- Combined

5.6.2	Successful Authentication Procedures, per W-APN

  - This measurement provides the number of successful authentication procedures per W-APN.

  - CC.

  - Receipt by the ePDG of an AAA successful authentication response message from the AAA server.

  - A single integer value.

  - AAA.SuccAuthResp.W-Apn.

  - PGWFunction, per W-APN.

  - Valid for packet switched traffic.

  - Combined.

3rd Modified Section

Annex A:
Use cases for performance measurements definition

The present annex provides the concrete use cases for the interworking system performance measurements defined in clause 4.

A.1	Use case for AAA authentication measurements

WLAN Access Authorization is used in 3GPP AAA Server to verify whether WLAN Access should be allowed to a subscriber and deciding what access rules/policy should be applied to a subscriber. If the Authentication success rate is very low, user experience will be affected, such as user will wait for a long time or even be rejected to access the network. Therefore it is important to define attempted and successful authentication information performance measurements to calculate authentication information retrieval success rate. The failed authentication ratio could be calculated according to the attempted authentication and successful authentication. It’s useful to facilitate trouble shooting.