3GPP TSG-SA5 Meeting #97	S5-145168

# Venice, Italy October 20-24,  2014														revision of S5-144abc

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.591 | CR | 0005 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirements for energy saving management for HeNBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM12 |  |  |  |  |  |  | Date: |  | 2014-10-12 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Based on several 3GPP discussions (Study and work items) the benefits of energy savings in the network by powering down cells during periods of low demand are well understood. Reference: 3GPP TS 32.551 Energy Saving Management (ESM); Concepts and requirements and 3GPP, TR 32.826 Study on Energy Savings Management (ESM) Small cells are ideal candidates for policy based energy saving. For instance: at certain times of the day (10:00 PM to 7:00 AM) enterprises are usually closed and a policy based shutdown would be desired. For HeNBs currently there are no parameters defined to set the energy savings policy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new requirements on Type 1 interface to add capability to configure energy saving policy on HeNBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support energy saving policy on HeNBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.1.1, 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	TR-069 Amendment 2, CPE WAN Management Protocol v1.1, Broadband Forum.

[5]	3GPP TS 32.435: "Performance Measurement, eXtensible Markup Language (XML) file format definition".

[6]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Alarm Integration Reference Point (IRP): Information Service (IS) ".

[7]	3GPP TS 22.220: "Service requirements for Home Node B (HNB) and Home eNode B (HeNB)".

[8]				3GPP TS 33.320: " Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[9]				3GPP TS 32.551: " Energy Saving Management (ESM); Concepts and requirements ".

[10]					3GPP TR 32.826:  " Study on Energy Savings Management (ESM) ".

# 5	Business level requirements

## 5.1	Requirements

### 5.1.1	Configuration Management

REQ-OAMP_CM-CON-001 The HeNB shall be able to automatically, i.e. without human operator on-line interaction or attention, configure itself to be ready for service when powered up and connected to HeMS.

REQ-OAMP_CM-CON-002 The HeNB shall be able to automatically, i.e. without human operator on-line interaction or attention, configure itself to be in service when powered up and connected to HeMS.

REQ-OAMP_CM-CON-003 The HeNB shall be able to automatically, i.e. without human operator on-line interaction or attention, upgrade its software/firmware and configuration.

REQ-OAMP_CM-CON-004 The HeNB auto-configuration shall be done in such way that the performance of the surrounding macro cells is not adversely affected.

REQ-OAMP_CM-CON-005 The HeNB auto-configuration function should be adaptive to react to change in the network and changes in the radio environment.

REQ-OAMP_CM-CON-006 The operator shall be able to remotely reboot the HeNB.

REQ-OAMP_CM-CON-007 The operator shall be able to remotely start/stop the radio transmission of the HeNB.

REQ-OAMP_CM-CON-008 In case IPsec is used, the system should be engineered to ensure that the HeNB IP address changes as minimally as possible.

REQ-OAMP_CM-CON-009 The operator shall be able to remotely reconfigure the frequency and radio bandwidth of HeNB to adapt to changes in the radio environment and required bandwidth.

REQ-OAMP_CM-CON-010 The HeNB should allow configuration of the IPsec or non-IPsec usage option based on the operator’s policy [8].

REQ-OAMP_CM-CON-00x The operator shall be able to remotely configure energy saving options based on operator policy [9][10].

REQ-OAMP_CM-CON-010 The operator shall be able to enable and disable energy saving for a selected part of the network [9][10].

Next modification

# 6	Specification level requirements

## 6.1	Requirements

### 6.1.1	Configuration Management

The requirements for configuration management are as follows:

REQ-OAMP_CM-FUN-001 The HeNB configuration shall be administered by the HeMS utilising the TR-069 CWMP Protocol, reference [4].

REQ-OAMP_CM-FUN-002 HeMS shall be able to reboot the HeNB.

REQ-OAMP_CM-FUN-003 HeMS shall have remote access to the HeNB to start/stop the radio transmission.

REQ-OAMP_CM-FUN-004 HeMS shall have remote access to the HeNB to start/stop the radio transmission on the frequencies specified by HeMS.

REQ-OAMP_CM-FUN-005 HeMS shall maintain the configuration data of the HeNB.

REQ-OAMP_CM-FUN-006 When the HeNB is initially powered up and connected to the HeMS, HeMS shall send the initially needed configuration data to the HeNB.

REQ-OAMP_CM-FUN-007 If the inner IPsec tunnel IP address of the HeNB changes and HeNB is connected to HeMS via IPsec Tunnel then the HeNB shall notify the HeMS using TR-069.

REQ-OAMP_CM-FUN-008 The HeMS shall specify which parameters it needs to be notified of when the HeNB changes their values through auto-configuration. The HeNB shall notify the HeMS of changes in the values of any such auto-configured parameters.

REQ-OAMP_CM-FUN-009 The HeNB shall inform the HeMS of its ability to auto-configure parameters or groups of parameters that are relevant to the HeMS.

REQ-OAMP_CM-FUN-010 HeMS shall be able to specify a value, or a valid range of values, for any parameter that is auto-configurable by the HeNB.

REQ-OAMP_CM-FUN-011 Configuration management capability for the HeNB shall be supported by means of TR-069 RPCs SetParameterValues,AddObject and DeleteObject . Optionally a bulk configuration management file may be supported. In this case the TR-069 manager uses the RPC download method to trigger a CM file download from a file server.

REQ-OAMP_CM-FUN-012 The HeNB shall provide a capability allowing the HeMS to manage downloading of HeNB software/firmware image files and provide mechanisms for version identification and notification to the HeMS of the success or failure of a file download.

REQ-OAMP_CM-FUN-013 In normal operation, the HeNB shall maintain its configuration data following a HeNB reboot.

REQ-OAMP_CM-FUN-014 The HeNB shall support capabilities to inform the HeMS about the results of specific actions triggered by the HeMS.

REQ-OAMP_CM-FUN-015 It shall be possible to initiate a management connection at the request of either the HeNB or the HeMS.

REQ-OAMP_CM-FUN-016 The HeNB shall be able to inform the HeMS of the changes in radio environment and required radio bandwidth.

REQ-OAMP_CM-FUN-017 The operator shall be able to remotely reconfigure the frequency and radio bandwidth of HeNB.

REQ-OAMP_CM-FUN-018 If the HeNB is unable to offer service for an operator configured time period it will deactivate the air interface.

REQ-OAMP_CM-FUN-019 The HeNB shall support a capability to allow the HeMS to activate Local IP Access.

REQ-OAMP_CM-FUN-020 The HeNB shall support a capability to allow the HeMS to deactivate Local IP Access.

REQ-OAMP_CM-FUN-021 The HeMS should be able to securely configure the HeNB according to the operator's policy, whether or not to use IPsec for subsequent connections.

REQ-OAMP_CM-CON-0xx The HeMS shall be able to enable and disable energy saving for a selected part of the network.

REQ-OAMP_CM-CON-0xx The HeNB shall support a capability to notify the HeMS when it goes into or out of energySaving state.

REQ-OAMP_CM-CON-0xx The HeNB shall support capability allowing the HeMS to configure the time period during which energy saving is allowed.

REQ-OAMP_CM-CON-0xx  The HeNB shall support capability allowing the HeMS to be able to remotely configure energy saving operator policy.

End of modifications