# Multi-SDO Project on Converged Management PM Interface definitions

# Document number:	S5eCPM0116

# Source:	Bernd Zeuner, Deutsche Telekom

# Title:		PM2: Metadefinitions for Measurements and KPIs V0.9 (9 Oct. 2014)

# Meeting date/time:	#28; Friday 31st October 2014

Introduction

This contribution provides version 0.9 of PM2.
Version 0.8 is contained in S5eCPM0107.

Changes made during Okt. 9th conference call (#27) are marked in this colour.

Fixed Mobile Convergence (FMC)

Converged Performance Management

PM2: Metadefinitions for Measurements and KPIs

Version 0.9 (draft)

Source: M-SDO Converged PM project

Editor: Christian Toche (Huawei Technologies)

Table of Contents

1	Scope	4

2	References	4

3	Definitions and abbreviations	4

3.1	Definitions	4

3.2	Abbreviations	4

4	Requirements	5

5	Definition of Performance Measurement Metadata	5

Annex A (Informative): Counters/KPIs properties	11

A1	3GPP Performance Metadata	11

A2	TM Forum Performance Metadata	11

A3	Mapping of 3GPP and TM Forum Performance Metadata	13

A4	Proposed JWG Measurement Profile	21

# 1	Scope

This document defines the metadata for Performance Management (PM) performance measurements (KPIs/counters). The definition would enforce consistency in description/definition of all standardized performance measurements (counters/KPIs) by various SDOs and organizations (or Multi-SDO) involved in the Management of Converged Networks [1,2]. This consistency would facilitate encoding and decoding of performance measurements (counters/KPIs) exchanged (via management protocols) and/or stored in (database or files).

The metadata related to PM operations/notifications is not part of this PM measurement metadata.

The metadata would be published as part of the Model Repertoire [3]. TBD.

# 2	References

- 3GPP TR 32.833 Study on Management of Converged Networks

- TM Forum TR 166 Information Model Federation Concepts and Principles (http://collab.tmforum.org/sf/go/doc13634?nav=1)

- Fixed Mobile Convergence (FMC) Model Repertoire V5.x

- IETF RFC 2578 Structure of Management Information Version 2 (SMIv2)

- 3GPP TS 32.401 V11.0.0 (2012-09) Performance Management (PM); Concept and requirements

# 3	Definitions and abbreviations

For the purposes of this document, the following definitions, symbols and abbreviations apply. For definitions, symbols and abbreviations not found here. See also Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) [3].

## 3.1	Definitions

Indicator/Measurement: tba

## 3.2	Abbreviations

ATM	Asynchronous Transfer Mode

ATML	?

ATT	?

CC	Cumulative Counter

DER	Discrete Event Register

FNIM	Federated Network Information Model

FMC	Fixed Mobile Convergence

G	?

GSM	Global System for Mobile

HHO	?

HO	?

ID	Identifier

IETF	Internet Engineering Task Force

IP	Internet Protocol

JWG	Joint Working Group

KPI	Key Performance Indicator

MIB	Management Information Base

MM	?

OS	?

PM	Performance Management

RAB	?

RL	?

RLM	?

RRC	?

SAEB	?

SDO	Standards Development Organisations (e.g., 3GPP, TM Forum)

Sgsn	?

SI		Status Inspection

SMS	?

TBA	To Be Added

TBC	To Be Checked

TBD	To Be Defined

U	?

UMTS	Universal Mobile Telecommunications System

UTRAN	UMTS Terrestrial Radio Access Network

VS	?

# 4	Requirements

Metadata is defined as data providing information about aspects of other data. Metadata can be exchanged in protocols, and can be stored and managed in a database, often called a Metadata registry or Metadata repository. Metadata can be defined in machine-readable syntax.

The PM performance measurement metadata is the properties of the performance measurements (counters/KPIs) including their associations to other performance measurements (counters/KPIs). The number of properties and the level of the details in the property definition depend on the metadata intended usage.

The purpose for defining the PM performance measurement metadata is to enforce a consistency in description/definition of all standardized counters/KPIs. This consistency facilitates encoding, decoding, interpreting and in general, processing of counters/KPIs defined by Multi-SDOs involved in the Management of Converged Networks [1, 2].

# 5	Definition of Performance Measurement Metadata

The following table provides the PM performance measurement metadata; i.e., the definition of the properties.

When defining a new measurement and when this new measurement is similar to a measurement already defined in another standard, then this standard should be referenced and only the missing properties should be defined in addition.

| Converged Property name | Description | Legal values |
| --- | --- | --- |
| name e.g, ErroredSeconds | Upper Camel Case. | String |
| GEN::documentation | This property contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | String |
| Counter::collectionMethodType | This property indicates the form in which this measurement data is obtained. COUNTER This literal defines an increasing counter. “The NE maintains a running count of the event being counted. The counter is reset to a well-defined value (usually "0") at the beginning of each granularity period.” See 3GPP 32.401 [5] where it is called Cumulative counter (CC). CUMULATIVE ## This literal defines an increasing counter. “The NE maintains a running count of the event being counted. The value of the counter monotonically increases until it reaches a maximum possible value. Then, it wraps around and starts increasing again from zero. Discontinuities in the monotonically increasing value normally occur at re- initialization of the management system.” See IETF RFC 2578 [4] where it is called Counter32/Counter64. GAUGE This literal defines a dynamic variable; used when data being measured can vary up or down during the period of measurement. “If a gauge is required to produce a consecutive readout over multiple granularity periods (e.g. cabinet temperature), then it shall only be reinitialised at the start of a recording interval.” See 3GPP 32.401 [5]. DISCRETE_EVENT This literal defines when data related to a particular event are captured every nth event is registered, where n ≥ 1. “Data related to a particular event is captured. Every nth event is registered, where n can be 1 or larger. The value of n is dependent on the frequency of occurrence of the event being measured. DER measurements shall be reset at the beginning of each granularity period and will only have a valid result at the end of the granularity period.” See 3GPP 32.401 [5] where it is called Discrete Event Registration (DER). STATUS_INSPECTION This literal defines a mechanism for high frequency samplings of internal counters at predefined rates; the output is an array of values. “Network elements maintain internal counts for resource management purposes. These counts are read at a predetermined rate, the rate is usually based upon the expected rate of change of the count value. Status inspection measurements shall be reset at the beginning of the granularity period and will only have a valid result at the end of the granularity period.” See 3GPP 32.401 [5]. VECTOR A set of ordered values, signifying certain measurement results. The number of values is fixed, and each value is uniquely identifiable. The order is known to the producer and consumer. LIST A bag of values in no specific order. The specified measurements are independent discrete values. | COUNTER, CUMULATIVE, GAUGE, DISCRETE_EVENT (*), STATUS_INSPECTION (*) VECTOR LIST (*) – Used for backward compatibility |
| unit | This property defines the unit by which the measurement is measured. For example: seconds, Celsius, Kelvin, Ampere, Volt, Watt, dB, kBit/s, rate per second, etc. | String |
| condition | This property indicates the condition which causes the measurement result data to be updated. This will be defined for example by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. | String |
| sourceStandard | This property indentifies the referenced standard. All the other properties (e.g., collectionMethodType) of this measurement must refer to the referenced standard if defined there. In case the definition of this measurement conflicts with the referenced definition, the referenced definition takes precedence. | String |
| measurementFamily | This property identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose. Note: when defining a new family name, consult the family names defined by other SDOs so that all the names are unique. It is not allowed to define a new family with the same semantics of an already existing family. | String |
| measurementName | This property captures the SDO agnostic measurement name (same name among several SDOs). The name does not carry the measurement family name. | String |
| sdoAliasNameList | This property captures the list of SDO specific measurement names. The measurement name convention is SDO specific. | String |
| measurementDataType | This property identifies the type of the value of the measurement, such as numeric, float. |  |
| monitoredObjectClassList | This property indicates the list of <<InformationObjectClass>> names to which this measurement is related. | Any defined <<InformationObjectClass>>, e.g. UtranCell, SgsnFunction. |
| KPI::formulaDescription | This property contains a textual description of the KPI::formulaSpecification. | String |
| KPI::formulaSpecification | This property contains the formula. | String |
| Counter::gaugeFormula | This property indicates the formula of a “GAUGE” collection method type measurement (Counter::collectionMethodType). | SUM, COUNT, MIN, MAX, AVERAGE, FIRST, LAST, PRODUCT, STDEV.S, STDEV.P, VAR.S, VAR.P, MEDIAN, MODE.SNGL, LARGE, SMALL, PERCENTILE.INC, QUARTILE.INC, PERCENTILE.EXC, QUARTILE.EXC OTHER |
| Counter::otherGaugeFormula | This property indicates the formula of a “GAUGE” collection method type measurement (Counter::collectionMethodType) with the value “OTHER”. This property is only relevant if Counter::gaugeFormula has the value “OTHER”. | String |
| ## Counter::samplingRate | This property indicates the sampling rate of a “GAUGE” collection method type measurement in the case when the measurement is generated by sampling. In this case the calculation of the gauge considers every Nth event where N>1. | Number(?) DER? |
| _indicator/measurementGroup | This property allows to group measurements (i.e., record concept). | tbd |

# Annex A (Informative): Counters/KPIs properties

This annex lists the 3GPP currently defined properties for its performance measurements and the TM Forum defined properties for its “indicator” (model for both counters and KPIs).

## A1	3GPP Performance Metadata

3GPP TS 32.40X series is using the following properties for the Measurement Definition Template to describe the counters that are passed over the northbound interface:

- Measurement Name

- Description

- Collection Method

- Condition (when/how is it generated)

- Measurement Value + Unit

- Measurement Type (short name)

- Measurement Object Instance

- Switching Technology

- Radio Technology (?)

The following properties are used for KPIs:

- Long Name

- Description

- Logical Formula Definition

- Physical Formula Definition

- Measurement names used for the KPI

- KPI Object

- KPI category

- Unit of the KPI

- Type of the KPI (Mean, Ratio, Cum)

- Remark

## A2	TM Forum Performance Metadata

TM Forum defines a single entity “Indicator” for modelling both counters and KPIs. Following are the “Indicator Specification” attributes (A SID Metadata entity):

- ID

- Name

- Description

- Perspective

- IndicatorCategory

- Indicator Type

- derivarionAlgorithm

- derivationMethod

- CollectionType

- IndicatorUnit

An Indicator Specification can be attached to:

- A monitored object

- A counter group

- A Counter category

- A Production/Collection job

TIP PM Interface is exposing this data over the interface. A PM consuming application can request the information using the following operations:

- getSupportedObjectClasses

- getSupportedObjects

- getSupportedIndicatorGroups

- getSupportedIndicators

- getSupportedTransport

- getSupportedFileFormat

- getCommonIndicatorGroupsGranularities

- getCommonIndicatorsGranularities

## A3	Mapping of 3GPP and TM Forum Performance Metadata

This section list all 3GPP and TM Forum defined performance metadata and maps them where possible.

The grey columns show the proposed converged performance metadata.

| 3GPP Property name | TMF Property name | Converged Property name | Description | Legal values |
| --- | --- | --- | --- | --- |
| name | name | name e.g, ErroredSeconds | Upper Camel Case. | String |
| description | description | documentation | This property contains a textual description of this modelled element. Should refer (to enable traceability) to a specific requirement. | String |
| collectionMethod | collectionType | collectionMethodType | This property indicates the form in which this measurement data is obtained. CUMULATIVE This literal defines an increasing counter. TBD GAUGE This literal defines a dynamic variable; used when data being measured can vary up or down during the period of measurement. TBD DISCRETE_EVENT This literal defines when data related to a particular event are captured every nth event is registered, where n ≥ 1. STATUS_INSPECTION This literal defines a mechanism for high frequency samplings of internal counters at predefined rates; the output is an array of values. DELTA This literal defines a ?? Editor’s note: Specify the moment when the value is to reset; not applicable to all values. | CUMULATIVE, GAUGE, DISCRETE_EVENT, STATUS_INSPECTION, DELTA |
|  | indicatorUnit | unit | This property defines the unit by which the measurement is measured. For example: seconds, KBs, rate per second, etc. Editor’s note: Should try to find a comprehensive list of units and allow the possibility to enhance it. | String |
| condition |  | condition? | This property indicates the condition which causes the measurement result data to be updated. TBC If possible, refer to an already existing standard whose property condition is exactly the same as here. | Any |
|  |  | sourceStandard | This property indentifies the referenced standard. All the other properties (e.g., collectionMethodType) of this measurement must refer to the referenced standard if defined there. Whenever possible, refer here to an already existing standard which defines the measurement in detail. In case the definition of this measurement conflicts with the referenced definition, the referenced definition takes precedence. <<<<<<<Stopped here after 21st January conference call >>>>>> |  |
|  | perspective | perspective? | The point of view for the measurement, such as a single user instance or an aggregation. May be a KPI property only. | String |
| measurementType |  | measurementFamily? | This contains a short form of the measurement name specified in the header, which is used to identify the measurement type in the result files. The measurement names are dotted sequences of items. The sequence of elements identifying a measurement is organised from the general to the particular. - The first item This property identifies the measurement family (e.g. HHO, RAB, SMS). Note that this family may also be used for measurement administration purpose. | (Edwin: I will move some examples from the 2nd column to here.) HHO RAB SMS |
|  |  | measurementKind? | - The second item identifies the name of the measurement itself, for which the following rules shall apply: - The second item of the measurement name can be divided into <Operation>, <Reason/Result> and <Direction> (and in that order) - Examples of Operation can be Establishment, Release, and Modification - Examples of Reason/Result can be Attempt, Failure, Success, Throughput and Volume - Examples of Direction can be Incoming, Outgoing, Uplink and Downlink | Operation - Establishment - Release - Modification Reason/Result - Attempt - Failure - Success - Throughput - Volume Direction - Incoming - Outgoing - Uplink - Downlink |
|  |  | measurementSubitem? | 3) Depending on the measurement type, additional items may be present to specify subcounters (failure causes, traffic classes, min, max, avg, G, U ...). In case of multiple additional items, they are also represented as a dotted sequence of items. When available, the template will describe to which standard it is referring to for these additional items (e.g. cause, traffic class). Otherwise, the additional item semantics must be described in details in the present document. Standardised causes will be a number. This number shall be derived according to which of the following rules applies: 4) For the standardised causes with numeric values explicitly specified in the reference spefication, the subcounter name will be the number assigned to this cause in this reference specification. 5) For the standardised causes without numeric values explicitly specified in the reference spefication, but where the causes are identified, the subcounter name shall be an number from 1 to n mapped in an incremental sequence to each of the specified causes following top down sequence in the order they are identified in the reference specification (e.g. RRC.ConnEstab.1, 1 idenfies the establishment cause “Originating Conversational Call”, see TS 25.331); 6) For the standardised causes without numeric values explicitly specified in the reference spefication and the causes identified and the causes have been divided into different cause groups, the subcounter should be defined as the form of ‘Cause group’.Cause, where: - the subcounter name of ‘Cause group’ shall be an incremental number from 1 to n to identify each cause group specified in a top down sequence following the order they are identified in the reference specification, - the subcounter name of cause within this cause group shall be an incremental number from 1 to n to identify each cause within the group specified in a top down sequence following the order they are identified within the cause group in the reference specification (e.g. RLM.FailRLSetupIub.2.1, 2.1 indenfies the cause “Transport resource unavailable” in the cause group “Transport Layer Cause”, see TS 25.433). Subcounter ‘Cause group’.sum is permitted to indentify the aggregate of measurement values of all the causes within the cause group (e.g. RLM.FailRLSetupIub.1.sum, 1.sum identifies the aggregate of all the causes within the cause group “Radio Network Layer Cause”, see TS 25.433). The non standardised causes should be a string (e.g. RRC.ConnEstab.NoReply). The set of values issued for a measurement does not depend on the associated collection method (CC, SI, Gauge, DER). For instance, a gauge collected counter does not necessarily provide min, max, average values. The vendor-specific UMTS and combined GSM/UMTS measurement names will all begin with the VS prefix. In addition, it is recommended that a prefix is added for non-UMTS measurements: - Q3 for Q3 measurements; - MIB for IETF measurements (ATM, IP); - OS for other standards measurements. The 3GPP standardised measurements name must not commence with the above prefixes. Examples of valid measurement names are: - HO.PrepAtt - SAEB.EstabSucc - RRC.ConnEstabAtt.Cause where Cause identifies the failure cause. Examples of valid measurement names for measurements defined before version 8.3.0 of the present document are: - VS.HO.InterSGSNReject.NoResource; - ATM. ATML.IngressCells; - HHO.SuccOutIntraCell; - MM.AttachedSubs.Max; - RAB.EstabAttCS.Conversational; - RRC.ConnEstab.Cause where Cause identifies the failure cause. Abbreviations to be used within measurement types can be found in subclause 3.2 of the present document. | minimum maximum average G? U? |
|  | indicatorType | type | This property identifies the type of the value of the measurement, such as numeric, float. |  |
| measurement ObjectClass | Relationship to monitored objects | monitoredObjectClassList | This property indicates the list of <<InformationObjectClass>> names to which this measurement is related. | Any defined <<InformationObjectClass>>, e.g. UtranCell, SgsnFunction. |
| switchingTechnology | indicatorCategory | technologyList | This property indicates the technology domain(s) that this measurement is applicable to. | Any (Circuit Switched, Packet Switched, Circuite Switch and Packet Switch). |
| generation |  | generation? |  |  |
| purpose |  | purpose? |  |  |
| logicalFormulaDefinition physicalFormulaDefinition | derivationAlgorithm | algorithm | This property identifies a step-by-step procedure used to calculate the value of the measurement. If case a …. If case b … If case c … … | String |
|  | derivationMethod | derivationMethod | For simple calculations, the method used to calculate the value of a measurement, such as average, minimum, maximum, sum, and so forth. | String |

The Indicator/Measurement needs to be related to other object classes in the model. Note that this information may not be seen as indicator/measurement properties.

| 3GPP Property name | TMF Property name | Converged Property name | Description | Legal values |
| --- | --- | --- | --- | --- |
|  | Relationship to monitored object class | _monitoredObject | This property … | «InformationObjectClass» MonitoredObject passedById (same as DN) |
|  | Relationship to indicator/measurement group | _indicator/measurementGroup | This property allows to group measurements (i.e., record concept). | tbd |
|  | Relationship to indicator/measurement category | _indicator/measurementCategory | This property allows to group measurements (i.e., family concept). <<<<<Stopped here after 13th February f2f meeting Madrid >>>> | «InformationObjectClass» Indicator/MeasurementCategory passedById (same as DN) |
|  | Relationship to production job | _productionJob | This property … | «InformationObjectClass» ProductionJob passedById (same as DN) |
|  | Relationship to collection job | _collectionJob | This property … | «InformationObjectClass» CollectionJob passedById (same as DN) |

## A4	Proposed JWG Measurement Profile

The converged metadata defined in the previous section are defined in a UML profile to use them as templates for defining measurement instances.

Figure 1: JWG Measurement Profile

The profile is associated to the Class metaclass on a case by case basis. This allows a model designer to define individual measurement instances. See an example below:

Figure 2: Example JWG Measurement Definition

3GPP is a trade mark of ETSI