3GPP TSG-SA5 Meeting #98	S5-146134

# San Francisco, USA November 17-21,  2014										revision of S5-144abc

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.592 | CR | 0032 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration parameters for energy saving management for HeNBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM13 |  |  |  |  |  |  | Date: |  | 2014-11-2 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | For HeNBs currently there are no parameters defined to set the energy savings policy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new configuration parameters on Type 1 interface to add capability to configure energy saving policy on HeNBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support energy saving policy on HeNBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR .. |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  | TS/TR ....CR ... S5-146132 |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[3]	3GPP TS 32.102: "Telecommunication management; Architecture".

[4]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[5]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[6]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[7]	TR-069 Amendment 2, HeNB WAN Management Protocol v1.1, Broadband Forum

[8]	3GPP TR 25.820: "3G Home NodeB Study Item Technical Report".

[9]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[10]	3GPP TS 25.401: " UTRAN Overall Description".

[11]	3GPP TS 25.433: "UTRAN Iub interface Node B Application Part (NBAP) signalling".

[12]	TR-106, “Data Model Template for TR-069-Enabled Devices”, Broadband Forum, 2009, http://broadband-forum.org/technical/download/TR-106_Amendment-2.pdf.

[13]	TR-196i2, "Femto Access Point Device Data Model " Broadband Forum, Issue 2 November 2011  http://www.broadband-forum.org/technical/download/TR-196_Issue-2.pdf. [14]	3GPP TS 32.432 "Telecommunication management; Performance measurement: File format definition".

[15]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[16]	3GPP TS 32.111-6: "Telecommunication management; Fault Management; Part 6: Alarm Integration Reference Point (IRP): Solution Sets (SS)".

[17]	IETF RFC 3280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile",  April 2002, http://www.ietf.org/rfc/rfc3280.txt.

[18]	IETF RFC 4301: "Security Architecture for the Internet Protocol", , December 2005, http://www.ietf.org/rfc/rfc4301.txt.

[19]	IETF RFC 4307: "Cryptographic Algorithms for Use in the Internet Key Exchange Version 2 (IKEv2)",  December 2005, http://www.ietf.org/rfc/rfc4307.txt

[20]	IETF RFC 4960: "Stream Control Transmission Protocol",  September 2007, http://www.ietf.org/rfc/rfc4960.txt.

[21]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications",  July 2003, http://www.ietf.org/rfc/rfc3550.txt.

[22]	IETF RFC 3873: "Stream Control Transmission Protocol (SCTP) Management Information Base (MIB"), , September 2004, http://www.ietf.org/rfc/rfc3873.txt.

[23]	ITU-T RecommendationE.118: "The international telecommunication charge card ", International Telecommunication Union, May 2006, http://www.itu.int/rec/T-REC-E.118/en.

[24]	3GPP TS 25.304: "User Equipment (UE) procedures in idle mode and procedures for cell reselection in connected mode".

[25]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[26]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[27]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer - Measurements".

[28]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[31]	3GPP TS 36.322: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification".

[32]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[33]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[34]	3GPP TS 45.008: "Radio subsystem link control".

[35]	3GPP TS 23.107:"Quality of Service (QoS) concept and architecture".

[36]	3GPP TS 22.220: "Service Requirements for Home Node B (HNB) and Home eNode B (HeNB)".

[37]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB)".

[38]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[39]	3GPP TS 32.522: " Self-Organizing Networks (SON) Policy Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS )".

# 6	Information Model Definition

### 6.1.x	Energy Savings Policy related parameters

Table below shows parameters specific to energy savings management [39]

| Parameter Name | Description | Valid Values | TDD/FDD | Applicable to HNB |
| --- | --- | --- | --- | --- |
| energySavingsEnable | This attribute determines whether the energy saving function is enabled or disabled. | ENUMERATED {True, False} This parameter is settable by operator. | TDD/FDD | No |
| energySavingsState | Specifies the status regarding the energy saving in the cell. This parameter is applicable when esEnable is True. | ENUMERATED {isEnegySaving, isNotEnergySaving} | TDD/FDD | No |
| esActivationCellOriginalLoadParameters | This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow a cell to enter the energySaving state. The time duration indicates how long the load needs to have been below the threshold. | Threshold: Integer 0..100 (Percentage of PRB usage, see 3GPP TS 36.314 [13]) TimeDuration: Integer (in unit of seconds) This parameter is settable by operator. | TDD/FDD | No |
| esActivationCandidateCellsLoadParameters | This attributes is relevant if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration, which are used by distributed ES algorithms to allow an ‘original’ cell to enter the energySaving state. Threshold and duration are applied to the candidate cell(s) which provide coverage backup of an original cell when it is in the energySaving state. The threshold applies in the same way for a candidate cell no matter which original cell it will provide backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been below the threshold before any original cells which will be provided backup coverage by the candidate cell enters energy saving state. | Threshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) This parameter is settable by operator. | TDD/FDD | No |
| esDeactivationCandidateCellsLoadParameters | This attributes is relevant, if the cell acts as a candidate cell. This attribute indicates the traffic load threshold and the time duration which is used by distributed ES algorithms to allow a cell to leave the energySaving state. Threshold and time duration are applied to the candidate cell when it provides coverage backup for the cell in energySaving state. The threshold applies in the same way for a candidate cell, no matter for which original cell it provides backup coverage. The time duration indicates how long the traffic in the candidate cell needs to have been above the threshold to wake up one or more original cells which have been provided backup coverage by the candidate cell. | allowedValues: Threshold: Integer 0..100 (Percentage of PRB usage (see 3GPP TS 36.314 [13]) ) TimeDuration: Integer (in unit of seconds) This parameter is settable by operator. | TDD/FDD | No |
| esNotAllowedTimePeriod | This attribute can be used to prevent a cell entering energySaving state. This attribute indicates a list of time periods during which energy saving is not allowed. Time period is valid on the specified day and time of every week. | The legal values are as follows: startTime and endTime: All values that indicate valid UTC time. endTime should be later than startTime. periodOfDay: structure of startTime and endTime. daysOfWeekList: list of weekday. weekday: Monday, Tuesday, … Sunday. List of time periods: {{ daysOfWeek daysOfWeekList, periodOfDay dailyPeriod}} This parameter is settable by operator. | TDD/FDD | No |

End of modifications