# 3GPP TSG SA WG5 (Telecom Management) Meeting #99	S5-151130

# 2-6 February 2015Taipei, Taiwan	revision of S5-140abc

# Source:	Alcatel-Lucent

# Title:	Introduce MBSFN MDT use case

# Document for:	Approval

Agenda Item:	6.4.4

# 1	Decision/action requested

Discussion and agreement to introduce use case as informative text in TS 32.421

# 2	References

[1] 3GPP TS 36.300 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); overall description; Stage 2

[2] 3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description, Stage 2".

# 3	Rationale

In LS on Multi-Broadcast Single Frequency Network (MBSFN) MDT (S5-144096 / R2-142873) from RAN2, SA5 has been requested to take into account work and agreements in the 3GPP RAN groups for collecting measurements for MBMS.

Refer discussion paper S5-151128 for discussion on MBSFN MDT.

Some use cases have been

# 4	Detailed proposal

Annex A (informative):
Trace use cases

# A.14	Use case #14 Collection of MBSFN measurements from the UEs

## A.14.1	Description

The operator needs information for analysing signal strength, signal quality and block error rates for MBSFN reception. MBSFN MDT data is also needed to support network verification, re-planning of MBSFN areas, and optimization of MBSFN operation parameters.

In addition if a user complains about their MBMS service, signalling based MDT for that particular user can be triggered to analyse and troubleshoot the issue.

## A.14.2	Example of required data to cover use case #14

The MBSFN MDT measurmernts are defined in TS 37.320 [11] is required for analysing MBMS QoS in the indicated area and for trouble shooting per user MBMS service. The measurement quantities for E-UTRA MBSFN measurement logging are fixed and consist of MBSFN RSRP, MBSFN RSRQ, BLER for signalling and BLER for data per MCH, in addition to the measurement quantities for downlink pilot strength measurements.

The MBSFN measurement results consist of, per MBSFN area where MBMS service is received:

-	MBSFN area identity

-	Carrier frequency

- 	MBSFN RSRP

-	MBSFN RSRQ

-	MCH BLER for signalling

-	MCH BLER for data, and related MCH index