3GPP TSG-SA WG6 Meeting #52-bis-e	S6-22xxxx

eMeeting, 16th – 20th January 2023	(revision of S6-22xxxx)

Source:	BDBOS

Title:	Generic ACMX procedure

Spec:	3GPP TR 23.700-38 version 0.5.0

Agenda item:	9.2

Document for:	Approval

Contact:	Dania.Azem@bdbos.bmi.bund.de

1. Introduction

This pCR defines a solution for generic ACMX procedure.

2. Reason for Change

This solution provides a general description of the generic ACMX procedure and the related information flows used in other solutions. This procedure provides the detailed description of the optional steps that are not included in other solutions for simplicity. This solution is not related to any specific key issue.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-38 v0.5.0.

* * * First Change * * * *

- 7	Solutions

- 7.1	Generic ACMX procedure

- 7.1.1	Solution description

- 7.1.1.1	General

This clause provides a description of the generic ACMX procedure and the related information flows used in other solutions. This procedure provides the detailed description of the optional steps that are not included in other solutions for simplicity.

This solution is not related to any specific key issue.

- 7.1.1.2	Information flows

##### 7.1.1.2.1	Process pending indication

Table 7.x.1.2.1-1 describes the information flow of the Process pending indication sent from a partner ACMS to the primary ACMS and the primary ACMC.

Table 7.x.1.2.1-1: Process pending indication

| Information element | Status | Description |
| --- | --- | --- |
| ACMX indicator | M | Identifies the ACMX pending request |

##### 7.1.1.2.2	Pending ACMX request notification

Table 7.x.1.2.2-1 describes the information flow of the pending ACMX request notification from a partner ACMS to the partner ACMC.

Table 7.x.1.2.2-1: Pending ACMX request notification

| Information element | Status | Description |
| --- | --- | --- |
| ACMX request indicator | M | An indicator of one or more pending ACMX requests |
| category | O | Notification identifiers, e.g. high, medium, low for alerting partner MC system of specific urgencies. |

##### 7.1.1.2.3	Pending ACMX response notification

Table 7.x.1.2.3-1 describes the information flow of the pending ACMX response notification from a primary ACMS to the primary ACMC.

Table 7.x.1.2.3-1: Pending ACMX response notification

| Information element | Status | Description |
| --- | --- | --- |
| ACMX response indicator | M | An indicator of one or more pending ACMX responses |

##### 7.1.1.2.4	Get pending request/response

Table 7.x.1.2.4-1 describes the information flow of Get pending request/response from a ACMC to the ACMS.

Table 7.x.1.2.4-1: Get pending ACMX request/response

| Information element | Status | Description |
| --- | --- | --- |
| ACMX indicator | M | Identifies ACMX pending requests/response |

##### 7.1.1.2.5	ACMX request/response

This information element is defined under the individual solutions.

- 7.7.1.3	Procedure

The procedure for generic ACMX transaction between MC systems is shown in figure 7.1.1.3-1.

Pre-conditions

-	The primary and partner MC system are configured to accept connections from relevant ACMCs via the respective ACMSs in each of the connected MC systems and have been configured and authorized successfully to allow exchange of administrative configuration information

-	The relevant ACMC and ACMS connection authorizations have been established successfully

- Figure 7.1.1.3-1: Generic ACMX Procedure

1.	The primary ACMC sends ACMX request to the primary ACMS.

2.	The primary ACMS performs an authorization check to verify that the MC service user is authorized to perform this action. A successful authorization check results in the ACMX request being forwarded to the relevant partner ACMS. Step 3 is not followed if the authorization was unsuccessful.

3.	The primary ACMS sends the ACMX request to the partner ACMS.

4. 	The partner ACMS stores the incoming request.

5.	The partner ACMS send process pending indication (clause 7.1.1.2.1) to the primary MC system.

NOTE 1:	There may be a considerable pause between step 4 and 7.

6.	Based on local policies and configurations, the partner ACMS may automatically handle the received request and send an ACMX response to the primary ACMS, or may require request verification by the ACMC as described in steps 8 to 12.

7. The authorized MC service user of partner MC system logs on to the partner ACMC.

8.	The partner ACMS notifies the ACMC of the pending request (clause 7.1.1.2.2).

9.	The partner ACMC queries the partner ACMS for pending requests (clause 7.1.1.2.4).

10.	The partner ACMS forwards the stored ACMX request to the partner ACMC.

11.	The authorized MC service user of partner MC system checks the content of the ACMX request and decides whether to approve it or not.

12. The partner ACMC sends ACMX response to the partner ACMS.

13. The partner ACMS sends ACMX response to the primary ACMS.

14.	The primary ACMC stores the response.

NOTE 2:	There may be a considerable pause between step 14 and 15.

15.	The authorized user of primary MC system logs on to primary ACMC.

16. The primary ACMS notifies the ACMC of the pending request (clause 7.1.1.2.3).

17. The primary ACMC queries the primary ACMS for pending responses (clause 7.1.1.2.4).

18.	The primary ACMS forwards the ACMX response to the primary ACMC.

### 7.1.2	Solution evaluation

This solution provides generic procedure for ACMX between MC systems.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>