3GPP TSG-SA WG6 Meeting #16	S6-170459

Dali, China, 3rd – 7th April 2017	(revision of S6-170325, 170387)

Source:	Motorola Solutions

Title:	Pseudo-CR on local group regrouping

Spec:	3GPP TR 23.782

Agenda item:	9.2

Document for:	Approval

Contact:	david.chater-lea@motorolasolutions.com

1. Introduction

Solutions #6-1 and #6-2 for group regrouping assume that regrouping is possible between the systems via the IWF. In practice, most LMR systems do not support such behaviour, and group regrouping must remain local within the each system. If local regrouping is supported, there will be further limitations, as simultaneous regroups within each system could lead to duplicate paths for media and contention for floor control. This pCR identifies the limitations and requirements needed to allow an independent regrouping regime to be included.

2. Reason for Change

To include this solution within TR 23.782.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.782 v 0.4.1.

* * * First Change * * * *

### 5.6.x	Key issue #6-x – Separate regrouping within each system

#### 5.6.x.1	Description

##### 5.6.x.1.1	Local regrouping overview

If one of the MCPTT or the LMR system does not support group regrouping signalling via the IWF, group regrouping is only possible within each system locally. The outcome of this will be one or more regrouped groups on one system connected to the other system via the IWF, without the other system necessarily being aware that the regrouping has taken place.

The following subclauses illustrate the issues with regrouping and identifies requirements for the solutions.

##### 5.6.x.1.2	Regrouping issues

###### 5.6.x.1.2.1	Temporary group identification

If group regrouping is initiated on the MCPTT system, the IWF will need to understand the relationship between the temporary identity used on the MC system and the original MC service group identity to ensure that group members within the LMR system can continue to take part in group communications with group members within the MCPTT system. The IWF may hide the use of the temporary MCPTT group identity from the LMR system.

###### 5.6.x.1.2.2	Media routing

Care must be taken to ensure that regrouping a group that is connected to the LMR system via the IWF does not lead to duplication of media.

### 6.7.x	Solution #6-x: Local group regrouping within each system

#### 6.7.x.1	Regrouping solution description

To prevent routing issues and complexity, the following rules can be applied:

-	If group regrouping signalling using temporary groups is used on the MC system, the IWF must prevent the regroup signalling from propagating to the LMR system; and

-	the IWF must handle the translation between temporary group identities on the MC system and the original interworking group identity used on the LMR system; and

-	regrouping can only take place within a system if

-	the re-grouped groups are not interworking groups; or

-	the system in which the regouping is performed is the controlling system for those interworking groups involved in the re-grouping; or

-	the system in which the regrouping is performed in a participating system and only one of the groups included in the re-grouping is an interworking group, and floor control for the set of regrouped groups in the participating system is deferred to the controlling system via the IWF.

#### 6.7.x.2	Impacts on existing nodes and functionality

If group regrouping is permitted on the MCPTT system, there is no impact on the MCPTT server with respect to group regrouping signalling sent between systems, as the IWF will isolate the LMR system from the regroup signalling.

Group configuration within the group management server should identify interworking groups, and provide restrictions on the ability to perform group regrouping for interworking groups.

The MCPTT server will need to allow the configuration of groups that are interworking groups and for which regrouping is not permitted, and to prevent regrouping in those groups.

#### 6.7.x.3	Solution evaluation

This solution provides a means of allowing group regrouping within a system and still providing communication between systems, subject to limitations.