3GPP TSG- SA WG6 Meeting #34	S6-192373

Reno NV, USA, November 11 – 15, 2019

Source:	Ericsson

Title:	pCR 23.758 – New solution

Document for:	Agreement

Agenda Item: 	11.5

Work Item / Release:	FS_EDGEAPP / Rel-17

Abstract of the contribution: This contribution proposes a solution that expands over several Key Issues

# 1	Introduction

This pCR proposes a solution where the discovery of the optimal Edge Aplication server is done on a per need basis, that is at the moment the UE Application Client requires it. With this proposal, the UE connects to the Edge DN only when the Edge DN applications servers are going to be used, and the Edge Application Server discovery can take into account the latest network status.

This solution does not require the UE to select and register in the Edge DN: there is no need to create and maintain a UE context in the EES and an Edge DN context in the EEC. Still, the EEC may keep Application server discovery information for efficiency. As a result, some of the key issues do not apply:

KIs #2 & #8: “Edge Data Network discovery and registration” and “Edge Data Network Selection”. They are Not Applicable. It is the optimal Edge Application Server that is selected. There is no Edge data network discovery and registration procedure as such that need to take place outside the actual Edge Application Server selection.

KI #10: “Dynamic availability”. Does not apply since it is the network (5GC, EEC, and actual EAS) that handles this.

This pCR addresses KI #4: Edge Application Server discovery.

For the solution to be optimal the selection of the Edge Application Server needs to be paired with the 3GPP 5GC connectivity and the steering of the traffic of that application. This aspect has also been included in this pCR.

During the discussion on the ad-hoc meeting oct. 10, a chicken and egg issue raised for QoS. The QoS is set up for the UE to AS, i.e. a QoS flow may be established for this communication, and the EES or UE will trigger this prior UE starts communicating with the AS.

# 2	Proposal

## ***************** START CHANGES *****************

## 7.x	Solution #x: EAS discovery without registration to EES

This solution addresses Key Issues #1, #4. The key characteristics of this solution assumes that the EES does not keep any registration state of the UE.

### 7.x.1	Solution description

#### 7.x.1.1	Edge Data network service execution

Assumption:

1) UE has the edge application client. Since the dedicated application client, the mobile OS or an embedded module in the UE may act as the EEC, the flow below uses the term EEC from the architecture.

2) Prior to discovery, the UE does not register with the EES, and existance of edge computing, however, the UE (e.g. the mobile OS) may be impacted to support this solution. It is only the application, mobile OS or embedded module in the UE that may have a certain behaviour due to edge computing.

3) An EAS deployed in the Edge Hosting environement either registers in the EES (Key Issue #3) using for example, solution #12, where the solution includes Edge Enabler Server authorization check to verify whether the Edge Application Server has the proper privileges to register as part of the process. Or, the EES is provisioned with the EASes by means of O&M. The latter has no impact on “existing” cloud applications.

4) If there is a need for a specific QoS class for the communication to the EES, then this is established as part of the PDU session establishment. Thus, this must be configured in the 5GC.

Figure 7.x.1.2-1 Edge Application Server Discovery

There is a trigger to execute an application (internal or by a user). If no established PDU session can be reused, the UE establishes a new PDU session to the corresponding DNN and slice according to URSP. That session is used for the communication described below:

- The EAS is identified by a EAS-FQDN. The EEC in the UE does a discovery of an EAS over EDGE-1 by sending a discovery request to the EES with the EAS-FQDN as query parameter. There are examples how this is done in clause 7.x.1.2.1. The EES authenticates and authorizes the UE for this application, for example by using 3GPP credentials.

2.	If uplink classifier (UL-CL) and/or IPv6 multihoming are used to provide connectivity, the IP address may not be used to determine the EAS closest to the UE, this mean that the EES needs to get the UE location from the 5GC via e.g. NEF or PCF as per TS 23.502. E.g. using the Npcf_PolicyAuthorization_Subscribe service operation with immediate reporting flag set.

3.	 If UL-CL and/or IPv6 multihoming are to be used, the EES influences routing as per TS 23.502, see clause 4.3.6.3 step 2d.

4.	EES may initiate establishment of a QoS flow for this application as per TS 23.502, e.g as in 4.16.5.2. The EES may have been provisioned with the QoS information when EAS registers to EES

NOTE:	step 3 and 4 can happen after step 5.

5.	EES responds with the address of the EAS.

6.	UE may initiate a QoS flow establishment (see step 4), according to 23.502 clause 4.3.3.2 step 1a.

7.	Applicaton traffic starts between Application Client and EAS.

#### 7.x.1.2	Example Discovery methods

See two examples of EAS discovery using IETF protocols in the following sub-clauses.

- HTTPS based protocol (e.g. DNS over HTTPS) (see 7.x.1.2.1)

- DNS based protocol (see 7.x.1.2.2)

NOTE:	The SA2 WI FS_enh_EC has a KI for discovery. This can also result in more solutions for discovery of the EAS.

##### 7.x.1.2.1	HTTPS based protocol

The application/OS uses a HTTPS protocol (e.g. DNS over HTTPS) to discover the EAS by using the EAS-FQDN as a query parameter. A HTTPS connection is set up to the EES. The EES authenticates the UE and authorizes if UE is allowed to use this application as an edge application. How to authenticate (what client credentials to use) is an SA3 issue.

There are several ways how route the HTTPS connection to the EES:

-	the EES has an anycast address known by EEC in the UE. The IP network routes the request to the closest EES;

-	the EES has a FQDN, the EES-FQDN. The DNS server in the 5GC has knowledge of the location of the UE and returns to the UE the address of the local EES; or

-	the EES has an EES-FQDN. The EES-FQDN is resolved to anycast address of EES. This any EES (e.g. central EES) resolves the EAS location as follows:

1)	this EES may have the address of the EAS for the UE location;

2)	this EES may use HTTP re-direct to another EES (e.g. local EES) that can respond with the address of the EAS in the location of the UE; or

3)	this EES may further interact with other EES (e.g. via normal DNS) to resolve the EAS-FQDN and answer back to the UE using secured HTTP with the address of the EAS.

NOTE: The deployment of such EES and other EES is to be specified.

NOTE: The central EES can have local configuration to discover the local EES.

##### 7.x.1.2.2	Using DNS

In this case it is assumed the the EES is operated by the operator and that the connection between the UPF and EES is secure. If the Internet DNN and slice are used, the EES forwards DNS request the Internet DNS Hierarchy for non-edge applications. There are a number of options:

- the EES may resolve the request and answer back with the EAS for the UE location; or

- the EES may forward the request to the EES that can resolve it for the UE location.

NOTE:	This solution assumes a hierarchical EES architecture.

The EES authenticates the EEC/UE and authorizes if EEC/UE is allowed to use this application as an edge application.

NOTE: It is upto SA3 for authentication/authorization of the UE.  Since 5GC gives the UE an IP address and the connection between EES and UPF is secure, the EES can use the IP address to authenticate and authorize the UE. The EES can get location of the UE and directly query the site DNS for the address of the AS.

### 7.x.2	Solution Evaluation

This solution (e.g. using DNS) has minimum impact on application in the UE. The mobile OS or software module in the UE shall support this EEC functionality described in this solution.

The solution has solved key issues 4, and if the EES identifier (e.g. EES-FQDN) is preconfigured in the EEC, there is no need to solve Key issue #1.

****************** END CHANGES ******************