3GPP TSG-SA WG6 Meeting #39-e	S6-201463

# e-meeting, 31st August – 8th September 2020								(revision of S6-xxxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0277 | rev | - | Current version: | 17.3.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhanced remotely initiated ambient listening call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT |  |  |  |  |  | Date: |  |  | 2020-08-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The existing remotely initiated ambient listening call setup procedure may also easily be exploited for malicious purposes. Although it is expected that only an authorized user can request a remote ambient listening call, it could still be misused. Therefore, an enhanced remotely initiated ambient listening call procedure is introduced to define that the MCPTT server can notify and optionally request the approval of ambient listening call requests to one or more authorized MCPTT clients defined as "witness" user(s) for ambient listening calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An enhanced remotely initiated ambient listening call procedure is introduced to define that the MCPTT server can notify and optionally request the approval of ambient listening call requests to one or more authorized MCPTT clients defined as "witness" user(s) for ambient listening calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing remotely initiated ambient listening call setup procedure may be used for malicious purposes even by authorized users. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.14.1, 10.14.2.1, 10.14.2.x, 10.14.2.y, (new) 10.14.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * *

### 10.14.1	General

The ambient listening call is a type of a private MCPTT call that only allows a "listened to" user to transmit media to a "listening" user such that there is no indication on the MCPTT UE of the "listened to" user about the call and the media transmission.

NOTE 1:	"listened to" user refers to the user who is transmitting media in an ambient listening call.

NOTE 2:	"listening" user refers to the user who is receiving media in an ambient listening call.

There are two types of ambient listening call as below:

-	Remotely initiated ambient listening is initiated by the authorized user (e.g., dispatcher) who wants to listen to another user. In this case, the "listened to" user is the called party, and shall automatically accept the call without causing any indication about the call and transmit the media to the "listening" user.

NOTE 3:	The remotely initiated ambient listening call can be enhanced by configuring one or more "witness" users in the MCPTT system. A "witness" user refers to an authorized user who can be notified and optionally requested to approve a remotely ambient listening call request.

-	Locally initiated ambient listening is initiated by an authorized user who wants another user to listen to the MCPTT UE communication. In this case, the "listened to" user is the calling party and shall automatically transmit the media to the "listening" user without causing any indication about the call processing and media transmission.

* * * Next change * * *

#### 10.14.2.1	Ambient listening call request

Table 10.14.2.1-1 describes the information flow ambient listening call request from the MCPTT client to the MCPTT server and MCPTT server to the MCPTT client.

Table 10.14.2.1-1: Ambient listening call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the "listening" user |
| MCPTT ID | M | The MCPTT ID of the "listened to" user |
| SDP offer | M | Media parameters of MCPTT client. |
| Ambient listening type | M | The ambient listening type indicates remotely initiated ambient listening call or locally initiated ambient listening call. |
| List of MCPTT IDs | O (NOTE) | Indicates the list of MCPTT IDs that can be used as "witness" users |
| NOTE: This information element may be present when the enhanced remotely initiated ambient listening call procedure, described in clause 10.14.3.x, is used. |  |  |

* * * Next change * * *

#### 10.14.2.x	Remote ambient listening call notification

Table 10.14.2.x-1 describes the information flow remote ambient listening call notification from the MCPTT server to the MCPTT client.

Table 10.14.2.x-1: Remote ambient listening call notification

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the "listening" user |
| MCPTT ID | M | The MCPTT ID of the "listened to" user |
| MCPTT ID | M | The MCPTT ID of the "witness" user |
| Approval request | M | Indicates if an approval response is requested or not from the "witness" user |

#### 10.14.2.y	Remote ambient listening call approval response

Table 10.14.2.y-1 describes the information flow remote ambient listening call approval response from the MCPTT server to the MCPTT client.

Table 10.14.2.y-1: Remote ambient listening call approval

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the "listening" user |
| MCPTT ID | M | The MCPTT ID of the "listened to" user |
| MCPTT ID | M | The MCPTT ID of the "witness" user |
| Approval response | M | Indicates if the ambient listening call request is approved or not by the "witness" user |

* * * Next change * * *

#### 10.14.3.x	Enhanced remotely initiated ambient listening call

The enhanced remotely initiated ambient listening call procedure is an enhanced version of the procedure described in clause 10.14.3.1, where upon the receipt of an ambient listening call request, the MCPTT server notifies and optionally requests the approval of the ambient listening call to one or more authorized MCPTT clients defined as "witness" user(s) for ambient listening calls. The enhanced remotely initiated ambient listening call setup procedure is intended to prevent a misuse of this feature.

NOTE 1:	The MCPTT server may receive a list of MCPTT IDs from the "listening" user that can be defined as "witness" users. This list can be beneficial in certain scenarios such as emergency or secret operations. However, it is up to the MCPTT server to ultimately define the authorized users that can be defined as "witness" users.

Figure 10.14.3.x-1 illustrates the information flow for the enhanced remotely initiated ambient listening call setup.

Pre-conditions:

-	MCPTT client 1 is the client of the authorized user who is authorized to invoke a remotely initiated ambient listening call to be set up at the requested MCPTT client 2.

-	MCPTT clients 3 to N are the clients of authorized users to be notified and optionally requested to approve a remotely initiated ambient listening call request.

-	MCPTT user 1 is the "listening" user at MCPTT client 1, MCPTT user 2 is the "listened to" user at MCPTT client 2, and MCPTT users 3 to N are the "witness" users at MCPTT clients 3 to N.

Figure 10.14.3.x-1: Enhanced remotely initiated ambient listening call

1.	MCPTT client 1 initiates a remotely initiated ambient listening call by sending the ambient listening call request to the MCPTT server. The remotely initiated ambient listening call type is included.

2.	The MCPTT server performs an authorization check to identify if the MCPTT user 1 is authorized to remotely initiate an ambient listening call towards the MCPTT user 2. If authorization fails, the MCPTT server provides a failure response to MCPTT client 1.

3.	If the MCPTT user 1 is authorized, the MCPTT server provides a remote ambient listening call notification to the MCPTT clients of the authorized users defined as "witness" users. The MCPTT server may request the "witness" users to approve the ambient listening call request.

4.	If requested by the MCPTT server, the MCPTT clients of the defined "witness" users provide a response to the MCPTT server to indicate if the ambient listening call request is approved or not.

5.	If an approval response was requested by the MCPTT server, the MCPTT server based on the approval response(s) defines if the remotely initiated ambient listening call request is authorized or not for the MCPTT client 1.

NOTE 2:	It is implementation specific how the MCPTT server manages the approval responses when multiple "witness" users are defined. For example, the MCPTT server can decide to authorize the request based on the reception of at least one of the expected approval responses.

6-11. Steps 6 to 11 are described in figure 10.14.3.1-1 from steps 3 to 8.

12.	The ambient listening call is released, e.g. as described in clauses 10.14.3.3 or 10.14.3.4.

13.	The MCPTT server sends an ambient listening call release notification to MCPTT clients 3 to N to notify the "witness" users about the release of the corresponding remotely initiated ambient listening call.