3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-210775

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Nokia, Nokia Shanghai Bell, UIC

Title:	Migration to another MC system during an ongoing private communication

Spec:	3GPP TR 23.700-90

Agenda item:	8.4

Document for:	Approval

Contact:	Martin Öttl (martin.oettl@nokia.com)

1. Introduction

This papers suggests a solution to address key issue 5 described in clause 5.5 on quick migration towards another MC system.

The solution provides the capability for an MC service user for migration to another MC system during an ongoing private call.

2. Reason for Change

Support for migration within an ongoing private communication.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-90 version 0.2.0.

* * * First Change * * * *

## 7.x	Migration during an ongoing private communication

### 7.x.1	General

This solution addresses the key issue 5 described in clause 5.5 on quick migration towards another MC system.

The solution provides the capability for an MC service user to migrate to another MC system during an ongoing private communication and to continue the private communication in the other MC system without MC service user interaction.

### 7.x.2	Solution description

#### 7.x.2.1	Procedure

The procedure is based on the following existing procedures:

-	Client initiated private call release procedure as described in TS 23.379 [2] clause 10.7.2.2.3.1, or

-	Private call release – MCPTT users in multiple MCPTT systems as described in TS 23.379 [2] clause 10.7.2.3.3.

-	MC service user receiving MC service from a partner MC system described in TS 23.280 [5] clause 10.1.4.3.2.

-	Private call setup in automatic commencement mode as described in TS 23.379 [2] clause 10.7.2.2.1, or

-	Private call setup in automatic commencement mode – MCPTT users in multiple MCPTT systems described in TS 23.379 [2] clause 10.7.2.3.1.

NOTE 1:	The solution is about MCPTT private calls but is applicable for other services too.

Pre-conditions:

- The MCPTT client has one or more ongoing private calls in the primary MC system.

- The MCPTT UE detects the need to change the MC system.

Figure 7.x.2.1-1:	Migration to partner MC system during an ongoing private call

1.	The MCPTT client 1 requests private call release as as described in TS 23.379 [2] clause 10.7.2.2.3.1 or TS 23.379 [2] clause 10.7.2.3.3. The MCPTT private call end request contains an indication to suppress the call termination notification to MCPTT user 2.

2.	After migration to the other MC system, the configuration management client 1 triggers retrieval of the MC service user profile used within the partner MC system (TS 23.280 [5] clause 10.1.4.3.2).

NOTE 2:	User authentication, service authorisation and signalling plane procedures are not shown.

3.	The MCPTT client 1 requests private call setup in automatic commencement mode as described in TS 23.379 [2] clause 10.7.2.2.1 or TS 23.379 [2] clause 10.7.2.3.1. The MCPTT private call request contains an indication to suppress the call establishment notification to the MCPTT user.

NOTE 3:	Any local call restrications are taken into account.

The MCPTT client may indicate the successful migration of private call communications to the MCPTT user.

#### 7.x.2.2	Infromation flows

##### 7.x.2.2.1	MCPTT private call end request

Table 7.x.2.2.1-1 relies on TS 23.379 [2] table 10.7.2.1.4a-1 and describes the information flow MCPTT private call end request from the MCPTT client to the MCPTT server and from the MCPTT server to the MCPTT client.

Table 7.x.2.2.1-1: MCPTT private call end request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Suppress user notification | O | Suppress sending a call termination notification to the MCPTT user |

##### 7.x.2.2.2	MCPTT private call request (MCPTT client to MCPTT server)

Table 7.x.2.2.2-1 relies on TS 23.379 [2] table 10.7.2.1.1-1 and describes the information flow MCPTT private call request from the MCPTT client to the MCPTT server.

Table 7.x.2.2.2-1: MCPTT private call request (MCPTT client to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID (see NOTE) | O | The MCPTT ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | O | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Location information | O | Location of the calling party |
| Requested priority | O | Application priority level requested for this call |
| Transfer indicator | O | Indicates that the MCPTT private call request is a result of a call transfer (true/false) |
| Forwarding indicator | O | Indicates that the MCPTT private call request is a result of a call forwarding. (true/false) |
| Suppress user notification | O | Suppress sending a call establishment notification to the MCPTT user |
| NOTE: At least one identity must be present. |  |  |

##### 7.x.2.2.3	MCPTT private call request (MCPTT server to MCPTT server)

Table 7.x.2.2.3-1 relies on TS 23.379 [2] table 10.7.2.1.2-1 and describes the information flow MCPTT private call request from the MCPTT server to the MCPTT server.

Table 7.x.2.2.3-1: MCPTT private call request (MCPTT server to MCPTT server) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Functional alias | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Requested priority | O | Priority level requested for the call. |
| Location information | O | Location of the calling party |
| Suppress user notification | O | Suppress sending a call establishment notification to the MCPTT user |

##### 7.x.2.2.4	MCPTT private call request (MCPTT server to MCPTT client)

Table 7.x.2.2.4-1 relies on TS 23.379 [2] table 10.7.2.1.2a and describes the information flow MCPTT private call request from the MCPTT server to the MCPTT client.

Table 7.x.2.2.4-1: MCPTT private call request (MCPTT server to MCPTT client) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Functional alias | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| Suppress user notification | O | Suppress sending a call establishment notification to the MCPTT user |

### 7.1.2	Solution evaluation

Editor's Note:	This clause will provide the evaluation of this solution.

* * * End of Change * * * *