3GPP TSG-SA WG6 Meeting #43	S6-211148

e-meeting, 24th May – 2nd June 2021	(revision of S6-21xxxx)

Source:	Tencent

Title:	Support for UAS identification usage in application layer architecture

Spec:	3GPP TS 23.255 v0.4.0

Agenda item:	7.10

Document for:	Approval

Contact:	Shuai Zhao <shuaiizhao@tencent.com>

1. Introduction

This contribution adds a new feature in UASAPP TS 23.255. It focuses on CAA-level ID update due to UAV replacement and describes the interactions between the UAE server and SEAL as well as how to use SEAL for group membership update when there is a UAV replacement.

2. Reason for Change

Solution#8 in TR 23.755 provides a viable solution to support dynamic UAV replacement.

3. Conclusions

UAV replacement is a valid use case per stage 1 requirement. UAS application layer shall support such use cases.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.255 v0.4.0.

* * * First Change * * * *

## 6.x 	UAS Identification Usage In Application Layer Architecture

### 6.x.1	General

This clause describes the procedure for UAS identification usage in application layer architecture. It focuses on CAA-level ID update due to UAV replacement and describes the interactions between the UAE server and SEAL as well as how to use SEAL for group membership update when there is a UAV replacement.

### 6.x.2	Procedures

#### 6.x.2.1	Procedure for UAS identification usage in application layer architecture

The pre-conditions for the considered solution space are as follows:

-	The UAV-C, UAV-1, and UAV-2 are all previously successfully subscribed with 3GPP and USS/UTM and received a 3GPP UE ID (e.g. GPSI) and a CAA-level ID.

-	The UAV-1 and UAV-C have previous given a group ID by SEAL GMS.

Figure 6.x.2.1-1 illustrates the group membership update when UAV-2 is used to replace UAV-1

Figure 6.x.2.1-1: Procedure for group membership update after UAV replacement

1.	The UAE server recognizes a new pair of UAV-2 and UAV-C by the new CAA-level UAV ID.

2.	The UAE server sends a group membership update request to the SEAL GM server using the procedure specified in clause 10.3.2.6 of 3GPP TS 23.434 [11].

3.	The SEAL GM server sends a group membership update response as specified in clause 10.3.2.7 of 3GPP TS 23.434 [11].

### 6.x.3	Information flows

#### 6.x.3.1	Group membership update request

Table 6.x.3.1-1 describes the information flow of group membership update requests from UAE client to UAE server.

Table 6.x.3.1-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| UAE client ID | M | The identifier of the UAE client |
| UAV-1 UE ID | M | The Identifier of UAV-1 UE ID |
| UAV-2 UE ID | M | The Identifier of UAV-2 UE ID, which is the replacement of UAV-1 |
| CAA-level UAV ID | M | The new CAA-Level UAV ID associated with UAV-2 |
| Timeout | O | The timeout period when group membership update response is not received. A new request may be sent again after the timeout. |

#### 6.x.3.2	Group membership update response

Table 6.x.3.2-1 describes the information flow of group membership update response from UAE server to UAE client.

Table 6.x.3.2-1: Group membership udpate response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The identification of successfulness of group membership update. A new group id may or may not assigned. |

* * * End of Change * * * *