3GPP TSG-SA6 Meeting #43-e	S6-211390

# Online, , 24th May 2021 - 2nd Jun 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0062 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of VAE server APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR provides the missing API definitions for the added procedures in 9.15.3.2 (PC5 provisioning in multi-operator V2X services) and 9.17.3 (UE-to-UE groupast/broadcast configuration). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 10, Table 10.2.1-1 included the new VAE server APIs, and in 10.2.x and 10.2.y the detailed new VAE server APIs description was provided for the procedures in 9.15.3.2 and 9.17.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API definitons will be missing from the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clauses 10.2.1, 10.2.x, 10.2.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 10.2	VAE server APIs

### 10.2.1	General

Table 10.2.1-1 illustrates the VAE server APIs.

Table 10.2.1-1: List of VAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| VAE_MessageDelivery API | Deliver_DL_Message | V2X application specific server | Request/ Response |
|  | Deliver_UL_Message | V2X application specific server | Subscribe/notify |
| VAE_FileDistribution API | Distribute_File | V2X application specific server | Request/ Response |
| VAE_ApplicationRequirement API | Reserve_NetworkResource | V2X application specific server | Subscribe/notify |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup API | Configure_DynamicGroup | V2X application specific server | Request/Response |
|  | Notify_DynamicGroup |  |  |
| VAE_ServiceContinuity API | Query_ServiceContinuity | VAE server | Request/Response |
| VAE_HDMapDynamicInfo API | Subscribe_HDMapDynamicInfo | V2X application specific server | Subscribe/notify |
|  | Notify_HDMapDynamicInfo |  |  |
| VAE_PC5ProvisioningRequirement API | Config_PC5ProvisioningRequirement | V2X application specific server | Subscribe/notify |
|  | Notify_PC5ProvisioningRequirement |  |  |
| VAE_V2VConfigRequirement API | Request_V2VConfigRequirement | V2X application specific server | Request/Response |

The SEAL server(s) APIs available via SEAL-S reference point as specified in 3GPP TS 23.434 [6] are consumed by the V2X application specific server via the VAE server (acting as a VAL server) over the Vs reference point. When V2X application specific server invokes a SEAL server API via the VAE server over the Vs reference point, the VAE server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response.

NOTE:	When V2X application specific server invokes a SEAL server API directly, the V2X application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [6].

Editor's note:	Whether the VAE_DynamicGroup API is to be moved to SEAL is FFS.

* * * * * * * SECOND CHANGE * * * * * * *

### 10.2.x	VAE_PC5ProvisioningRequirement API

#### 10.2.x.1	General

API description: This API enables the V2X application specific server to communicate with the VAE server to request from VAE server the PC5 provisionng service in multi-operator V2X scenarios.

#### 10.2.x.2	Config_PC5ProvisioningRequirement operation

API operation name: Config_PC5ProvisioningRequirement

Description: The V2X application specific server provides a V2X PC5 provisioning requirement to the VAE server.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.15.2.1

Outputs: Acknowledgement message.

See subclause 9.15.3 for the details of usage of this API operation.

#### 10.2.x.3	Notify_PC5ProvisioningRequirement operation

API operation name: Notify_ PC5ProvisioningRequirement

Description: Notify the result of multi operation PC5 provisioning requirement to the V2X UEs.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.15.2.6

Outputs: None

See subclause 9.15.3 for the details of usage of this API operation.

### 10.2.y	VAE_V2VConfigRequirement API

#### 10.2.y.1	General

API description: This API enables the V2X application specific server to provide a V2V configuration requirement to the VAE server to manage the UE-to-UE broadcast/groupcast communication.

#### 10.2.y.2	Request_V2VConfigRequirement operation

API operation name: Request_V2VConfigRequirement

Description: The V2X application specific server provides a V2V configuration requirement request to the VAE server to manage the UE-to-UE broadcast/groupcast communication.

Known Consumers: V2X application specific server.

Inputs: Refer subclause 9.17.2.1

Outputs: Refer subclause 9.17.2.2

See subclause 9.17.3 for the details of usage of this API operation.