3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212249

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0054 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | List of subscriptions to the CN in EEC context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR addresses the EN “Additional IE(s) for the subscriptions to CN the list of IEs is FFS, pending SA2 discussions (see TR 23.748 clause 9.2.6).” in clause 8.2.8. In SA2, it is agreed that target AF may invoke Nnef_TrafficInfluence_Update service operation to deliver the relocation information when edge relocation involving AF change occurs as described in clause 4.3.6.2 in TS 23.502 and clause 6.3.2 in TS 23.548. In this regard, when the relocation occurs, the subscription information (such as subscription correlation ID) of source EES needs to be delivered to the target EES. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The list of subscriptions to the CN is added in the service session context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editor’s note remains unresolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.8	EEC Context

The EEC Context includes information about an EEC for receiving edge enabler services.

Table 8.2.8-1: EEC Context

| Information element | Status | Description |
| --- | --- | --- |
| EEC ID | M | Unique identifier of the EEC. |
| EEC Context ID | M | Identifier assigned to the EEC Context |
| Source EES Endpoint | M | The endpoint address (e.g., URI, IP address) of the EES that provided EEC context ID. |
| UE Identifier | O | The identifier of the hosting UE (i.e., GPSI or identity token) |
| List of EDGE-1 subscriptions | O | List of subscriptions IDs for capability exposure to the EEC ID |
| UE location | O | Latest UE location of the UE hosting the EEC which was available at the EES. |
| List of AC Profiles | O | Information about the ACs as described in Table 8.2.2-1. |
| List of subscriptions to core network | O | List of subscription correlation ID(s) for network capability exposure. This IE is used by T-EES to update the subscription which the S-EES performed to the core network. |
| List of Service Session Contexts | O | List of associated Service Session Context IEs. Each Service Session Context includes information maintained by the EES for the services (involving UE related resources) received from an EAS registered to the EES. |
| > Service Session Context | M | Service Session Context is described in Table 8.2.8-2 |

Table 8.2.8-2: Service Session Context

| Information element | Status | Description |
| --- | --- | --- |
| EAS ID | M | Identifier of the EAS providing the application services |
| EAS Endpoint | M | Endpoint information of the EAS. |
| AC ID | O | Identifier of the AC ID for which the service session is provided, if determined. |

Editor's Note:	It is FFS whether the EEC Context information captured in table 8.2.A-2 requires further updates and alignment to include Edge-3 subscription information.

* * * End of Change * * * *