3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212328

# e-meeting, October 11 – 19, 2021	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0005 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Server triggered UE leave multicast MBS session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Server triggered UE leave multicast MBS session is one of the basic features for supporting MCX over 5G MBS. Without supporting such it is not possible to enable MCX server initiate the removal for a certain UE(s) to not receive the MBS session data. This CR is to add the related solution for Server triggered UE leave multicast MBS session, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description about Server triggered UE leave multicast MBS session in section 7.X.Y.A1 - Adding associated procedure about Server triggered UE leave multicast MBS session in section 7.X.Y.A2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about supporting Server triggered UE leave multicast MBS session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new), 7.x.y (new), 7.x.y.1 (new), 7.x.y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.X	MC service over 5G MBS

### 7.X.Y	Server triggered UE leave multicast MBS session

#### 7.X.Y.1	General

The MC service server can request one or several MC service clients to leave the multicast MBS session by indicating its MBS session ID. This can be triggered for example once no further MC media is needed to be delivered to the targeted MC service group, the multicast MBS session is no longer associated to the MC group, or a certain MC service client is no longer authorized to receive data to the associated MC service group. It is applicable only for multicast MBS sessions. The UE session leave aspects are described in 3GPP TS 23.247 [xx].

Editor's note: The feasibility of re-assigning multicast MBS session to different groups is FFS, and depends on SA2's supporting of authorization, multicast MBS session update due to authorization update.

#### 7.X.Y.2	Procedure

The procedure in figure 7.X.Y.2-1 describes how MC service client(s) leaves a certain multicast MBS session.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	An MBS session is configured to address the corresponding MC service group with certain service requirements and optionally with a certain multicast service area. The session is announced and established for group communication purposes for the MC service group.

-	MC service clients 1 to n have successfully joined the multicast MBS session.

Figure 7.X.Y.2-1: UE multicast MBS session leave

1.	The MC service server decides that one or several MC service clients receives should leave a certain multicast MBS session. This can be triggered for example once no further MC media is needed to be delivered to the targeted MC service group, the multicast MBS session is no longer associated to the MC group, or a certain MC service client is no longer authorized to receive data to the associated MC service group.

2.	The MC service server sends an MBS session leave request towards the MC service client(s) indicating the multicast MBS session ID.

Editor's note: Whether the MBS session leave request can be de-announcement / UnMapGroupToMBSSession /de-affiliation is FFS.

3.	The MC service client(s) initiates an MBS session leave procedure to leave the indicated MBS session in order to release the respective network resources, as defined in 3GPP TS 23.247 [xx].

4.	The MC service client(s) provides a response to the MC service server indicating that they have left the corresponding multicast MBS session.

.

# * * * * End of changes * * * *