3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212329

# e-meeting, October 11 – 19, 2021	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selection of multicast and broadcast service for a group communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Selection of multicast and broadcast service for a group communication is one of the basic features for supporting MCX over 5G MBS. And during TR phase, the solution was once discussed. Without supporting such it is not possible for the MC server to select multicast and broadcast service for a group communication. This CR is to add the related solution for Selection of multicast and broadcast service for a group communication, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description about Selection of multicast and broadcast service for a group communication 7.X.Y |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about supporting Selection of multicast and broadcast service for a group communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new), 7.x.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	Selection of multicast and broadcast service for a group communication

Editor's Note:	The description in this clause is for the transport only mode. Full service mode is FFS.

Both the multicast MBS session and broadcast MBS session can be used to enable the the point-to-multipoint tranmission for a group communication. When selecting the multicast and broadcast service for a group communication the following principles is considered:

-	During the group communication lifetime, either the multicast MBS service or the broadcast MBS service is used to enable the point-to-multipoint transmission for a group communication. Both or the dynamic switch between multicast and broadcast session are not supported.

-	For a specific group communication, the selection of multicast MBS session or broadcast MBS session is determined by the MC service server and implementation specific. The following aspect maybe considered: the MC service operator’s policy, static group configuration e.g., specify the MBS session type to be used in the group configuration data, or dynamically group attributes e.g., group size, service type, group is a broadcast group etc.

-	If multiple selection criteria is used, the precedence shall be specified. For example the order of precedence may be dynamically group attributes, static group configuration, MC service operator’s policy.

# * * * * End of changes * * * *