3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212331

# e-meeting, October 11 – 19, 2021	(revision of S6-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0008 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call connect and disconnect over 5G MBS in MCPTT context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR, the Call connect and disconnect over 5G MBS in MCPTT context were discussed and it is an important feature for MCX over 5G MBS. It is worthwhile to include MCPTT related procedures in a holistic view among MCPTT server and the clients. This CR is to add the related solution for Call connect and disconnect over 5G MBS in MCPTT context, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description about Call connect and disconnect over 5G MBS in MCPTT context - Adding procedures and the associated Information flows for Call connect and disconnect over 5G MBS in MCPTT context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No related content about Call connect and disconnect over 5G MBS in MCPTT context in current TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.X (new), 7.x.y (new), 7.x.y.1 (new), 7.x.y.2 (new), 7.x.y.2.1 (new), 7.x.y.2.2 (new), 7.x.y.2.2.1 (new), 7.x.y.2.2 (new), 7.x.y.2.2.1 (new), 7.x.y.2.2.2 (new), 7.x.y.2.3 (new), 7.x.y.2.3.1 (new), 7.x.y.2.3.2 (new), 7.x.y.2.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 7.X	MC service over 5G MBS

### 7.X.Y	MBS transmission in MCPTT

#### 7.X.Y.1	General

The application layer signalling such as the floor control messages may be transmitted along with the MC data over the same MBS session however with different QoS requirements, or over other means such as unicast downlink or a different MBS session.

For broadcast MBS sessions:

- MCPTT may use pre-established broadcast MBS session for the different types of MCPTT group calls. Both pre-arranged group calls and chat group calls can use the pre-established broadcast MBS session for distributing the DL data. The broadcast MBS session can be used by any MCPTT group. Depending on the capacity of the MBMS bearer, the broadcast MBS session can be used to broadcast one or more group calls in parallel.

For multicast MBS session:

- Multicast MBS session can be only used by one MCPTT group at a time. The MCPTT service shall support the procedure for using pre-established MBS sessions.

- MCPTT may use dynamic MBS session establishment for the different types of MCPTT group calls. Both pre-arranged group calls and chat group calls can use the dynamic MBS session for distributing the DL data. The MCPTT service shall support the procedure for using dynamic MBS sessions.

- In order to receive the corresponding MC data, the MCPTT UE initiates the session join towards the 5GS, when it is aware of via the service announcement. After the MC media transmission is over, if the MCPTT server decides to stop the corresponding MBS session, it sends an MBS session stop request towards the 5GS, where the latter removes the joined MCPTT UEs from the corresponding MBS session in case of multicast sessions.

When using the procedures for pre-established or dynamic MBS session establishment for MCPTT, the MCPTT server perform the procedure of call connect and disconnect over MBS session as defined in the next clause at the group communication session establishment phase.

#### 7.X.Y.2	Call connect and disconnect over MBS session procedures

##### 7.X.Y.2.1	General

MBS session can be used for MCPTT group calls. One MBS session may be not permanently associated to one specific group or group call.

Before sending media packets of a group call over MBS session:

- If one multicast session is re-assigned to different groups, the MCPTT server sends the association information between group call and the MBS session. The group call setup procedure indicates the media stream within one MBS session that is used for the specific group call.

- If one Multicast session only for one group, the AF then triggers to establish the MBS session.

When the group call over the MBS session is finished:

- If one multicast session is re-assigned to different groups, the temporary association information of an MCPTT group call to specific resources on a MBS session is undone.

- If one Multicast session only for one group, the AF then can send service de-announcement request to the associated UEs and release the MBS session. Note that if the AF decides to release the MBS session, the AF shall send the de-announcement request to the UEs.

NOTE X:	It is implementation-specific that one multicast session can be re-assigned to different groups, or is associated to only one group.

The procedure in clause 7.X.Y.2.2 requires that the group session is setup before the media transmission start. This eliminates the need for the receiving clients to continuously use a unicast bearer. Prior to this the MBS session is activated and announced to the MCPTT clients.

##### 7.X.Y.2.2	Procedure

###### 7.X.Y.2.2.1	Group call connect over MBS session

In figure 7.X.Y.2.2.1-1 the MCPTT client 1 is the client that initiate an MCPTT chat group call and also the transmitting client. MCPTT client 1 may, but does not have to be in a broadcast service area if the MBS session is broadcast MBS session. The MCPTT client 2 to MCPTT client N represents MCPTT clients receiving the MCPTT call over an MBS session. There may be other receiving clients over unicast delivery or over this MBS session, however they are not illustrated in this figure.

The same procedure as for chat group call can also be applied for pre-arranged calls. However that requires that the pre-arranged sessions are already active. If a pre-arranged group session is initiated the use of pre-established sessions are required.

Pre-conditions:

-	The MBS session is already established and announced to the participating users. For multicast MBS session, the the participating users have successfully joined the call and notify the MCPTT server. For the broadcast MBS session, the participating users have notified the listening status to the MCPTT server.

-	All users participating in the MCPTT group call are already affiliated to the group.

-	All participating users have joined the group session.

-	The MCPTT clients 2 to n are within the MBS service area (if the session is limited to an area) where the MBS session is configured. MCPTT clients 1 may be within the MBS service area.

Figure 7.X.Y.2.2.1-1: Group call connect on MBS session

1.	MCPTT client 1 initiates the MCPTT group call (i.e., a video group call) by sending an initial transmit media request over a PDU session to the MCPTT server (reference point MCPTT-4).

2.	The MCPTT server will send a MapGroupToMBSsession message over a previously activated MBS session to all users that will receive the call via the MBS delivery. The MapGroupToMBSsession message includes association information between the group call and MBS session. The MapGroupToMBSsession message includes MCPTT group ID and information about the media stream identifier of the established MBS session and may include the MBS session identifier (e.g., the TMGI) of the MBS session for the DL media. The message is sent over reference point MCPTT-9.

If the acknowledgement indication is included in the MapGroupToMBSsession, the MCPTT client(s) may send the acknowledgement message to the MCPTT server.

NOTE 1:	Step 2 can be deferred until step 4 and the MapGroupToMBSsession message can then be included in the floor taken message.

3.	The MCPTT server grants the right to transmit for MCPTT client 1 and by that sends a transmit media granted message to the MCPTT client 1. This message is sent over a unicast PDU session (reference point MCPTT-4).

4.	A media transmission notification message is sent from the MCPTT server to all receiving users. This message includes the MCPTT ID of the transmitting MCPTT client as well as the MCPTT group ID. The message is sent over the MBS session. The message is sent over reference point MCPTT-9.

5.	The users of MCPTT clients are informed about the details of video transmission and the video transmitter, and process the related messages.

6.	The MCPTT clients may send a receive media request to the MCPTT server, to indicate the reception of media over the associated MBS session.

7.	MCPTT server accepts the media receive request and sends a receive media response message to MCPTT clients over the associated MBS session by using reference point MCPTT-9.

NOTE 2:	The receive media response can also be sent by unicast to the MCPTT clients that request to receive media over the MBMS bearer.

NOTE 3:	Step 7 and 8 may be omitted when auto-receive transmission mode is used, e.g. for emergency group calls.

8.	The media is sent from MCPTT client 1 to the MCPTT server over unicast bearer and from the MCPTT server to the MCPTT client 2 and MCPTT client 3 over the MBS session.

NOTE 4:	Additional floor request messages in the same call will not trigger the MapGroupToMBSsession message to be sent.

###### 7.X.Y.2.2.2	Group call disconnect from MBS session

The MCPTT server informs the MCPTT clients that the group call is being dissociated from the specified broadcast or multicast MBS session once the MC media transmission is over. Figure 7.X.Y.2.2.2-1 shows the high level procedure. For this purpose, an UnmapGroupFromMBSsession message is sent by the MCPTT server to the MCPTT clients to indicate that the MCPTT group call is being dissociated from the MBS session.

The MCPTT server may initiate an MBS session stop towards the 5GS, where the latter removes the MCPTT clients to reserve the network resources in the 5GC and NG-RAN.

Pre-conditions:

-	The MC group call is taking place over the associated MBS session. MCPTT client 1 is sending the MC media over a unicast PDU session to the MCPTT server;

-	The MCPTT delivers the MC media over the associated broadcast or multicast MBS session to MCPTT clients 2 to n.

Figure 7.X.Y.2.2.2-1: Group call disconnect from MBS session

1.	MCPTT server has determined to disconnect the call over the MBS session for the MCPTT clients.

2.	After the MC media transmission is over, i.e., no further media to be sent over the group communication, an UnmapGroupFromMBSsession message is sent by the MCPTT server to MCPTT client 1 to MCPTT client n on via MBS session.

NOTE X1:	The UnmapGroupFromMBSsession message can be sent as part of other messages.

The MCPTT server may decide to stop the MBS session by sending an MBS session stop request towards the 5GS indicating the MBS session ID to be stopped.

NOTE X2:	In case of multicast MBS sessions, the 5GS may remove the MCPTT UEs 2 to n from the multicast MBS session as a response to the session stop request. Hence, the resources in both NG-RAN and 5GC are released.

##### 7.X.Y.2.3	Information flows for MBS transmission

###### 7.X.Y.2.3.1	MapGroupToMBSsession.

The MCPTT server provides the MCPTT clients with the necessary information to map a specific group call with the stream to be utilized within the announced and established broadcast or multicast MBS session. Such information can be provided via the MapGroupToSessionStream message.

Table 7.X.Y.2.3.1-1 defines the MapGroupToSessionStream to be sent from the MCPTT server to MCPTT clients to provide specific required information to receive the data related to a group communication within an MBS session.

Refer to the Data delivery, for MapGroupToMBSsession, it is with the following clarifications:

-	The MC service client is the MCPTT client;

-	The MC service server is the MCPTT server; and

-	The MC service group ID is the MCPTT group ID.

Table 7.X.Y.2.3.1-1: MapGroupToSessionStream

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT group ID | M | This element identifies the MCPTT group related to a group communication to be delivered over the MBS session |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group communication within the MBS session |
| MBS session ID | O | Indicating the MBS session ID, if the MapGroupToSessionStream is not sent over the same MBS session |

###### 7.X.Y.2.3.2	UnmapGroupFromMBSsession

Table 7.X.Y.2.3.2-1 describes the information flow to disconnect a MCPTT group call from a MBS session. It is sent from the MCPTT server to the MCPTT client.

Table 7.X.Y.2.3.2-1: UnmapGroupFromMBSsession

| Information element | Status | Description |
| --- | --- | --- |
| MCPTT group ID | M | This element identifies the MCPTT group related to a group call to be dissociated over the MBS session |
| Media stream identifier | M | This element identifies the media stream of the SDP, which is no longer used for the group call within the MBS session |
| MBS session ID | O | Indicating the MBS session ID, if the information is sent over another MBS session or unicast path |

###### 7.X.Y.2.3.3	MapGroupToMBSsession ACK

Refer to the general case of Data delivery over multicast MBS session, with the following clarifications:

-	The MC service client is the MCPTT client;

-	The MC service server is the MCPTT server; and

-	The MC service group ID is the MCPTT group ID.

# * * * * End of changes * * * *