3GPP TSG-WG SA6 Meeting #45-bis-e	S6-212423

# e-meeting, October 11 – 19, 2021	(revision of S6-212326, merge of 2240)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0003 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for MBS session configuration and service announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 5G MBS session configuration and service announcement is an important feature for supporting MCX over 5G MBS. In TR 23.783, the solutions #10, #12-#15 are for session configuration and service announcement. Sol#10 and Sol#12 ~ Sol#15 address how MC service server configure the MBS session to be established and provide the service announcement to the MC service UE. They are technical aligned with each other, but with different documentation fashions. It is concluded in TR 23.783 that Sol#10 will be used as the baseline for the TS. Pre-configuring 5G MBS session before MCX communication setup can save totally delay of starting media delivery over 5G MBS. This CR is to add the related solution for Pre-configured and Dynamic 5G MBS session, based on TR conclusion on this topic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding general description about 5G MBS session configuration - Adding general description about 5G MBS service announcement. - Adding general description about procedure of 5G MBS session configuration and service announcement. - Adding procedure for pre-configured MBS session and service announcement. - Adding procedure for dynamic MBS session and service announcement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G MBS cannot be used MCX communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.x (new), 7.x.1 (new), 7.x.1.1 (new), 7.x.1.2 (new), 7.x.1.3 (new), 7.x.1.4 (new), 7.x.1.4.1 (new), 7.x.1.4.2 (new), 7.x.1.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# * * * * Second change* * * *

## 7.X	MC service over 5G MBS

### 7.X.1	MBS session configuration and service announcement

#### 7.X.1.1	General

When the MC service server decides to use MBS sessions for MC service group communications, the MC service server is first required to configure with the 5GC one or more MBS sessions to deliver the media associated to the MC service group communications.

Service Announcement provides the MC users by MC service server with descriptions specifying the multicast or broadcast services to be delivered as part of MBS Session.

#### 7.X.1.2	MBS session configuration

Editor's note:	The term "configuration" can be revisited if SA2 determines to use other names.

When the MC service server decides to use MBS sessions for MC service group communications, the MC service server is first required to configure with the 5GC one or more MBS sessions to deliver the media associated to the MC service group communications. As specified in 3GPP TS 23.247 [xx], the MBS session configuration request includes the MBS session type and required service requirements associated to the MBS session. Among other aspects, the MBS session configuration request may also include MBS service area information, which can be Cell ID list, TAI list or geographical area information.

NOTE 1:	It is implementation specific whether the MC service server decides to use multicast or broadcast MBS sessions for MC service group communications.

Unless the MB-SMF information is already available at the MC service server, e.g. it is locally configured, the MC service server (directly or via NEF/MBSF) performs an MB-SMF discovery and selection. This can be done by requesting this information towards the 5GS during a session configuration phase, as defined in 3GPP TS 23.247 [xx]. For this purpose, MBS broadcast or multicast session related information, among others, e.g. S-NSSAI(s) and DNN(s) are utilized during an MB-SMF discovery.

Moreover, the MC service can decide to configure one MBS session per MC service group. In principle, an MBS session should be sufficient to handle all MC service group communications related to the corresponding MC service group, where different service requirements can be addressed by different MBS QoS flows within the MBS session. For instance, if different required QoS requirements are considered for application level control messages, e.g. floor control messages, and media for an MCPTT group call, a QoS flow can be configured for the application level control messages and another QoS flow for the media.

However, for the case of multicast MBS sessions, the MC service server could also decide to configure more than one MBS session per MC service group to deliver the same MC service media. This can avoid creating a one-to-one mapping between the members of an MC service group and the dedicated set of UEs associated to a multicast MBS session. Hence, the MC service server can hide any sensitive MC service group configuration from the 5GC.

NOTE 2:	It is implementation specific whether the MC service server decides to configure one or multiple MBS sessions to transmit the MC service media associated to an MC service group.

As in eMBMS, the MC service server could also decide to not permanently associate an MBS session to a specific MC service group or MC service group communication. This could particularly be applicable for broadcast MBS sessions where all MC service UEs within a broadcast service area may be able to receive the same broadcasted media. However, for the case of multicast MBS sessions where the same media is only distributed to a specific set of users, the MC service server should, therefore, configure an MBS session per MC service group.

The MC service server can decide to configure MBS sessions based on different conditions. For instance, the MC service server can configure one or multiple MBS sessions based on MC service group status, e.g. when a specific group is active with affiliated users. This can be considered regardless of any MC service group communication has been initiated. The MBS session(s) can later be used by the MC service server for the different MC group communications associated to the specific MC service group. For this case and based on the MC service group configuration, the MC service server can in advance provide expected required service requirements to be associated to the MBS session. Also, the MC service server may dynamically update MBS sessions, e.g. to modify service areas or provide new required service requirements on demand. For this purpose, the MC service server initiates an MBS session update towards the 5GS. As described in 3GPP TS 23.247 [xx], this can either be an MBS session update to the MB-SMF when PCC is not used, or an MBS session configuration update to the PCF when PCC is used.

Likewise, the MC service server can also decide to dynamically configure MBS sessions on demand based on, e.g. the establishment of specific MC service group communications, as defined in 3GPP TS 23.280 [3] for MBMS transmissions.

NOTE 3:	It is implementation specific whether the MC service server decides to configure MBS sessions in a pre-configured mode or dynamic mode.

For multicast MBS sessions, as described in 3GPP TS 23.247 [xx], three states have been defined: configured state, active state and inactive state. Based on these states, after an MBS session is configured and the required network resources are reserved, the MC service server can request the multicast MBS sessions to be either in active or inactive state. For instance, the MC service server can request the configuration of an MBS session in inactive state while there are neither active group communications associated to a specific MC service group nor MC media to be transmitted yet. The inactive session can then be activated to reserve radio resources hence send MC media. The activation may also be triggered by the 5GC (in specific MB-UPF) once it receives the MC media to be transmitted. The MC service server may activate the MBS session once the MC service media becomes available to be transmitted over the multicast MBS sessions, e.g. when a group communication is established for the corresponding MC service group. This can be by explicitly sending a multicast MBS session activation request towards the 5GC. Hence, network resources can be handled in a more efficient way.

NOTE 4:	For broadcast MBS sessions, the session is activated and started as part of session configuration procedures, and network resources both in NG-RAN and core network are reserved, as described in 3GPP TS 23.247 [x].

As part of the MBS session configuration, specific MBS information like MBS session ID and how the media can be provided to be distributed over the MBS session is defined and available at the MC service server.

Also, the MC service server can request the configuration of an MBS session with two modes: transport only mode or enhanced service mode. In the transport only mode, the MC service media is transmitted transparently through the 5GS. In the enhanced service mode, the MBS services may include additional services, e.g. FEC.

#### 7.X.1.3	MBS service announcement

When an MBS session is configured, the MC service server shall announce the corresponding MBS session(s) to the target MC users via an MBS service announcement. As being defined in 3GPP TS 23.247 [xx], the MBS service announcement shall include, among others, the MBS session ID, session type, SDP information, and any other information required to enable that MC service UEs can receive MC service media over a specific MBS session.

Based on the MBS service announcement the MC service UE can become able to receive MC service media over the MBS session.

Table 7.X.1.3-1 specifically describes the MBS service announcement to be sent from the MC service server to MC service clients to enable that MC service UEs can receive MC service group communication media over MBS sessions.

Table 7.X.1.3-1: MBS service announcement for the use of MBS session for MC service group communications

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | The identity of the MBS session used to deliver MC service group communication media. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| SDP information | M | SDP related to application-level control signalling or media to be transmitted over the MBS session (e.g., codec, protocol ID, FEC information, IP address and ports) |
| List of MBS Service Area information | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| Service announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service announcement |
| Multicast MBS session related information (NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session, such as PLMN ID of the default PLMN service provider in case of source specific IP multicast address, DNN, and S-NSSAI of the PDU session associated with the multicast MBS session |
| UE MBS session join notification (NOTE 2) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| Monitoring state (NOTE 3) | O | Indicate if the MC service client is required to actively monitor the broadcast MBS session quality and report it to the MC service server |
| NOTE 1: Such information may be pre-configured in the MC service UE, or provided in any other implementation specific way NOTE 2: It is applicable for multicast MBS session NOTE 3: It is applicable for broadcast MBS session |  |  |

#### 7.X.1.4	Procedures

##### 7.X.1.4.1	General

The procedures in this clause describes how pre-configured and dynamic MBS session configuration and MBS service announcement can be used for the transmission of MC service group communication media over either multicast or broadcast MBS sessions.

##### 7.X.1.4.2	Procedure for pre-configured MBS session and service announcement

For pre-configured MBS session, it is pre-configured for any group communication or for a certain group communication, and it is able to satisfy the MC group communication. In addition, the MC service server already retrieves the TMGI from the 5GC as specified in TS 23.247 [xx].

When the MC service server decides to use an MBS session for the DL transmission, the MC service server interacts with 5GC to prepare the necessary network resource for the MBS delivery. For the case of multicast MBS session, the MC service server also instructs the UE to join the multicast MBS session.

NOTE 1:	The MC service server logic for determining when to establish the new multicast MBS session is implementation specific.

Figure 7.X.1.4.2-1 demonstrates the MBS session configuration and service announcement for dynamic and pre-configured use cases, respectively.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	The MC service server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

Figure 7.X.1.4.2-1: Pre-configured MBS sessions and MBS service announcement

1.	The MC service server decides to configure a multicast or broadcast MBS session targeting the MC service group X, e.g. based on the affiliation status of the group members. As specified in 3GPP TS 23.247 [xx], the MBS session configuration procedure is initiated by the MC service server by requesting an MBS session to the 5GC, where the session can either be configured with or without PCC. The MC service server provides, among others, the MBS session type and service requirements, e.g. required QoS, for the MBS session. The MC service server may also indicate how media can be distributed to the 5GC to be transmitted over the MBS session. As a response, the MC service server receives the MBS session ID and other parameters associated to the configured MBS session.

NOTE 1:	For broadcast MBS sessions, the session is established as part of the session configuration procedures, hence the network resources are reserved both in 5GC and NG-RAN.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the configured MBS session via an MBS service announcement. As described in Table 7.X.1.3-1, the service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information.

NOTE 2:	The service announcement can either be sent via unicast PDU session, or via an existing MBS session, and it can be repeated as needed.

For multicast MBS sessions, further information related to the MBS session may be sent to enable that MC service clients to join the announced multicast MBS session.

For broadcast MBS sessions, the session announcement may indicate if the MC service server requires the MC service client to actively monitor and report the quality of the announced broadcast MBS session.

3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the receipt of the corresponding MBS service announcement.

5.	Based on the MBS session mode; either multicast or broadcast, the following actions take place:

5a.	For multicast MBS sessions, MC service clients initiate a UE session join request towards the 5GC using the information provided via the MBS service announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [xx].

5b.	For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring status back to the MC service server.

6.	The MC service clients provide a notification related to the announced MBS session in the form of an MBS listening status report.

NOTE 3:		It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g. measurements of radio level signalling such as the reference signals from the NG-RAN node(s), packet loss.

7.	An MC service group communication session is established as specified in TS 23.379 [aa], TS 23.281 [bb], or TS 23.282 [cc]. The MC service server determines to use the pre-configured MBS session for this group communication. If the pre-configured MBS session fails to satisfy the group communication, the MC service server interacts with the 5GC to update the service requirement of the MBS session.

#### 7.X.1.4.3	Procedure for dynamic MBS session and service announcement

The procedure in figure 7.X.1.4.3-1 describes how dynamic MBS session configuration and MBS service announcement can be used for the transmission of MC service group communication media over either multicast or broadcast MBS sessions.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	The MC service server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

Figure 7.X.1.4.3-1: Dynamic MBS service announcement for multicast and broadcast MBS sessions

1.	The MC service server decides to configure a multicast or broadcast MBS session targeting the MC service group X, e.g. based on the affiliation status of the group members. As being specified in 3GPP TS 23.247 [xx], the MBS session configuration procedure is initiated by the MC service server by requesting an MBS session to the 5GC, where the session can either be configured with or without PCC. The MC service server provides, among others, the MBS session type and service requirements, e.g. required QoS, for the MBS session. The MC service server may also indicate how media can be distributed to the 5GC to be transmitted over the MBS session. For the case of a multicast MBS session, the MC service server can indicate the state for the MBS session to be configured. As a response, the MC service server receives the MBS session ID and other parameters associated to the configured MBS session.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the configured MBS session via an MBS service announcement. As described in table 7.X.1.3-1, the service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information.

For multicast MBS sessions, further information related to the MBS session may be sent to enable that MC service clients join the announced multicast MBS session.

For broadcast MBS sessions, the session announcement may indicate if the MC service server requires the MC service client to actively monitor and report the quality of the announced broadcast MBS session.

3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS service announcement.

5.	For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

NOTE 1:		It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signalling such as the reference signals from the NG-RAN node(s), packet loss.

6.	For multicast MBS sessions, MC service clients initiate a multicast MBS Session UE Join towards the 5GS using the information provided by the MBS service announcement.

7.	The MC service clients provide a notification related to the announced MBS session. For the case of multicast MBS sessions, the clients send a multicast MBS session UE join notification towards the MC service server to indicate that the announced MBS session has been joined successfully. For the case of broadcast MBS sessions, the clients send a notification to the MC service server to indicate the monitoring status of the announced MBS session, e.g., similar to the listening status report used in MBMS.

# * * * * End of changes * * * *