3GPP TSG-SA WG6 Meeting #46-e	S6-212574

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0015 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service announcement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2021-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the development done in TS 23.247 on MBS sessions, this CR focuses on MBS service announcement, and the subsequent procedures taken by the MC service clients based on the mode of the announced session. Furthermore, the information elements provided during the MBS service announcement are presented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR presents the information elements sent from the MC service server to the targeted MC service clients during the MBS service announcement. Furthermore, it describes the subsequent procedure done by the MC service clients upon the successful reception of the MBS service announcement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No information or content related to the MBS service announcement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x.y (new), 7.x.y.1 (new), 7.x.y.2 (new), 7.x.y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.x.y MBS service announcement

#### 7.x.y.1 General

Whether being created prior to group communication set up, or dynamically upon demands, the information related to the MBS session is announced towards the targeted MC group. Based on the session mode, i.e., whether a broadcast or multicast MBS session, the MC service UE needs to react to the MBS service announcement accordingly. For example, in case of multicast MBS sessions, further information related to the MBS session may be sent to enable that MC service clients join the announced multicast MBS session. In case of broadcast MBS sessions, the session announcement may indicate if the MC service server requires the MC service client to actively monitor and report the quality of the announced broadcast MBS session. However, either MBS session modes, the MBS listening status report is useful for broadcast and multicast MBS sessions in order to facilitate service continuity between MBS sessions and unicast PDU sessions.

#### 7.x.y.2 Information flow

Table 7.x.y.2-1 provides the information elements during MBS service announcement, which are sent by the MC service server to the clients.

Table 7.x.y.2-1: MBS service announcement for the use of MBS session for MC service group communications

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID | M | The identity of the MBS session used to deliver MC service group communication data. It is either TMGI for broadcast MBS and multicast MBS sessions, or source specific IP multicast address for multicast MBS session |
| MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| MC service group ID | O | Indicate the MC service group ID associated to the MBS session |
| SDP information | M | SDP related to application-level control signalling or media to be transmitted over the MBS session (e.g., codec, protocol ID, FEC information, IP address and ports) |
| List of MBS Service Area information | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| Service announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS service announcement |
| Multicast MBS session related information (NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session such as PLMN ID of the default PLMN service provider in case of source specific IP multicast address, DNN, and SNSSAI of the PDU session associated with the multicast MBS session |
| MBS UE session join notification (NOTE 2) | O | Indicate if the MC service server requires a notification from the MC service client once it has joined the multicast MBS session |
| Monitoring state (NOTE 3) | O | Indicate if the MC service client is required to actively monitor the broadcast MBS session quality and report it to the MC service server |
| NOTE 1: Such information may be pre-configured in the MC service UE, or provided in any other implementation specific way NOTE 2: It is applicable for multicast MBS session NOTE 3: It is applicable for broadcast MBS session |  |  |

#### 7.x.y.3 Procedure

The procedure in figure 7.x.y.3-1 presents the MBS service announcement and the subsequent behaviour of the MC service UEs based on the session mode.

Pre-conditions:

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server has decided to use an MBS session for MC service group communications associated to MC service group X.

-	The MC service server has performed an MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	MC service server has interacted with the 5GC (based on the configuration option under consideration and whether dynamic PCC rule is considered) to request the allocation of TMGI(s) and create MBS session(s) based on the given service requirements (e.g., QoS requirements).

Figure 7.x.y.3-1: MBS session creation and service announcement

- The MC service server provides the MC service clients affiliated to MC service group X with the information related to the configured MBS session via an MBS service announcement. As described in table 7.x.y.2-1, the service announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type).

NOTE 1: 	The service announcement can either be sent via a unicast PDU session, or via an existing MBS session, and it can be repeated as needed.

2.	MC service clients store and process the received MBS session information in order to react according to the mode of the announced MBS session.

3.	MC service clients may provide an MBS service announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS service announcement.

4.	Based on the MBS session mode (either multicast or broadcast) the following actions take place:

4a. For multicast MBS sessions, MC service clients initiate a UE session join request towards the 5GC using the information provided via the MBS service announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved. The established session can either have an active or inactive state.

NOTE 2: 	For broadcast MBS sessions, the session is established as part of the session configuration procedures, hence the network resources are reserved both in 5GC and NG-RAN.

4b. For broadcast MBS sessions, the MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS service announcement information, MC service clients report the monitoring state back to the MC service server.

5. The MC service clients provide a notification related to the announced MBS session in the form of an MBS listening status report.

NOTE 3: It is implementation specific whether the broadcast reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signalling such as the reference signals from the NG-RAN node(s), packet loss.