3GPP TSG-SA WG6 Meeting #47-e	S6-220232

e-meeting, 14th – 22nd February 2022	(revision of S6-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS23.289 | CR | 0039 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Updates to usage of 5MBS for MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Enhanced MCPTT group call setup procedure with MBMS bearer is specified in TS 23.379. The similar mechanism should be supported when using 5GMBS. The supporting of using 5G MBS for application layer paging message is already mentioned in clause 7.3.3.6. The further clarification about to support this feature is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the description about how to use 5GMBS for application layer paging message. - Adding description of the usage of 5G MBS for MCPTT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5MBS related features will not be aligned all MC services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.10.1, 7.3.3.11.1, 7.3.3.11.X (new) 7.3.3.3.11.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*********************************Start of the change************************

##### 7.3.3.10.1	General

The application layer signalling such as the floor control messages, application layer paging messages may be transmitted along with the MC media over the same MBS session however with different QoS requirements, or over other means such as unicast downlink or a different MBS session.

When using the procedures for pre-configured or dynamic MBS session establishment for MCPTT, the MCPTT server performs the procedure of call connect and disconnect over MBS session at the group communication session establishment phase.

*********************************next change************************

##### 7.3.3.11.1	General

MCVido server can determine to use 5G MBS for the transmission of DL link media for different types of MCVideo group calls.

The application layer signalling such as the transmit media request and transmit media granted messages, may be transmitted along with the MC media over the same MBS session however with different QoS requirements, or over other means such as unicast downlink or a different MBS session.

When using the procedures for pre-configured or dynamic MBS session establishment for MCVideo, the MCVideo server perform the procedure of call connect and disconnect over MBS session.

The MCVideo service shall support the procedure for using MBS sessions as specified clause 7.3.3.1 with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-8 and MCVideo-9 reference points for this procedure.

*********************************The second change*********************************************

#### 7.3.3.11.x		Switching from MBS session to unicast session or eMBMS

The MCVideo service shall support the procedure for switching from MBS session to unicast session as specified 7.3.3.8 and clause 7.3.3.9 with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-4, MCVideo-7, MCVideo-8 and MCVideo-9 reference points for this procedure.

*********************************End of the change************************

### 7.3.3.11.y	DL media transmission with 5MBS

The MCViedo service shall support the procedure for using pre-created MBS session, or dynamic MBS session as specified clause 7.3.3.1.

MCVideo may use pre-created MBS session for the different types of MCVideo group calls. Both pre-arranged group calls and chat group calls can use the pre-created MBS session for distributing the media. The MBS session can be used by any group.

When using the pre-created MBS session for MCVideo, the MCVideo server may perform the procedure of call connect and disconnect over MBMS as defined in clause 7.3.3.11.2 for the group communication session establishment step, or send map group communication to MBS session as defined in clause 7.3.2.4 after group cumminication session establishment .

MCVideo may also use dynamic MBS session for the different types of MCVideo group calls. Both pre-arranged group calls and chat group calls can use the dynamic MBS session for distributing the media.

When using the procedures for dynamic MBS session establishment for MCVideo, the MCVideo server perform the procedure of triggering MBS session configuration as defined in clause 7.3.3.1 and mapping group communication to MBS session as defined in defined in clause 7.3.2.4  at the group communication session establishment step.

Both the media packets as well as the transmission control messages to the receiving MCVideo clients are sent on the MBS seesion. Optionally a separate MBS session could be used for the transmission control messages

*********************************End of the change************************