3GPP TSG-SA WG6 Meeting #47-e	S6-220271

e-meeting, 14th – 22nd February 2022	(revision of S6-220136)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0026 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use of 5G ProSe UE-to-network relay service for MC services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5GProSe |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The introduced CR provides a description of the proper use of 5G ProSe UE-to-network relay. Moreover, it provides the necessary requirements and parameters to enable the 5G ProSe UE-to-network relay service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A general description of the 5G ProSe Layer-2 and Layer-3 UE-to-network relay is presented, with alignment to 3GPP TS 23.304. - The necessary parameters and requirements to enable the 5G ProSe UE-to-network relay service are presented. - Reference to 3GPP TS 23.304 is made within the document. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 5G Prose UE-to-network relaying services are not enabled to relay MC traffic in 5G system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 4.x.1(new), and 4.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.x Use of 5G ProSe UE-to-network relay

### 4.x.1 General

The MC service shall support the capabilities for 5G ProSe UE-to-network relay. For this matter, 5G ProSe Layer-2 and 5G ProSe Layer-3 UE-to-network relaying techniques can be utilized, as described in 3GPP TS 23.304 [x].

A 5G ProSe UE-to-network relay supporting MC service UE provides means of connectivity and relaying of MC traffic to remote MC service UE(s). For this matter, the 5G ProSe UE-to-network Relay Discovery service allows the MC service remote UE to discover a potential UE-to-network relay UE supporting MC service in its proximity as described in 3GPP TS 23.304 [x]. Upon its discovery, the 5G ProSe Direct UE-to-network Relay Communication functionality is utilized to achieve communication to provide the MC service remote UE access to 5GS, and relay MC traffic via the UE-to-network relay UE over the NR PC5 reference point.

### 4.x.2 5G ProSe UE-to-network relay service requirements

In order to enable 5G ProSe UE-to-netowrk relaying capabilities – whether based on Layer-3 or Layer-2 UE-to-network relaying techniques, the MC service system provides the appropriate parameters and configurations to the MC service UE(s).

As defined in 3GPP TS 23.304 [x], among these parameters are: Relay Service Code(s) (RSCs) which can be associated to a certain MC service group, User Info, ProSe Layer-2 Group ID and ProSe Group IP multicast address. Moreover, the MC service group ID is resolved to the ProSe Layer-2 Group ID and ProSe Group IP multicast address, which are utilized within the 5G ProSe Relay Discovery and 5G ProSe Direct Communication procedures, as described in 3GPP TS 23.304 [x]. Furthermore, the RSCs are utilized to restrict the necessary UE-to-network relay service and related procedures within members of a certain MC service group.

# * * * * Second change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[x]	3GPP TS 23.304: "Proximity based Service (ProSe) in the 5G System (5GS); Stage 2".