3GPP TSG-SA WG6 Meeting #47-e	S6-220275

e-meeting, 14th – 22nd February 2022	(was S6-220031)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0023 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of section 4.7 of 23.289 with latest version of 23.247 (v 17.1.0) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Huawei, Hisilicon. |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 18 Feb 2022 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR merges changes in clause 4.7 from S6-220031 and S6-220233_Rev1. In December 2021, SA2 has updated 23.247 from version 17.0.0 to version 17.1.0, making some changes impacting 23.289, making alignment necessary. Specifically: - Reference point Nmb3 has been eliminated - Reference point Nmb4 was replaced by Nmb8 - Reference point Nmb6 was replaced by Nmb5 or Nmb10 - “Legacy” interfaces (e.g. MB2) are referenced only for interworking with eMBMS - The terms “5MBS” and “5G MBS” have been replaced with “MBS” This CR aligns 23.289 with 23.247 version 17.1.0 as mentioned above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR impacts only clause 4.7 and its subclauses. - Updated figures to reflect the changes in reference points; show Nmb5 used always with N33; and use 5GC+NG-RAN in lieu of 5GS (which includes UE) - Replaced “5MBS”, “5G MBS” with “MBS” to use same term as SA2 in 23.247. - Correct some missspellings and unnecessary color hilights - Removed no longer necessary EN regarding need for alignment with 23.247 interfaces and nomenclature. - Added text clarifying use of N5 by MCX server when MBS is used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment of interfaces could make implementation impossible or wrong. Use of alternative names for the same systems would result in confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 4.7.1, 4.7.2, 4.7.3, 4.7.3.1, 4.7.3.2, 4.7.3.3, 4.7.4, 4.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.7	Architectural aspects of MC services using MBS

### 4.7.1	General

The main purpose of 5G Multicast-Broadcast Service (MBS) use by mission critical services is to provide efficient downlink delivery of user traffic in group calls and communications. The architectural figures in this clause are aligned  with the 5GS architecture for MBS shown in Figure 5.1-2 of 3GPP TS 23.247 [15], which identifies both mandatory and optional functional entities and interfaces, in reference point representation, available for use by the MC services.

Multicast and broadcast services in 5G for MC group communications rely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the MC service server to multiple MC users (i.e., users affiliated to a certain MC group) is supported either as point-to-point or point-to-multipoint over the radio. The MBS sessions can either be of broadcast or multicast type and consist of one or multiple QoS flows for different service requirements.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the MC service server decides whether to create broadcast or multicast MBS sessions to be associated with certain MC groups. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the MC service UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED state.

The following capabilities (non-exhaustive list) provided by MBS could be used by MCX services server:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Transparent MBS Data forwarding;

-	Dynamic PCC control for MBS session.

Editor’s note:	 For Rel-17/Rel-18, the use by MCX of capabilities enabled by optional functional entities (e.g. MBSF/MBSTF) and their associated interfaces, e.g. features such as MCData file delivery via User service or media features (e.g. RoHC, FEC), is FFS.

Editor's Note: In clause 4.7 and its subclauses, the architecture representations and the reference points are FFS.

### 4.7.2	General on-network architecture for use of MBS by MC services

Figure 4.7.2-1 presents a high-level architectural view of mission critical services when using MBS. The shown architecture is consistent with 3GPP TS 23.501 [7], 3GPP TS 23.247 [15] and 3GPP TS 23.468 [16].

MCX services use MBS control plane capabilities by initiating access via Nmb13, Nmb10 or N33. MBS user plane capabilities can be accessed via N6mb or Nmb8. MCX servers can initiate access to MBS PCC capabilities supported by PCF via N5 or N33. If the MCX server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

Figure 4.7.2-1: Architectural view of a mission critical system when using MBS

NOTE 1:	Support of interfaces associated to 5GS optional entities (e.g. MBSF, MBSTF, NEF) is necessary only if features enabled by these entities are supported.

NOTE 2:	When the MCX server uses MBS, the N5 reference point is used in accordance to clause 5.3.1.

### 4.7.3	Specific instantiations of on-network architecture for use of MBS by MC services

#### 4.7.3.1	Instantiation without optional entities and associated interfaces

Figure 4.7.3.1-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entities (MBSF, MBSTF and NEF) and their associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.3.1-1.

MCX services use MBS control plane capabilities by initiating access via Nmb13. MBS user plane capabilities can be accessed via N6mb. MCX servers can initiate access to MBS PCC capabilities supported by PCF via N5.

Figure 4.7.3.1-1: Architectural view of a mission critical system when using MBS without optional MBS interfaces

#### 4.7.3.2	Instantiation without MBSF / MBSTF and associated interfaces

Figure 4.7.3.2-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entities MBSF and MBSTF and their associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.2-1.

MCX services use MBS control plane capabilities by initiating access via Nmb13 or N33. MBS user plane capabilities can be accessed via N6mb. MCX servers can initiate access to MBS PCC capabilities supported by PCF via N5 or N33. If the MCX server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

Figure 4.7.3.2-1: Architectural view of a mission critical system when using MBS without optional MBSF/MBSTF entities and their associated interfaces

#### 4.7.3.3	Instantiation without NEF and associated interfaces

Figure 4.7.3.3-1 presents a high-level architectural view of mission critical services when using MBS without the presence or use of the optional entity NEF and its associated interfaces. The shown architecture is a particularization of the general architecture shown in figure 4.7.2-1.

MCX services use MBS control plane capabilities by initiating access via Nmb13 or Nmb10. MBS user plane capabilities can be accessed via N6mb or Nmb8. MCX servers can initiate access to MBS PCC capabilities supported by PCF via N5.

Figure 4.7.3.3-1: Architectural view of a mission critical system when using MBS without optional NEF entity and its associated interfaces

### 4.7.4	Service layer-based interworking between eMBMS and MBS

Figure 4.7.4-1 presents a high-level architectural view of mission critical services interworking between eMBMS and MBS at the service layer. The shown architecture is consistent with 3GPP TS 23.247 [15], subclauses 5.2, 6.8 and configurations 2 and 3 in Annex A.

The interworking between eMBMS and MBS for mission critical operation is enabled by the Joint BM-SC, MBSF and MBSTF functional entity. MCX services can use control plane capabilities by accessing the Joint entity directly via MB2-C or Nmb10 or indirectly (using NEF) via N33+Nmb5. User plane traffic delivery is supported via MB2-U or Nmb8. If the MCX server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS PCC capabilities.

Figure 4.7.4-1: Service layer-based mission critical interworking between eMBMS and MBS

NOTE:	Support of interfaces associated to 5GS optional entities (e.g., NEF) is necessary only if features enabled by these entities are supported.

### 4.7.5	Application layer based interworking between eMBMS and MBS

Figure 4.7.5-1 presents a high-level architectural view of mission critical services interworking between eMBMS and MBS at the application layer. The shown architecture does not use the MBSF/MBSTF entities defined in 3GPP TS 23.247 [15] and is inclusive of configuration 1 in Annex A of 3GPP TS 23.247 [15].

MCX services initiate access to control plane capabilities via MB2-C (for eMBMS) and via Nmb13 or N33 (for MBS). User plane capabilities can be accessed via MB2-U (for eMBMS) and via N6mb (for MBS). MCX servers can initiate access to PCC capabilities via the Rx interface (for the PCRF in the EPS) and via the N5 or N33 interfaces (for the PCF in the 5GS). If the MCX server and the 5GS are in different trust domains with respect to MBS, N33 needs to be used to gain access to the MBS control plane capabilities and the PCC capabilities.

Figure 4.7.5-1: Application layer-based mission critical interworking between eMBMS and MBS

NOTE:	Support of interfaces associated to 5GS optional entities (e.g., NEF) is necessary only if features enabled by these entities are supported.